3GPP TSG-WG SA2 Meeting #153E e-meeting 	S2-2208885

Elbonia, Oct 10th – 17th, 2022	(revision of S2-220xxxx)

Source:	China Mobile

Title:	KI#2 Update to Conclusion

Document for:	Approval

Agenda Item:	9.25

Work Item / Release:	FS_UPEAS / Rel-18

Abstract: This contribution proposes a conclusion update for KI#2.

- Introduction/Discussion

This paper proposes a conclusion update on the key aspects of the Support UPF expose information to other NFs discussed in solutions adopted in the TR for Key Issue 2.

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-62.

* * * * First Change * * * *

8.2	Conclusions for KI#2

The following interim conclusions are proposed for KI#2:

1.	Subscription to UPF events via SMF is the rule except for the cases listed in bullet 2; Subscription via SMF means the final consumer of UPF event notifications sends the subscription request to the SMF and then the SMF is doing a third-party subscription onto UPF on behalf of this final consumer. Conversely the notifications are directly sent by the UPF to the final consumer of UPF notifications.

NOTE 1:	Optimizing notifications is more important than optimizing subscriptions.

NOTE 2:	 For the case that the subscription is related with the AoI, SMF should firstly get the UE list within the AoI and keep updating the UE list by subscribing to AMF, and secondly subscribe/unsubscribe to the relevant UPF(s) on behalf of the final consumer with the UE list information. This allows the UPF not having to determine the AMF where to subscribe for UE presence in the AoI.

NOTE 3:	For event subscriptions requiring interactions with 5G AN, a solution where the UPF event consumer would directly subscribe to UPF and then UPF would ask SMF to send N2 SM signalling to 5G AN would be more complex and not bring advantage.

2.	Direct subscription to UPF (i.e. not requiring third party subscription to UPF via SMF) shall be possible for data collection where UPF is the source as defined in TS 23.288 [5], i.e. the following cases:

A.	TS 23.288 [5] Table 6.5.2-2: Data collected by NWDAF for UPF load analytics recalled in item 2 of Annex A of the TR.

B.	For analytics targeting "any UE" (possibly for specific DNN and or slices) and not related with an AoI or with a specific data flow.

NOTE 4:	This can relate to use cases such as Data collected by NWDAF for UPF load analytics, User Data Congestion Analytics, Data Volume dispersion analytics, WLAN performance analytics.

Editor's note:	Whether other direct subscriptions are possible is FFS.

3.

Editor's note:	When a SMF is doing a third-party subscription on behalf of the final consumers of UPF notifications, it is FFS whether SBI and/or PFCP (TS 29.244 [8]) is used for interaction with UPF.

4.	In Rel18:

A.	the only defined consumers of UPF event SUBSCRIBE are SMF, and NWDAF.

B.	the only defined consumers of UPF event notifications are AF/NEF, TSNAF/TSCTSF and NWDAF.

Editor's note:	Whether AF/NEF can directly subscribe to UPF is FFS.

5.	UPF event exposure Service description: This service provides events related to PDU Sessions towards consumer NF. The service operations exposed by this service allow other NFs to subscribe and get notified of events happening on UPFs.

The following events may be subscribed by a NF consumer:

-	Event: QoS monitoring. This event provides QoS Flow level performance information (information listed in Solution #8, clause 6.8.2).

-	Event: UserDataUsageMeasures. This event provides information of user data usage of the User PDU Session (information listed in Solution #7, clause 6.7.2).

-	Event: UserDataUsageTrends. This event provides statistical measurements (information listed in Solution #7, clause 6.7.2).

Editor's note:	The following part is FFS "Both UserDataUsageMeasures and serDataUsageTrends events provide measurement context (for example, time stamps for the packets and the measures) and information of the PDU Session. When the information refers to an application, the Application Id or Packet Filter Set is included".

6.	To determine which SMF to contact the final consumer of UPF events proceeds as follows:

-	If the event targets any UE or AoI, the final consumer of UPF events looks up the NRF to discover all suitable SMF(s) with Event Exposure Service capabilities (e.g. SMF(s) that serve the target combination of DNN and S-NSSAI).

-	If the event targets a unique UE identified by its SUPI or a group of UEs, the final consumer of UPF events sends Nudm_UECM_Get_Request (SUPI, type of requested information set to SMF Registration Info and the S-NSSAI and DNN) to UDM to get the SMF ID serving the target UE.

Editor's note:	If the consumer of NWDAF service doesn't provide the necessary parameters to NWDAF, for example, the DNN, S-NSSAI (these parameters are optional for NWDAF service defined in TS 23.288 [5]), how to discover the SMF in any UE situation without these parameters is FFS.

7.	For the UPF data collection, the event subscription includes Reporting suggestion information as described in Sol#14, which is used to assist the UPF event notification. Per Reporting suggestion information UPF can concatenate several notification messages to the same notification endpoint in one notification message.

* * * * End of Change * * * *