3GPP TSG-WG SA2 Meeting #153E e-meeting 	S2-2208912

Elbonia, October 10 – 17, 2022	(revision of S2-220xxxx)

Source:	Huawei, HiSilicon

Title:	KI#2: Conclusion update on rule priority

Document for:	Approval

Agenda Item:	9.26

Work Item / Release:	FS_5G_ProSe_Ph2 / Rel-18

Abstract: KI#2 conclusion update on rule priority.

# 1. Introduction/Discussion

Based on the current conclusion of KI#2, both URSP rules and re-selection rules provided by an application server can be used for path switch. It is not clear that how to use them when both are available on Remote UE.

Based on the principle of Authorization and Provisioning for ProSe service defined in clause 5.1.1 of TS 23.304, the parameters and policy from PCF take higher priority than the ProSe Application Server. Thus this paper proposes to conclude that URSP rules take higher priority than the re-selection rule provided by an application server for path switch.

If the same parameters described in clauses 5.1.2.1, 5.1.3.1 and 5.1.4.1 are provided by different sources, the UE shall consider them in the following priority order:

-	provided or updated by the PCF (including parameters determined by the PCF itself and parameters provided by the ProSe Application Server to the PCF as specified in clause 6.2.5);

-	provided or updated by the ProSe Application Server via PC1 reference point;

-	configured in the UICC;

-	provisioned in the ME.

# 2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-33.

# * * * * First change * * * *

## 8.2	Key Issue #2: Support of path switching between two indirect network communication paths for UE-to-Network Relaying with service continuity consideration

For Key Issue #2: Support of path switching between two indirect network communication paths for UE-to-Network Relaying with service continuity consideration, the followings are taken as conclusions:

-	For the triggers and criteria for path switching, the following principles are adopted in normative work:

-	The Remote UE can path switch when the NG-RAN configured measurement thresholds and the criteria are satisfied or based on re-selection rules from application layer if any.

-	If multiple UE-to-Network Relay UEs satisfy the Relay re-selection criteria, the Remote UE selects the target UE-to-Network Relay for path switch based on the 5G ProSe Policy in clause 6.5.4 of TS 23.304 [3] or URSP rules and the Remote UE traffic handling described in clause 6.5.4 of TS 23.304 [3] with following considerations:

-	The Remote UE first selects a Target relay UE which has same type as original relay UE, if this fails, then the Remote UE re-evaluates the URSP or keeps evaluating the next RSD of the selected URSP for Target Relay selection.

-	The Remote UE may use re-selection rules from the application layer (e.g. provided by an application server), the Remote UE may discover that there are multiple UE-to-Network Relay UEs that satisfy the re-selection rules and then selects the target UE-to-Network Relay UE based on priority in the re-selection rules from the application layer.

-	URSP rules take higher priority than the re-selection rules from the application layer.

NOTE 1:	The measurement trigger criteria for path switch between two indirect network communication paths will be defined by RAN WG.

NOTE 2:	SA2 will not define the re-selection rules from application layer and how they are provided to the UE.

-	To supporting path switch with service continuity, solution#15 is taken as baseline for normative work.

NOTE 3:	The mechanisms supporting path switching by application layer for service continuity are left to application layer implementation and normative work is not needed.

NOTE 4:	Handover procedures for path switch between Layer-2 UE-to-Network Relay indirect communication paths require coordination with RAN WGs.

-	For switching case between two indirect Layer-3 UE-to-Network Relay paths using N3IWF, solution#36 using MOBIKE is selected as basis for normative work.

# * * * * End of changes * * * *