3GPP TSG-WG SA2 Meeting #153E e-meeting 	S2-2208919

Elbonia, October 10 – 17, 2022	(revision of S2-220xxxx)

Source:	Huawei, HiSilicon

Title:	KI#6: Conclusion

Document for:	Approval

Agenda Item:	9.26

Work Item / Release:	FS_5G_ProSe_Ph2 / Rel-18

Abstract: This paper proposes the conclusion for KI#6 Support of PC5 Service Authorization and Policy/Parameter Provisioning.

# 1. Introduction/Discussion

This KI#6 focuses on Support of PC5 Service Authorization and Policy/Parameter Provisioning.

Based on Table 6.0-1, the solutions for KI#6 are Sol#8, Sol#16, and Sol#25.

There are several scenarios that need to be considered and concluded on:

-	UE-to-UE Relay.

-	Path switching between two indirect network communication paths for UE-to-Network Relaying.

-	Direct communication path switching between PC5 and Uu.

-	Multi-path transmission for UE-to-Network Relay.

Conclusions for the above aspects are proposed.

# 2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-33.

# * * * * First change (all new)* * * *

## 8.X	Key Issue #6: Support of PC5 Service Authorization and Policy/Parameter Provisioning

For Key Issue #6: Support of PC5 Service Authorization and Policy/Parameter Provisioning, the followings are taken as conclusions:

-	The mechanism and procedure defined in clause 5.1 of TS 23.304 [3] is used for provisioning the Authorization and Policy/Parameters for UE-to-UE Relay, Multi-path transmission for UE-to-Network Relay, Direct communication path switching between PC5 and Uu, and Path switching between two indirect network communication paths for UE-to-Network Relaying.

NOTE:	Backward compatibility with Rel-17 shall be considered when concluding this key issue for Rel-18 ProSe.

-	For UE-to-UE Relay, Sol#8 is selected for normative work and can be further updated based on the conclusion for KI#1 in clause 8.1.

-	For Path switching between two indirect network communication paths for UE-to-Network Relaying, the existing ProSe policy and URSP is reused, and can be further updated based on the conclusion for KI#2 in clause 8.2.

-	For Direct communication path switching between PC5 and Uu, the path switching policy is used, and can be further updated based on the conclusion for KI#3 in clause 8.3.

-	For Multi-path transmission for UE-to-Network Relay, policy authorization for multi-path transmission via Layer 3 U2N Relay UE without N3IWF is needed, and can be further updated based on the conclusion for KI#5 in clause 8.5.

# * * * * End of changes * * * *