3GPP TSG-WG SA2 Meeting #153E e-meeting 	S2-2208980

Elbonia, 10th – 17th October, 2022	(revision of S2-220xxxx)

Source:	Samsung

Title:	KI#1 Evaluation and Conclusions

Document for:	Approval

Agenda Item:	9.14

Work Item / Release:	FS_eNS_Ph3 / Rel-18

Abstract: This paper proposes solution evaluation and conclusions for KI#1.

1. Introduction/Discussion

The conclusions for KI#1 is proposed as below where the key aspect to highlight is that when the alternative S-NSSAI is determined (either within Subscribed S-NSSAIs or not), the URSP rule needs to be updated so that the UE uses use, for the new traffic, the new S-NSSAI instead of the old S-NSSAI.

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-41 V0.4.0.

# * * * * First change * * * *

8.X	Conclusions for Key Issue #1

The following principles are concluded for the normative work.

NOTE:	The IE names for the alternative S-NSSAI and other new information in the conclusion is to be determined during normative phase.

0)	The trigger to replace a currently used S-NSSAI with an alternative S-NSSAI is either based on local configuration (e.g. based on OAM) or based on PCF notification.

Editor's Note:	It is FFS what SBI service is used for the PCF notification.

1)	The AMF determines the alternative S-NSSAI to be used to replace the old S-NSSAI. The AMF may optionally interact with the PCF and/or with the NSSF to determine the alternative S-NSSAI.

Editor's Note:	It is FFS whether to allow the alternative S-NSSAI to be outside of the UE's Subscribed S-NSSAI. The S-NSSAI used by the UE for URSP needs to be a subscribed S-NSSAI, i.e. if the alternative S-NSSAI is not part of the UE’s Subscribed S-NSSAI mapped S-NSSAI logic or similar is required.

2)	In case of a same S-NSSAI associated with multiple NSIs and the change of the NSI is required, when an existing PDU session is decided as to be migrated to another NSI, the SMF performs PDU Session re-establishment by using the mechanism of SSC mode#2 or SSC mode#3. When a PDU session establishment request from the UE arrives at the AMF, the AMF performs NSI re-selection i.e. removes from the UE context the old NSI and the AMF selects another NSI, e.g. as described in solution #15.

Editor's Note:	It is FFS whether this principle is in scope of SA5. It could be sufficient if the AMF selects the appropriate NSI at PDU Session establishment.

3)	In cases where the old S-NSSAI and the alternative S-NSSAI are associated with the same NSI, the PDU session is associated with multiple S-NSSAIs in the network (i.e. NG-RAN, AMF, SMF, UPF). During PDU session establishment or HO procedure, the AMF notifies the alternative S-NSSAI to SMF and the SMF provides the alternative S-NSSAI to the NG-RAN without UE impact, as described in solution #1.

Editor's Note:	Further details are FFS, and FFS whether suitable to have a PDU Session to be associated with multiple S-NSSAIs and to use different S-NSSAI towards the UE compared to what is used within the network.

4)	In cases where the old S-NSSAI is replaced by an alternative S-NSSAI deployed on a different NSIs, the following principles apply:

a)	The AMF performs UE MM configuration update (e.g. UCU procedure) to include the alternative S-NSSAI in the Allowed NSSAI and/or in the Configured NSSAI, if not included yet. If the alternative S-NSSAI is not part of the Subscribed S-NSSAIs, the AMF provides the alternative S-NSSAI associated with a replaced/mapped value to the old S-NSSAI in both roaming or non-roaming case.

Editor's Note:	It is FFS whether the existing Mapping Of Allowed NSSAI can be re-used, or a alternative IE (e.g. Replaced NSSAI) has to be specified.

Editor's Note:	If there are existing PDU Sessions on the old S-NSSAI, it is FFS whether the AMF includes in the Allowed NSSAI the alternative S-NSSAI and temporarily the old S-NSSAI to ensure that first the PDU Session transfer to the alternative S-NSSAI is performed before releasing the PDU Session on the old S-NSSAI, which is applicable only if the PDU Session is of SSC mode 3. Alternatively, the AMF provides information in MM message to the UE such that the UE does not locally release the old S-NSSAI as well as not Establising new PDU Sessions using the old S-NSSAI. Or alternatively, the AMF provides the message to the PCF including the indication to update the URSP rules, the old S-NSSAI, and the alternative S-NSSAI. Then, the PCF updates the URSP rules to ensure that the alternative S-NSSAI is used instead of the old S-NSSAI and provides the updated URSP rules to the UE. The AMF provides information in MM message to the UE such that the UE does not locally release the old S-NSSAI.

b)	The URSP rules are updated to so that the UE uses, for new traffic, the new S-NSSAI instead of the old S-NSSAI.

c)	For a new PDU Session establishment request from the UE on the old S-NSSAI:

Editor's Note:	It is FFS whether (1) the AMF rejects the UE's request for PDU Session establishment with an indication to use an alternative S-NSSAI; or (2) the AMF proceeds with the PDU Session establishment to the SMF, the AMF indicates the alternative S-NSSAI to the SMF and the SMF proceeds with the PDU Session establishment on the alternative S-NSSAI. Alternatively, the AMF provides information in MM message to the UE such that the UE does not locally release the old S-NSSAI as well as not Establising new PDU Sessions using the old S-NSSAI

d)	For an existing PDU Session transfer from the old S-NSSAI to an alternative S-NSSAI:

-	The AMF notifies the current SMF, e.g. by triggering Nsmf_PDUSession_UpdateSMContext service operation, that the PDU Session is to be relocated to an alternative S-NSSAI and optionally indicates the alternative S-NSSAI.

-	The SMF sends to the UE either PDU Session Modification Command (if the PDU Session is of SSC mode 3) or PDU Session Release/Modification Command (if the PDU Session is of SSC mode 1 or 2) and indicates to the UE that the PDU Session can be re-established on another S-NSSAI. If AMF indicated the alternative S-NSSAI the SMF provides the alternative S-NSSAI to the UE.

Editor's Note:	It is FFS whether the SMF sends to the UE (1) the alternative S-NSSAI or (2) only an indication that the PDU Session can be re-established on another S-NSSAI.

-	the UE triggers a new PDU Session Establishment procedure to establish a PDU Session as follows:

-	If the alternative S-SNSAI is part of alternative RSD of the same URSP rule (e.g. as per 4b after URSP rule update), the PDU Session establishment request includes only the alternative S-NSSAI; otherwise

-	The PDU Session establishment request includes the alternative S-NSSAI together with the old S-NSSAI (similar as to use the mapped S-NSSAI value of the VPLMN when applying the RSD matched to the HPLMN S-NSSAI).

-	(alternative to the two previous bullets) If the UE didn’t receive an alternative S-NSSAI from the SMF, the UE uses the URSP to determine the S-NSSAI to use and in the process the UE does not use URSP rules with the old S-NSSAI. If the UE received an alternative S-NSSAI from the SMF, the UE includes the alternative S-NSSAI in the PDU Session establishment request.

Editor's Note:	It is FFS whether (1) the PCF updates the URSP rules to include the alternative S-NSSAI for new traffic; and (2) whether to allow UE to modify the URSP rule by itself without explicit update by the PCF.

# * * * * End of changes * * * *