3GPP TSG-WG SA2 Meeting #153e	S2-2208988

E-meeting, October 10-17, 2022	(revision of S2-220xxxx)

Source:	Samsung

Title:	KI #2, conclusion

Document for:	Approval

Agenda Item:	9.22

Work Item / Release:	FS_UEPO / Rel-18

Abstract: This contribution proposes conclusion on Key Issue #2.

1. Background

Let’s remind what we agreed on the key issue #2.

This key issue address WT#4: whether and how the network can be made aware when the UE enforces URSP rule for a traffic and investigate whether and what actions the 5G network can perform.

The following aspects will be studied:

-	Whether and how the 5GC can be made aware whether or when the UE enforces a URSP rule to route an application traffic to a PDU Session based on the URSP rule provisioned by 5GC.

-	Whether there are any actions the 5GS can take after 5GC is aware whether the UE enforces a URSP rule for specific application traffic or not. If any, what action 5GC should take?

2. Proposal

It is proposed to update the following changes in TR 23.700-85.

# * * * * 1st change (all new text) * * * *

8.2	Conclusion on Key Issue #2

The normative work is based on the following principles:

- In order for the 5GC to be made aware whether and when the UE enforces a URSP rule, the following shall be supported:

  - The UE reports the information to identify the URSP rule to the 5GC.

  - The PCF checks the Route Selection Descriptor of the identified URSP rule during the PDU Session Establishment. UE-PCF is reported from the SM-PCF if two PCFs are different.

  - The PCF checks the Traffic Descriptor of the identified URSP rule after the PDU Session Established by using application detection. UE-PCF is reported from the SM-PCF if two PCFs are different.

- After 5GC is aware of whether the UE enforces a URSP rule for specific application traffic 5GC shall be able to take the following actions:

  - The PCF shall be able to reject the PDU Session if the PDU Session Parameters do not match the Route Selection Descriptor of the identified URSP rule.

  - The PCF shall be able to make a new policy decision such as updating the PCC rule for gating traffic or updating URSP rule using the existing policy mechanisms.

# * * * * End of changes * * * *