3GPP TSG-SA WG2 Meeting #15-e	S2-2209038

# E-Meeting, 10th – 17th Oct 2022

Title:	[draft]LS Out on NSSRG restriction on Pending NSSAI

Response to:	S2-2205406/ S2-2205406/ C1-224073

Release:	Rel-17

Work Item:	eNS_Ph2

Source:	SA2

To:	CT1

Cc:	None

Contact Person:

Name:	Xiaowen Sun

E-mail Address:	sunxiaowen@vivo.cn

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

1. Overall Description:

SA2 thanks for CT1 LS on the NSSRG restriction on Pending NSSAI. SA2 would like to provide the following feedback:

Question: Should the UE take the S-NSSAIs in the Pending NSSAI into account when applying NSSRG restrictions in the Requested NSSAI?

Answer:

When UE requests a new Requested NSSAI, whether the value of NSSRG needs to consider the value of NSSRG in the Pending NSSAI, there has two possibilities:

1. If the NSSRG restriction does not include the NSSRG value of Pending NSSAI, a UE may use a new Requested NSSAI to access into network, instead of sharing a common NSSRG value with the slice in Pending NSSAI.  Under this circumstance, it will weaken the effect of NSSRG restriction, and conflicts may occur due to the deployment, for example the new Requested NSSAI does not share the same AMF with Pending NSSAI.

2. If the NSSRG restriction includes the NSSRG value of Pending NSSAI, the UE may lose part of its flexibility to select a new set of network slice for registration with Pending NSSAI. UE can only choose to register the new S-NSSAIs which has the same NSSRG value with the Pending NSSAI, and the S-NSSAIs which has the different value of NSSRG will have no chance to be used.

Therefore, an indication is required to allow UE and network to consider the NSSRG value of Requested NSSAI by ignoring Pending NSSAI, in order to increase the flexibilities of network slice service.  Meanwhile, for the possibilities of NSSAA which take long time or UE which don’t support the feature of NSSRG, SA2 asks CT1 to take care of the scenarios.

SA2 has agreed the attached CRs.

2. Actions:

To CT1 working group.

ACTION: 	SA2 kindly asks CT1 to take the above information into account.

3. Date of Next SA2 Meetings:

3GPP TSG SA2#154e		14-18, Nov,2022	E-Meeting

3GPP TSG SA2#155e		20-24, Feb,2023	E-Meeting