3GPP TSG-WG SA2 Meeting #153 e-meeting 	S2-2209056

# Elbonia, October 10 - 17, 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.402 | CR | 2999 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | User plane QoS differentiation via control plane |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Peraton Labs, CISA ECD, AT&T, Verizon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | MPS_WLAN |  |  |  |  |  | Date: |  |  | 2022-09-30 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | As per discussion paper S2-2209054, an operator may use User plane QoS differentiation via control plane to send a UE traffic profile and instruction(s) to the Untrusted WLAN to ask for WLAN QoS treatment. |  |  |  |  |  |  |  |  |
| Summary of change: |  | #### New clause on User plane QoS differentiation via control plane feature. #### New clause that describes how/where existing procedures are expanded to support this feature. The feature reuses SWm and SWa Diameter procedures. - Add reference to a stage 3 Diameter AAA interfaces specification. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No reliable, secure approach to send an Untrusted WLAN a traffic profile and an instruction(s) on how the access network (i.e., WLAN) may treat these traffic profiles. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.10.X, 7.X, 2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS/TR 23.401 CR 3714… |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

### 4.10.X	User Plane QoS Differentiation via Control Plane

This clause specifies an optional feature that enables the 3GPP system to send to the Untrusted non-3GPP access network traffic profiles and instructions on how the access network (i.e., WLAN) may treat these traffic profiles.

The ePDG provides the AAA Server a traffic profile(s) and instruction(s) for a UE by using SWm Authorization. The AAA Server relays the traffic profile and instruction(s) to the WLAN via the SWa Re-Authentication and Re-Authorization procedure, per TS 29.273 [xxx].

For this release, this feature is specified for use with GTP on S2b.

There are two types of traffc profiles:

- A traffic profile including UE IP address (local address assigned by WLAN) and the ePDG IP address; or

- A traffic profile including UE IP address (local address assigned by WLAN), the ePDG IP address and IPSec SA(s), identifying any IPSec SA as described in clause 4.10.5.1, clause 4.10.5.2, and clause 4.12.

The instruction(s) include an indication to turn on/off a WLAN QoS mechanism. The instruction may require a specific trigger such as a specific QoS. The mapping between trigger and instruction is with the ePDG.

NOTE 1:	The WLAN may interpret this instruction and potentially use other information (e.g., subscription information) to select an appropriate WLAN QoS mechanism at the discretion of the WLAN.

NOTE 2:	The WLAN QoS mechanism is outside the scope of 3GPP.

NOTE 3:	For MPS, the instruction is an indication to turn on/off priority. The trigger is MPS specific QoS.

The ePDG sends the traffic profile and instruction(s) to the AAA Server and the AAA Server transfers this to the WLAN, as described in clause 7.X.

SECOND CHANGE

## 7.X User Plane QoS Differentiation via Control Plane

This clause describes the procedures used for User Plane QoS Differentiation via Control Plane, per clause 4.10.X.

The ePDG sends a traffic profile and instruction(s) for a specific UE to the AAA server over SWm with an ePDG- initiated Authorization, per TS 29.273 [xx], and the AAA server transfers this to the WLAN over SWa using the SWa Re-authentication and Re-authorization, per TS 29.273 [xx]. In the case of roaming, this shall also include SWd between the AAA proxy and the AAA server.

NOTE:	In each case, a trigger may need to be specified for the ePDG that dictates the particular instruction. For example, in the case of MPS, the receipt of MPS specific QoS, depending on one of the following procedures, leads to a “Priority on” or “Priority off” instruction.

-	in clause 7.2.4, and in clause 7.6.3 for each additional PDN, when the ePDG receives an indication of a successfully created IPSec SA before step E.1, the ePDG sends to the AAA Server the traffic profile and a turn on instruction.

-	in clause 7.4 when the ePDG received an indication of a successfully deleted IPSec SA after step B.1, and similarly in clause 7.4.4 in step A.1, the ePDG sends to the AAA Server the traffic profile and a turn off instruction.

-	in clause 7.9.2, when the ePDG receives an indication of a successfully deleted IPSec SA in step 3a, the ePDG sends to the AAA Server the traffic profile(s) and a turn off instruction(s).

-	in clause 7.10, when the ePDG receives a successfully created IPSec SA in step 4, the ePDG sends to the AAA Server the traffic profile(s) and a turn on instruction(s).

-	in clause 7.11.1, when the ePDG receives an indication of a successfully deleted IPSec SA in step 4, the ePDG may send to the AAA Server the traffic profile(s) and an appropriate turn on/off instruction(s) depending on the change of the bearer QoS.

-	THIRD AND LAST CHANGE

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	ITU-T Recommendations I.130: "Method for the characterization of telecommunication services supported by an ISDN and network capabilities of an ISDN".

[3]	ITU-T Recommendation Q.65: "The unified functional methodology for the characterization of services and network capabilities".

[4]	3GPP TS 23.401: "General Packet Radio Service (GPRS) Enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access".

[5]	3GPP TS 23.234: "3GPP System to Wireless Local Area Network (WLAN) Interworking; System Description".

[6]	3GPP TS 36.300: "Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Overall description Stage 2".

[7]	Void.

[8]	IETF RFC 5213: "Proxy Mobile IPv6".

[9]	IETF RFC 7296: "Internet Key Exchange Protocol Version 2 (IKEv2)".

[10]	IETF RFC 5555: "Mobile IPv6 support for dual stack Hosts and Routers (DSMIPv6)".

[11]	IETF RFC 3748: "Extensible Authentication Protocol (EAP)".

[12]	IETF RFC 5944: "IP Mobility Support for IPv4, revised".

[13]	Void.

[14]	Void.

[15]	IETF RFC 4282: "The Network Access Identifier".

[16]	3GPP TS 23.003: "Numbering, addressing and identification".

[17]	IETF RFC 5844: "IPv4 Support for Proxy Mobile IPv6".

[18]	IETF RFC 4555: "IKEv2 Mobility and Multihoming Protocol (MOBIKE)".

[19]	3GPP TS 23.203: "Policy and Charging Control Architecture".

[20]	3GPP TS 22.278: "Service requirements for evolution of the system architecture".

[21]	3GPP TS 23.060: "General Packet Radio Service (GPRS); Service description; Stage 2".

[22]	IETF RFC 4877: "Mobile IPv6 Operation with IKEv2 and the Revised IPsec Architecture".

[23]	IETF RFC 2784: "Generic Routing Encapsulation (GRE)".

[24]	IETF RFC 2890: "Key and Sequence Number Extensions to GRE".

[25]	IETF RFC 3543: "Registration Revocation in Mobile IPv4".

[26]	Void.

[27]	Void.

[28]	IETF RFC 2131: "Dynamic Host Configuration Protocol".

[29]	IETF RFC 4039: "Rapid Commit Option for the Dynamic Host Configuration Protocol version 4 (DHCPv4)".

[30]	IETF RFC 3736: "Stateless Dynamic Host Configuration Protocol (DHCP) Service for IPv6".

[31]	Void.

[32]	3GPP2 C.S0024-A v2.0: "cdma2000 High Rate Packet Data Air Interface Specification".

[33]	Void.

[34]	IETF RFC 2794: "Mobile IP Network Access Identifier Extension for IPv4".

[35]	Void.

[36]	Void.

[37]	Void.

[38]	IETF RFC 4861: "Neighbor Discovery for IP Version 6 (IPv6)".

[39]	IETF RFC 5446: "Service Selection for Mobile IPv4".

[40]	IETF RFC 5026: "Mobile IPv6 bootstrapping in split scenario".

[41]	IETF RFC 6611: "Mobile IPv6 (MIPv6) Bootstrapping for the Integrated Scenario".

[42]	Void.

[43]	IETF RFC 5779: "Diameter Proxy Mobile IPv6: Mobile Access Gateway and Local Mobility Anchor Interaction with Diameter Server".

[44]	IETF RFC 5447: "Diameter Mobile IPv6: Support for Network Access Server to Diameter Server Interaction".

[45]	3GPP TS 33.402: "3GPP System Architecture Evolution: Security aspects of non-3GPP accesses".

[46]	3GPP TS 31.102: "Characteristics of the Universal Subscriber Identity Module (USIM) application".

[47]	3GPP TS 22.011: "Service accessibility".

[48]	IETF RFC 3948: "UDP Encapsulation of IPsec ESP Packets".

[49]	3GPP2 C.S0087-0: "E-UTRAN - HRPD and CDMA2000 1x Connectivity and Interworking: Air Interface Aspects".

[50]	IETF RFC 4739: "Multiple Authentication Exchanges in the Internet Key Exchange (IKEv2) Protocol".

[51]	3GPP2 X.S0057-B: "E-UTRAN - eHRPD Connectivity and Interworking: Core Network Aspects".

[52]	3GPP TS 36.331: "Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Resource Control (RRC); Protocol specification".

[53]	3GPP TS 23.122: "Non-Access-Stratum (NAS) functions related to Mobile Station (MS) in idle mode".

[54]	Void.

[55]	3GPP TS 23.261: "IP Flow Mobility and seamless WLAN offload; Stage 2".

[56]	IETF RFC 6276: "DHCPv6 Prefix Delegation for Network Mobility (NEMO)".

[57]	3GPP TS 29.274: "3GPP Evolved Packet System (EPS); Evolved General Packet Radio Service (GPRS) Tunnelling Protocol for Control plane (GTPv2-C); Stage 3".

[58]	IETF RFC 4862: "IPv6 Stateless Address Autoconfiguration".

[59]	Void.

[60]	3GPP TS 23.221: "Architectural requirements".

[61]	3GPP TS 32.240: "Charging architecture and principles".

[62]	3GPP TS 32.251: "Charging management; Packet Switched (PS) domain charging".

[63]	3GPP TS 29.281: "General Packet Radio System (GPRS) Tunnelling Protocol User Plane (GTPv1-U)".

[64]	IEEE Std 802.11-2012: "IEEE Standard for Information technology - Telecommunications and information exchange between systems - Local and metropolitan area networks - Specific requirements - Part 11: Wireless LAN Medium Access Control (MAC) and Physical Layer (PHY) Specifications".

[65]	IEEE Std 802.1X-2004: "IEEE Standard for Local and Metropolitan Area Networks: Port-Based Network Access Control".

[66]	IETF RFC 791: "Internet Protocol".

[67]	IETF RFC 2460: "Internet Protocol, Version 6 (IPv6) Specification".

[68]	IETF RFC 6106: "IPv6 Router Advertisement Options for DNS Configuration".

[69]	3GPP TS 36.413: "Evolved Universal Terrestrial Radio Access Network (E-UTRAN); S1 Application Protocol (S1AP)".

[70]	3GPP TS 29.276: "3GPP Evolved Packet System (EPS); Optimized handover procedures and protocols between E-UTRAN access and cdma2000 HRPD Access; Stage 3".

[71]	IETF RFC 768: "User Datagram Protocol".

[72]	IETF RFC 5448: "Improved Extensible Authentication Protocol Method for 3rd Generation Authentication and Key Agreement (EAP-AKA')".

[73]	3GPP TS 24.312: "Access Network Discovery and Selection Function (ANDSF) Management Object (MO)".

[74]	3GPP TS 23.228: "IP Multimedia Subsystem (IMS); Stage 2".

[75]	WiFi Alliance Technical Committee, Hotspot 2.0 Technical Task Group: "Hotspot 2.0 (Release 2) Technical Specification", 2013-04-09.

Editor's note:	The above document cannot be formally referenced until it is publically available in accordance with TR 21.801 and until it is designated as an approved specification.

[76]	3GPP TS 24.244: "Wireless LAN control plane protocol for trusted WLAN access to EPC".

[77]	ETSI ES 282 004 v3.4.1 (2010): "Telecommunications and Internet converged Services and Protocols for Advanced Networking (TISPAN); NGN Functional Architecture; Network Attachment Sub-System (NASS)".

[78]	3GPP TS 25.331: "Universal Terrestrial Radio Access (UTRA); Radio Resource Control (RRC); Protocol specification".

[79]	3GPP TS 36.304: "Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) procedures in idle mode".

[80]	3GPP TS 25.304: "User Equipment (UE) procedures in idle mode and procedures for cell reselection in connected mode".

[81]	IETF RFC 3633: "IPv6 Prefix Options for Dynamic Host Configuration Protocol (DHCP) version 6".

[82]	IETF RFC 7148: "Prefix Delegation Support for Proxy Mobile IPv6".

[83]	3GPP TS 23.167: "IP Multimedia Subsystem (IMS) emergency sessions".

[84]	3GPP TS 36.463: "Evolved Universal Terrestrial Radio Access Network (E-UTRAN) and Wireless LAN (WLAN); Xw application protocol (XwAP)".

[85]	IETF RFC 6696: "EAP Extensions for the EAP Re-authentication Protocol (ERP)".

[86]	WFA: "Wi-Fi Multimedia Technical Specification, version 1.2.0".

[87]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[88]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[xx]	3GPP TS 29.273: “Evolved Packet System (EPS); 3GPP EPS AAA interfaces”.

END OF CHANGES