3GPP TSG-WG SA2 Meeting #153E e-meeting 	S2-2209075

Elbonia, October 10 – 17, 2022	(revision of S2-220xxxx)

Source:	Huawei, HiSilicon

Title:	Update of Key issue #2 conclusion

Document for:	Approval

Agenda Item:	9.10

Work Item / Release:	FS_eLCS_Ph3 / Rel-18

Abstract: this paper is to update conclusion of key issue #2.

1. Introduction/Discussion

In the key issue#2 conclusion, there is an Editor’s note whether the UE-associated NRPPa response message is routed by UE’s serving AMF or can be a local AMF/any AMF which does not have the UE context.

The transactions compose of two part:

Downlink direction (request):

LMF->serving AMF: LMF invokes AMF_N1N2MessageTransfer Service Operation, and in the message body include N2 Information Container (NRPPa message)

AMF->NG RAN, AMF looks up UE context to determine the serving NG-RAN. Further, AMF extracts the NRPPa message in the N2 Information Container, and capsulate it with NG-AP message header, setting the message title “UE associated NRPPa message”.

Uplink direction(response): NG-RAN ->AMF->LMF.

It is a common behaviour that the response shall send back to the entity sending the request message, i.e. serving AMF in this case. Because serving AMF is waiting for the response after it sends a UE level NG-AP message, like for all the other cases. If the serving AMF does not receive a response, it will re-send the message for implementation-specific times. The serving AMF will indicate to the LMF a failure indication if no response message received from RAN.

Therefore, the Editor’s note should be resolved to explicitly describe that RAN responds UE Associated Network Positioning message to LMF via the serving AMF.

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-71.

# * * * * First change * * * *

8.2	Key Issue #2: enhanced positioning architecture for NPN deployment

The Local AMF/Any AMF in NPN deployment is used to transfer the Non-UE Associated Network Assistance Data between the RAN and LMF, as described in solution 4 and solution 5.

For UE Associated Network Positioning procedure, the LMF sends the UE Associated Network Positing message towards the RAN via the serving AMF. RAN responds UE Associated Network Positioning message to LMF via the serving AMF.

The LMF notifies the UE location to GMLC directly, i.e. without going through the serving AMF.

# * * * * End of changes * * * *