3GPP TSG-WG SA2 Meeting #153E e-meeting 	S2-2209077

Elbonia, October 10 – 17, 2022	(revision of S2-220xxxx)

Source:	Huawei, HiSilicon

Title:	Key issue#12 conclusion

Document for:	Approval

Agenda Item:	9.12

Work Item / Release:	FS_eLCS_Ph3 / Rel-18

Abstract: the paper is to propose key issue #12 conclusion.

1. Introduction/Discussion

Key issue#12 only has one solution documented in the TR, so it makes sense to capture the principle in that paper as conclusion.

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-71.

# * * * * First change (all texts new)* * * *

## 8.12	Key Issue #12: support of low power and/or high accuracy positioning

The following aspects are concluded for normative work:

- UDM is enhanced to include LPHAP indication in the UE LCS subscription data.

- During the positioning procedure, AMF provides the LPHAP indication to the LMF, either obtaining from the GMLC, or in the UE LCS context which received during UE registration procedure.

- LMF is enhanced to receive from AMF of the LPHAP indication in the location request, and determine positioning method, by taking into account the LPHAP requirement. LMF also sends LPHAP indication to RAN in the NRPPa message.

NOTE: RAN impact needs to coordinate with RAN WG, e.g. whether LPHAP indication needs to be sent to RAN at an earlier time, before positioning procedure is triggered.

# * * * * End of changes * * * *