3GPP TSG-SA2 Meeting # 151E (e-meeting)	S2-2209091

October 10 – 17 2022, Elbonia

Source: 	Samsung

Title:	UE Policy Association Establishment when AMF relocation with PCF change

Document for:	Approval

Agenda Item:	4.1

Work Item / Release:	TEI17 / Rel-17

Abstract of the contribution: this DP is triggered by the LS S2-2208132/C3-224697

- 1 Discussion

In LS from CT3, the following issue and questions are asked:

Some CT3 companies understand that, at the reception of the UE Policy Association creation request, the new selected PCF interprets that the UE is triggering the 5GS Initial Registration and the UE does not have any UE Policy Section stored for the HPLMN and/or serving PLMN. In this case, there is the following issue:

If the UE stored, due to a previous UE Policy Provisioning procedure, e.g. UE_PolicySection_1, UE_PolicySection_2 and UE_PolicySection_3, and the new selected PCF for the new AMF determines that only e.g. UE_PolicySection_1 and UE_PolicySection_3 apply, the new selected PCF will provide to the UE the UE_PolicySection_1 and the UE_PolicySection_3 (which is unnecessary, because they are already stored in the UE) and the new selected PCF will not be able to remove UE_PolicySection_2 in the UE (which was the only required action in this example).

After this procedure, the UE will remain misconfigured until any trigger makes the UE can invoke a 5GS Initial Registration with the indication of the stored PSIs, ANDSP and UE OSId as applicable. Meanwhile, the PCF will not reflect either the actual PSIs stored in the UE.

Question 1: At the reception of UE Policy Association creation request, when no indication about the list of stored PSIs, ANDSP support, and UE OSId is received, shall the PCF interpret that the UE is triggering initial registration and the UE does not have UE Policies stored? If not, how does the PCF determine it is a different scenario?

Question 2: Additionally, since the new selected PCF does not receive information about the UE's support for ANDSP or UE’s support of OSIds, which affect to the proper determination of the ANDSP, how does the ANDSP policies

The case above in terms of list of PSIs can be depicted and summarized as Figure 1 in below:

Figure 1 UE Policy Association Establishment with PCF re-selection during AMF relocation

- UE to New AMF: Registration Request. UE does not provide the stored list of PSIs.

- Old AMF to New AMF: The information about UE Policy Association (including Trigger information, PCF ID). In the roaming case, V-PCF ID and H-PCF ID are included.

- New AMF: Decision to use new PCF.

- New AMF to New PCF: UE Policy Control Create Request. No list of PSIs is included.

- New PCF and UDR: In Initial Registration case, the new PCF retrieves the list of PSIs and its content stored in the UDR for this SUPI.

- New PCF: Policy decision with the list of PSIs and its contents received from the UDR.

At bullet 4, the New PCF will interpret that the UE is triggering initial registration and UE does not have UE Polices stored. However, this interpretation does not make any problem if the new PCF can receive it from the UDR. According to current TS 23.503, the PCF retrieves the list of PSIs and its content stored in the UDR:

During Initial Registration, the (H-)PCF retrieves the list of PSIs and its content stored in the (H-)UDR for this SUPI while the V-PCF (in the roaming scenario) retrieves the list of PSIs and its content stored in the V-UDR for the PLMN ID of this UE (alternatively, the V-PCF can have this information configured locally).

If the new PCF determines it is an initial registration request, then the new PCF will retrieve the list of PSIs and its content from the UDR, and actually it is the latest list of PSIs delivered to the UE (bullet 5). Therefore, the new PCF can make appropriate decision (bullet 6).

It is assumed that the old PCF and the new PCF are connecting to the same UDR. There is not much information in the LS that how the New PCF can determine that only PSI#1 and PSI#3 are applying when the PSI#1, PSI#2, and PSI#3 are installed in the UE. However, according to the current specifications, the PCF maintains the latest list of PSIs delivered to the UE and stores it to UDR. Therefore, it is also assumed that there’s another input to New PCF from other than the UDR and the New PCF compares it with the latest list of PSIs and its content received from the UDR, and performs necessary updates (e.g. just removing PSI#2). So there’s no difference in PCF’s behavior with the real Initial Registration case.

Observation 1: When the new PCF interpret that the UE is triggering initial registration and the UE does not have UE Policies stored, the PCF retrieves the list of PSIs and its content stored in the UDR which are actually the latest list of PSIs and its contents delivered to the UE. Therefore, the new PCF can make appropriate decision on the list of PSIs to be delivered to the UE via new (relocated) AMF. So there’s no difference in PCF’s behavior compared to the real Initial Registration case.

However, the spec text can be clarified as below since the PCF does not distinguish initial registration and AMF relocation with PCF change when the UE Policy Association Establishment is triggered:

When the UE Policy Association Establishment is triggered and UE does not provided the list of PSIs stored in the UE, the (H-)PCF retrieves the list of PSIs and its content stored in the (H-)UDR for this SUPI while the V-PCF (in the roaming scenario) retrieves the list of PSIs and its content stored in the V-UDR for the PLMN ID of this UE (alternatively, the V-PCF can have this information configured locally).

Proposal 1: Clarify the spec text (TS 23.503) regarding PCF retrieving the list of PSIs and its content stored in the UDR when the UE Policy Association Establishment is triggered and UE does not provided the stored list of PSIs.

The case above in terms of the indication of UE support for ANDSP and the OSId can be depicted and summarized as Figure 2 in below:

Figure 2 UE Policy Association Establishment with PCF re-selection during AMF relocation (including roaming)

- UE to New AMF: Registration Request. UE does not provide the indication of UE support for ANDSP support, and/or OSId.

- Old AMF to New AMF: The information about UE Policy Association (including Trigger information, PCF ID). In the roaming case, V-PCF ID and H-PCF ID are included.

- New AMF: Decision to use new PCF.

- New AMF to New V-PCF: UE Policy Control Create Request. No indication of UE support for ANDSP support, and/or OSId are included.

- New V-PCF to New H-PCF: UE Policy Control Create Request is forwarded.

- New H-PCF and UDR: The new H-PCF gets policy subscription related information and the latest list of PSIs from the UDR. If the AMF relocates and the PCF changes, H-PCF gets the OSId and/or the indication of UE support for ANDSP.

- New-PCF to New V-PCF: UE Policy Control Create Response.

- New PCF: Policy decision with the policy subscription related information (including the indication of UE support for ANDSP and/or OSId).

The same assumption and procedure can be applied for the roaming case as summarized above. For bullet 5, it is specified in step 6 and NOTE 2 in 4.16.11 of TS 23.502 as below:

6. The (H-)PCF gets policy subscription related information and the latest list of PSIs from the UDR using Nudr_DM_Query service operation (SUPI, Policy Data, UE context policy control data, Policy Set Entry) if either or both are not available and makes a policy decision. The (H-)PCF may get the PEI, the OSId or the indication of UE support for ANDSP in the UDR using Nudr_DM_Query including DataSet "Policy Data" and Data Subset "UE context policy control data" if the AMF relocates and the PCF changes. ……

NOTE 2:	Step 6 (and step 7) can be omitted. Then the (H-)PCF creates the UE policy container including UE polices in step 2 (in the case of non-roaming) or step 3 (in the case of roaming). This means that the potential interactions with UDR as in step 6 will have to be executed in step 2 (non-roaming) or step 3 (roaming).

Observation 2: The new (H-)PCF gets policy subscription related information from the UDR since it is not available at the new (H-)PCF. In AMF relocation with PCF change case, the (H-)PCF gets the OSId or the indication of UE support for ANDSP from the UDR.

However, the spec text does not explicitly described whether the policy subscription related information is forwarded from H-PCF to V-PCF in roaming case is not explicitly described. It can be clarified as below, so that the V-PCF can make appropriate policy decision based on UE’s support for ANDSP or OSId as well;

6. The (H-)PCF gets policy subscription related information and the latest list of PSIs from the UDR using Nudr_DM_Query service operation (SUPI, Policy Data, UE context policy control data, Policy Set Entry) if either or both are not available and makes a policy decision. The (H-)PCF may get the PEI, the OSId or the indication of UE support for ANDSP in the UDR using Nudr_DM_Query including DataSet "Policy Data" and Data Subset "UE context policy control data" if the AMF relocates and the PCF changes. In roaming case, H-PCF provides the policy subscription related information to V-PCF. ……

Proposal 2: Clarify the spec text (TS 23.502) regarding PCF(s) retrieving the policy subscription related information from the UDR during the UE Policy Association Establishment when the AMF relocates and the PCF changes.

# 2	Proposal

It is proposed to approve following changes:

Proposal 1: Clarify the spec text (TS 23.503) regarding PCF retrieving the list of PSIs and its content stored in the UDR when the UE Policy Association Establishment is triggered and UE does not provided the stored list of PSIs. The change is proposed in S2-2209095.

Proposal 2: Clarify the spec text (TS 23.502) regarding PCF(s) retrieving the policy subscription related information from the UDR during the UE Policy Association Establishment when the AMF relocates and the PCF changes. The change is proposed in S2-2209093.

Proposal 3: CT3’s questions can be answered based on discussions above. The LS reply is proposed in S2-2209089.