3GPP TSG-SA WG2#153E e-meeting	S2-2209111

Elbonia, 10-14th October, 2022

Source:	Qualcomm Incorporated

Title:	Update to conclusions for KI#1: Support of UE-to-UE relay

Document for:	Discussion/Approval

Agenda Item:	9.26

Work Item / Release:	FS_5G_ProSe_Ph2/Rel-18

Abstract of the contribution: This paper discusses changes to the conclusions in clause 8.1, Key Issue #1: Support of UE-to-UE Relay.

# 1.	Discussion

Conclusions Clause 8.1, Key Issue #1: Support of UE-to-UE Relay, has the below conclusion

The following conclusions are specific for Layer-3 UE-to-UE Relay:

-	For UE-to-UE Relay Per-hop links setup (i.e. PC5 link establishment between Source UE and UE-to-UE Relay, as well as between UE-to-UE Relay and Target UE), Source UE initiates the PC5 link setup with UE-to-UE Relay, and UE-to-UE Relay initiates the PC5 link setup with the target UE.

Above conclusion is applicable for both Layer-3 and Layer-2 relay per-hop link setup. So we propose some editorial changes in that context.

Some solutions (sol#11, sol#12,sol#34), consider the case where the same unicast link can be shared for relaying traffic between different pair of source UEs and target UEs via the UE-to-UE relay UE. This per hop unicast link sharing aspect is missing in the conclusions section and it is essential to have this architectural requirement captured in the study conclusions to support efficient discussions during normative work.

# 2.	Text proposal

It is proposed to consider the below updates to the conclusions section for Key Issue #1: Support of UE-to-UE Relay.

# >>>>BEGINNING OF CHANGES<<<<

# 8	Conclusions

Editor's note:	This clause will list conclusions that have been agreed during the course of the study item activities.

## 8.1	Key Issue #1: Support of UE-to-UE Relay

For Key Issue #1 (Support of UE-to-UE Relay), the followings are taken as conclusions:

The following conclusions are common for both Layer-3 UE-to-UE Relay and Layer-2 UE-to-UE Relay:

-	For UE-to-UE Relay discovery, both Model A and Model B discovery are supported.

Editor's note:	It is FFS the exact information to be included in discovery messages.

Editor's note:	It is FFS if discovery integrated into PC5 unicast link establishment procedure is supported.

-	For UE-to-UE Relay selection, the Source UE performs the UE-to-UE Relay selection for both Model A and Model B discovery.

Editor's note:	It is FFS if Target UE can also perform the UE-to-UE Relay selection for Model B discovery.

-	For UE-to-UE Relay reselection, the negotiated UE-to-UE Relay reselection between Source UE and Target UE and the UE-to-UE Relay selection procedure can be used under different conditions.

NOTE 1:	UE-to-UE Relay selection/reselection requires cooperation with RAN WGs during normative work.

-	IP, Ethernet and Unstructured traffic types are supported.

NOTE 2:	Ethernet and Unstructured traffic types can be encapsulated in IP traffic type if supported by source and target UE.

NOTE 3:	Conclusion on Service Authorization information related to UE-to-UE Relay operation sent by AMF to NG-RAN can be finalized based on feedback from RAN WGs.

The following conclusions are specific for Layer-3 UE-to-UE Relay:

-	For UE-to-UE Relay Per-hop links setup (i.e. PC5 link establishment between Source UE and UE-to-UE Relay, as well as between UE-to-UE Relay and Target UE), Source UE initiates the PC5 link setup with UE-to-UE Relay, and UE-to-UE Relay initiates the PC5 link setup with the target UE.

- The PC5 link between a 5G ProSe U2U UE and 5G ProSe UE-to-UE Relay can be shared by multiple E2E links belonging to the 5G ProSe U2U UE.

- The establishments of the E2E links for the 5G ProSe U2U UE can be separate from the establishment of PC5 link between 5G ProSe U2U UE and 5G ProSe UE-to-UE Relay.

The following conclusions are specific for Layer-3 UE-to-UE Relay:

-	For QoS control of Layer-3 UE-to-UE Relay, the UE-to-UE Relay receives E2E QoS from Source UE and determines the per-hop QoS parameters to satisfy the E2E QoS.

The following conclusions are specific for Layer-2 UE-to-UE Relay:

-	Per-hop links (i.e. PC5 link between Source UE and UE-to-UE Relay, as well as between UE-to-UE Relay and Target UE) needs to be established before E2E PC5 link establishment is performed.

NOTE 4:	How the E2E PC5-S messages are forwarded by the UE-to-UE Relay is to be determined by RAN WGs.

NOTE 5:	For Layer-2 UE-to-UE Relay, RAN WGs will define how the E2E QoS will be handled and split over the PC5 links.

# >>>>END OF CHANGES<<<<