SA WG2 Meeting #153e	S2-2209127

# Oct 10th – 14th , 2022;

Source: 	Nokia, Nokia Shanghai Bell

Title: 	KI#8 – Update to Solution #33: Support of LCS continuity when UE moves between 5GS and EPS

Document for: 	Approval

Agenda Item: 	9.10

Work Item / Release:	FS_eLCS_Ph3 / Rel-18

Abstract of the contribution: This paper proposes an update to the solution #33 for KI#8

# 1	Discussion

The FS_eLCS_Ph3 SID objectives reads

- WT#6: Study  enhancements to support the location service continuity for UE mobility, i.e. between EPS and 5GS;

Key Issue #8: support of location service continuity in case of UE mobility says the following

- For commercial location service, in particular the use case on vehicle(V2X) UE, it is very likely that UE moves between EPS and 5GS, and continuous UE positioning (periodical location service/LDR) is required. Following UE mobility scenarios are considered under this key issue:

- UE mobility between EPS and 5GS (bi-direction).

- UE mobility between NG-RAN node.

This proposal provides an update to solution #33 for LCS continuity between EPS and 5GS in both directions..

# 2 Proposal

It is proposed to update TR 23.700-71-"Study enhancement to the 5GC LoCation Services (LCS); Phase 3" as follows

*** Start of changes ***

## 6.0	Mapping Solutions to Key Issues

Table 6.0-1: Mapping Solutions to Key Issues

|  | Key Issues |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Solutions | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 |  |  |  |
| X |  |  |  |  |  |  |  | X |  |  |  |  |  |  |  |

*** Next Change ***

### 6.x	Solution #8: Support of LCS mobility when UE moves between  5GS and EPS

### 6.X.1	Introduction

This solution addresses the KI “Key Issue #8: support of location service continuity in case of UE mobility”.

Continuity of certain basic services like a PDU session during mobility is defined currently. However, the handling of an LCS session during mobility procedures is not specified. In particular, there is no specification on whether an ongoing LCS session shall be terminated or continued with after the mobility and if continued, how the continuity shall work. There can be many effects due to this missing part

- LCS sessions may be terminated at mobility with the LCS Client having to re-initiate the session after the procedure.

- The LMF may not be aware of the mobility at all, leading to inaccurate positioning estimates

- Since there are no clear specifications, inter-vendor tests could fail

- LCS sessions could be enabled as part of other critical services (like emergency calls) – those services could be impacted due to the current specifications – leading to regulatory issues.

All of the above issues could be prevented if there is a clear specification on how LCS sessions are to be handled in different mobility procedures and in particular, if continuity can be guaranteed by the network.

The main considerations in the solution are:

- Keep the changes minimal in EPC so that legacy functionality requires only a small incremental change

- Not needing a standardized interface between the EPC-GMLC and 5GC-GMLC. As per section 4.2a. in TS 23.273, this interface, denoted as Lr’ is not standardized and in case EPC-GMLC and 5GC-GMLC are co-located, Lr’ is not needed altogether. This solution doesn’t require the standardization of this interface.

-  provide a common procedure for all types of Location Request (MO-LR/MT-LR with/without deferred location)

- provide a simple solution that works regardless of the current status of the on-going location procedure when the mobility event occurs

### 6.X.2	Functional Description

This proposal provides solutions for mobility between 5GS and EPS.

### 6.X.3	Procedure

Fig.1 : LCS Continuity Solution for 5GS to EPS Mobility

Description :

Steps 1-4 describe the normal MT-LR LCS session on 5GS (similar to steps 1.a. to 12 in Fig. 6.1.2-1 in TS 23.273.

Steps 5-14 describe the normal 5GS to EPS HO procedure (similar to steps 1 to 12.b in Fig. 4.11.1.2.1-1 in TS 23.502

The AMF could initiate steps 15 and 17 in parallel to Step 11 when it receives the Relocation Response (Step 10)

Note : Steps 15/16 is meant to be completed before the UE is handed over to LTE. If it is determined to be too close to the HO procedure, then the normative phase could choose to trigger Step 15 in parallel with Step 7.

Steps 15-16 : The LCS Session on 5GS is cancelled

Step 17 : The AMF indicates failure to the 5GC-GMLC with reason “HO to EPC” and including the target MME ID

Step 18 : The 5GC-GMLC forwards the location request to the EPC-GMLC including all the parameters of the LCS Request and the target MME ID

Steps 19-24 : The LCS Session is started on EPS (similar to steps 1-10 in Fig. 9.18 in TS 23.271)

Steps 25-26 : The UE location estimate is returned by the EPC-GMLC to the 5GC-GMLC which then forwards the same to the LCS Client

Note :

- As can be observed, the entire procedure is done in a way where the LCS Client is not aware of the mobility handling and the LCS session continuity is ensured.

- There could be a mapping of LCS QoS (from the values defined for 5GS to values defined in 4GS) – this could be done by the GMLC.

  - For QoS attributes that are common between 5GS and EPS, the original LCS QoS defined values can be used as is in EPS.

  - For QoS attributes that are specific to 5GS -> the QoS mapping could be done to the most stringent QoS value with best effort handling in QoS Class.

- In case an LCS session was active over user plane in 5GS when the HO is triggered, the AMF/LMF share assistance data to the UE that minimizes the TLS handshake workload as per clause 6.1.1.4 in OMA-TS-ULP specification, once the UE moves to EPS

Fig.2 : LCS Continuity Solution for EPS to 5GS Mobility

Description :

Steps 1-4 describe the normal MT-LR LCS session on EPS (similar to steps 1 to 3 in Fig. 6.1.2-1 in TS 23.271.

Steps 5-14 describe the normal EPS to 5GS HO procedure (similar to steps 1 to 16 in Fig. 4.11.1.2.2.2-1 and steps 1-4 in 4.11.1.2.2.3-1in TS 23.502)

The MME could initiate steps 15 and 17 in parallel to Step 11 when it receives the Forward Relocation Response (Step 10)

Steps 15-16 : The LCS Session on EPS is cancelled

Step 17 : The MME indicates failure to the EPC-GMLC with reason “HO to EPC” and including the target AMF ID

Step 18 : The EPC-GMLC forwards the location request to the 5GC-GMLC including all the parameters of the LCS Request and the target AMF ID

Steps 19-24 : The LCS Session is started on 5GS (similar to steps 1-23 in Fig. 6.1.2-1 in TS 23.273)

Steps 25-26 : The UE location estimate is returned by the 5GC-GMLC to the EPC-GMLC which then forwards the same to the LCS Client

Note :

- As can be observed, both the procedures are defined in a way to keep the LCS Client unaware of the mobility signaling and the LCS session continuity is ensured.

- There could be a mapping of LCS QoS (from the values defined for 5GS to values defined in 4GS or vice versa) – this could be done by the GMLC.

  - For QoS attributes that are common between 5GS and EPS, the original LCS QoS defined values can be used as is in 5GS.

  - For QoS attributes that are specific to EPS -> the QoS mapping could be done to the most stringent QoS value with best effort handling in QoS Class.

- In case the LMF is configured to use LCS over user plane in 5GS, then LMF will choose to initiate the LCS session over user plane at step 24.

### 6.X.4	Impacts on services, entities, and interfaces

|  | GMLC | AMF | MME | LMF/eSMLC/RAN/UE |
| --- | --- | --- | --- | --- |
| 5GS -> EPS Mobility | Transfer of LCS context from 5GS to EPS Mapping of LCS QoS from 5GS to EPS Initiating LCS session on EPS on target MME given by AMF | Provide the target MME ID to GMLC as part of Namf_Location_Response Trigger the sending of LCS UP server assistance data to the UE as part of the HO procedure |  | None |
| EPS -> 5GS Mobility | Transfer of LCS context from EPS to 5GS Mapping of LCS QoS from EPS to 5GS |  | Provide the target AMF ID to GMLC as part of Location_Response | None |

# *** End of changes ***