SA WG2 Meeting #S2-153E	S2-2209175

10 – 14 Oct , 2022, E-meeting

Source:	Xiaomi

Title:	conclusion for key issue#4

Document for:	Approval

Agenda Item:	9.4

Work Item / Release:	FS_eNPN_Ph2/Rel-18

Abstract of the contribution: This paper proposes to update the evaluation and solution#22 for Key Issue#3.

*** Start of changes ***

8.4	Key Issue #4: Enabling UE to discover, select and access NPN as hosting network and receive localized services

Editor's note:	Conclusions for Key issue #4 are FFS.

Hosting network broadcasts a Hosting networking service indication, which is considered by the UE to select the Hosting network, when UE is triggered to access to the localized service.

UE can get the localized services information from hosting network to decide whether the expected localized service can be provided.

UE can access to the hosting network with credentials owned by Credentials Holder or home network.

Hosting network can provide restricted PDU session for UE to access to the localized service when there is no suitable credentials. Secondary authentication may be needed.

*** End of changes ***