SA WG2 Meeting #S2-153E	S2-2209179

10 – 14 Oct. 2022, E-meeting

Source:	Xiaomi

Title:	update conclusion#3 to remove the EN

Document for:	Approval

Agenda Item:	9.26

Work Item / Release:	FS_5G_ProSe_Ph2/Rel-18

Abstract of the contribution: This paper proposes to update the conclusion#3 to remove the EN.

Discussion

As Path switching policy is concluded to indicate which path(s) is permitted for all or specific ProSe services (i.e. PC5 permitted, or Uu permitted, or both PC5 and Uu permitted). But when UE doesn’t know whether the target UE can support the same Path switching policy before performing path switching. It may lead unnecessary signalling when there is unmatched path switching policy between source UE and target UE.

For example, UE 1 may trigger to perform switching from PC5 to Uu path, but UE2 only supports PC5 path, at this time, it is not necessary and will waste resource to perform the path switching.

Proposal: UEs may need further negotiation of the path switching policy of each other before performing path switching.

*** Start of changes ***

8.3	Key Issue #3: Support direct communication path switching between PC5 and Uu (i.e. non-relay case)

For Key Issue #3 (Support direct communication path switching between PC5 and Uu (i.e. non-relay case)), the followings are taken as conclusion:

-	A Path switching policy is used that indicates which path(s) is permitted for all or specific ProSe services (i.e. PC5 permitted, or Uu permitted, or both PC5 and Uu permitted).

NOTE 1:	Whether the path switching policy can be combined into the path selection policy as defined in TS 23.304 [3] will be determined in normative phase.

-	The path switch policy can be (pre)configured in the UE or provided by the PCF.

-	The granularity of path switch is per ProSe service.

-	UEs may further negotiate to determine whether to perform path switching by considering the Path switching policy of target UE and which ProSe services to be are switched.

-	For the path switching from the a PC5 path to a Uu path, one of the UEs may notify the peer UE of being not able to perform the path switching (e.g., under congestion control or mobility restriction) or having no willing to perform the path switching to avoid the peer UE performing the path switching.

-	A make-before-break mechanism is used when path switching between PC5 and Uu, i.e., when path switching occurs the UEs prepare the target path to communicate with each other, then performing the path switching from the PC5 path to Uu path and vice versa and communicate over that target path, and at which point for former path may be released.

NOTE 2:	If UEs maintain a PC5 unicast link after path switching from a PC5 path to a Uu path, the UEs can determine to switch back to the PC5 path from the Uu path based on e.g. the policy related to path switching, the PC5 signal level of the maintained unicast link.

-	For the path switching from the PC5 path to Uu path, the Uu QoS each UE will request from their network using the existing 5GS mechanism is based on PC5 QoS and negotiated via the existing PC5 connection between two UEs.

-	For the path switching from the PC5 path to Uu path, optionally the IP addresses used for Uu path can be shared between the UEs as assist them to establish communication directly with each other via the Uu path.

-	Optionally, in Keep Alive procedure, each UE may inform the peer UE whether it can initiate PDU session establishment corresponding to the ProSe service in use for path switch.

*** End of changes ***