3GPP TSG SA WG2 Meeting #153E (e-meeting)		S2-2209197

October 10-17 2022

Title:	[DRAFT] LS on Satellite coverage data transfer to a UE using UP versus CP

Response to:

Release:	Rel-18

Work Item:	FS_5GSAT_Ph2

Source:	SA2

To:	CT1

Cc:	RAN2, RAN3, SA3

Contact person:	Stephen Edge

sedge AT qti.qualcomm.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	S2-2208540

# 1	Overall description

SA2 has tentatively concluded on supporting transfer of coverage map data to a UE for one or more satellite RATs to enable the UE to determine the timing of satellite coverage gaps. The coverage map data would be transferred from an external server or AF to a UE via user plane (e.g. HTTPS over TCP/IP) or control plane using NAS. The coverage map data would indicate for one or more locations and one or more future times whether a UE can expect to receive or not receive coverage from at least one of the satellite RATs at each of the locations and for each of the future times. The one or more locations may correspond to fixed locations, e.g. corresponding to grid points in a rectangular or hexagonal grid point array, or may correspond to locations along a known or predicted UE trajectory.

SA2 expects to add more stage 2 applicable detail regarding content and transfer of coverage maps during a normative phase of specification.

One aspect of transport where feedback from CT1 would greatly assist the normative phase concerns whether transport could be defined by CT1 at a stage 3 level for each of user plane and control plane transfer and whether CT1 has any preference for either method. SA2 believes that control plane transfer using NAS would have higher 5GCN impact and could possibly restrict the amount of coverage data provided to a UE, but could also allow more PLMN control including better security, ability to support subscription and charging and ability to support a wider range of UE service requests (e.g. such a request based on a known UE trajectory). Conversely, SA2 believes user plane transfer should have lower or even no 5GCN impact and no restriction on the amount of coverage data provided to a UE, but could be less secure, less amenable to subscription and charging and only applicable to a narrow range of UE service requests. SA2 would appreciate comments from CT1 on these aspects also.

# 2	Actions

To CT1

ACTION: 	SA2 asks CT1 to comment on use of user plane versus control plane using NAS to support transfer of coverage map data to a UE for one or more satellite RATs and in particular to indicate whether CT1 would be able to support each of these at a stage 3 level, may have a preference for one or the other and sees any possible issues regarding the amount of coverage data provided to a UE, support of security, subscription and charging and flexibility of UE service requests for either alternative.

# 3	Dates of next TSG SA WG2 meetings

SA2#154E			14-18 November 2022		Toulouse, France

SA2#155E			20-24 February 2023		Europe