SA WG2 Meeting #153E	S2-220xxxx

Elbonia, October 10 – 14, 2022	(revision of S2-220xxxx)

Source:	Intel

Title:	KI#5 Evaluation and Conclusion

Document for:	Approval

Agenda Item:	9.26

Work Item / Release:	FS_ProSe_Ph2 / Rel-18

Abstract: This contribution proposes to update the conclusion for KI#5.

# 1.	Discussion

For below EN:

Editor's note: Whether policy authorization for multi-path transmission via direct Uu path and Layer 3 U2N Relay UE with N3IWF is needed is FFS.

There was different view at SA2#152E on whether policy authorization for multi-path transmission via direct Uu path and Layer 3 U2N Relay UE with N3IWF is needed.

In TS 23.304, it says the Layer 3 U2N Relay UE with N3IWF is regarded as non-3GPP access, thus some people think the current non-3GPP Access preference in the Access Type preference of the RSD can be reused, but in fact it’s not normal non-3GPP access since the normal non-3GPP access doesn’t go through NG-RAN at all. In order for the UE to be able to differentiate it from the normal non-3GPP access, some finer granularity on existing Access Type preference or new RSD component needs to be defined in normative phase.

# 2.	Proposal

It is proposed to capture the following changes into TS 23.700-85.

- * * * * First change * * * *

# 8	Conclusions

Editor's note:	This clause will list conclusions that have been agreed during the course of the study item activities.

## 8.5	Key Issue #5: Support of multi-path transmission for UE-to-Network Relay

For Key Issue #5 "Support of multi-path transmission for UE-to-Network Relay", the followings are taken as conclusions:

-	Policy authorization for multi-path transmission via direct Uu path and Layer 3 U2N Relay UE with and without N3IWF is needed, detailed URSP enhancement design and potential Registration procedure enhancement (e.g. in Sol #25) will be determined in normative phase.

NOTE 1:	Whether and how to authorize the multi-path transmission based on ProSe policy will be determined in normative phase.

-	Multi-path transmission via direct Uu path and Layer 3 U2N Relay UE with N3IWF can rely on MA-PDU Session (introduced in clause 5.32 of TS 23.501 [7]) as described in Sol #28 and Sol #41, no system enhancement is required; in normative phase, how to use the MA PDU Session to support multi-path transmission via direct Uu path and Layer 3 U2N Relay UE with N3IWF will be described for information (e.g. as described in Sol #28 and Sol #41).

Editor's note: Redundancy and duplication for multipath transmission depends on R18 study ATSSS_Ph3 feature.

-	For multi-path transmission via direct Uu path and Layer 2 U2N Relay UE:

-	Multi-path transmission via direct Uu path and Layer 2 U2N Relay will be defined by RAN WGs and alignment work can be made by SA2 based on the progress and outcome in RAN WGs.

- * * * * End of changes * * * *