3GPP TSG-WG SA2 Meeting #153E e-meeting 	S2-2208729

Elbonia, October 10 – 17, 2022	(revision of S2-2206087)

- Source:	Huawei, CBN

- Title:	New WID on Architectural enhancements for 5G multicast-broadcast services Phase 2 (5MBS_Ph2)

- Document for:	Approval

Agenda Item:	10.2

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

# Title: 	Architectural enhancements for 5G multicast-broadcast services Phase 2

## Acronym: 5MBS_Ph2

## Unique identifier:

Potential target Release: Rel-18.

Note that this field above indicates the proposed Release at the time of submission of the WID to TSG approval. It can later be changed without a need to revise the WID. The updated target Release is indicated in the Work Plan.

## 1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes | X | X | X | X |  |
| No |  |  |  |  |  |
| Don't know |  |  |  |  | X |

## 2	Classification of the Work Item and linked work items

### 2.1	Primary classification

This work item is a …

| X | Feature |
| --- | --- |
|  | Building Block |
|  | Work Task |
|  | Study Item |

### 2.2	Parent Work Item

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |
| FS_5MBS_Ph2 | SA2 | 940067 | Study on architectural enhancements for 5G multicast-broadcast services Phase 2 |

### 2.3	Other related Work Items and dependencies

| Other related Work Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
| 940099 | Enhancements of NR Multicast and Broadcast Services | RAN-related WID to standardize the RAN functions for broadcast/multicast in 5GS |
| 960041 | Study on security enhancements for 5G multicast-broadcast services Phase 2 | Security work on 5G Multicast-Broadcast services |
| 930016 | Mission Critical Services over 5MBS | Enable the multicast-broadcast service in Mission critical scenario. |

Dependency on non-3GPP (draft) specification:

{This section is to be typically used to identify the IETF dependencies. Delete the header "Dependency on non-3GPP (draft) specification:" if no such dependency.}

## 3	Justification

In Rel-18 FS_5MBS_Ph2 study, the following enhancements have been studied and concluded:

-	Enabling UE's receiving Multicast MBS Session data in RRC Inactive state.

-	Study feasible and efficient resource utilization for the same broadcast content to be provided to 5G MOCN network sharing scenarios (i.e. multiple CNs are connected to the same NG-RAN).

-	Study whether and how to support on demand multicast MBS session triggered by AF, and efficient resource utilization via 5GC choosing multicast and/or unicast delivery for a certain service.

-	Study whether and how to support group message delivery for capability-limited devices, including NEF enhancement, coexistence of existing power saving mechanisms and MBS.

-	Study whether there are any identified performance issues for high number of public safety UEs, and if yes study necessary enhancements to 5MBS for that scenario.

The WID proposal is to specify the concluded aspects in TR 23.700-47.

## 4	Objective

The objective of this work item is to specify architecture enhancements for 5G multicast-broadcast services as per conclusions reached within TR 23.700-47 (clause 8).

The detailed objectives are as follows:

Support of the enhancement of the multicast and broadcast communication service within 5G system, specifically:

  - Support of UE's receiving Multicast MBS Session data in RRC Inactive state.

  - Support of efficient resource utilization for the same broadcast content to be provided to 5G MOCN network sharing scenarios (i.e., multiple CNs are connected to the same NG-RAN).

  - Support of on demand multicast MBS session triggered by AF, and efficient resource utilization via 5GC choosing multicast and/or unicast delivery for a certain service.

  - Support of group message delivery.

  - Support of coexistence with existing power saving mechanisms for capability-limited devices.

  - Support of public safety demands considering high numbers of UEs.

The normative work will be done in collaboration with RAN WGs.

Editor's note:	The above objectives may be updated based on SA2#153E progress.

## 5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |

{Note 1: Only TSs may contain normative provisions. Study Items shall create or impact only TRs.
"Internal TR" is intended for 3GPP internal use only whereas "External TR" may be transposed by OPs.}

{Note 2: The first listed Rapporteur is the specification primary Rapporteur. Secondary Rapporteur(s) are possible for particular aspect(s) of the TS/TR. In this case, their responsibility has to be provided as "Remarks".}

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| 23.501 | Reference to new TS | TSG#99 (Mar, 2023) |  |
| 23.502 | Possible Updates to existing APIs | TSG#99 (Mar, 2023) |  |
| 23.503 | 5MBS Policy Control | TSG#99 (Mar, 2023) |  |
| 23.247 | Updates to support 5MBS_Ph2 | TSG#99 (Mar, 2023) |  |

## 6	Work item Rapporteur(s)

Meng Li, Huawei, raymond.limeng@huawei.com

## 7	Work item leadership

SA2

## 8	Aspects that involve other WGs

SA3 for the Security aspects, SA5 for the Charging Aspects, RAN for the RAN related issues.

## 9	Supporting Individual Members

{At least 4 supporting Individual Members are needed. There is an expectation that these companies will provide resources to progress the work. Note that having 4 supporting companies is a necessary but not sufficient condition: the usual TSG approval process by consensus is needed for the WID approval.}

Supporting IM name

Huawei

HiSilicon

CBN