3GPP TSG-WG SA2 Meeting #153E e-meeting 	S2-220xxxx

Elbonia, October 10 – 14, 2022	(revision of S2-220xxxx)

Source:	Huawei, HiSilicon

Title:	[Title]

Document for:	Approval

Agenda Item:	[Agenda Item]

Work Item / Release:	[WID/SID Code] / Rel-18

Abstract: A summary of what this is about. Don’t start it with “This contribution…”

1. Introduction/Discussion

Evaluation and conclusion for KI#3.

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-xx.

# * * * * First change * * * *

7	Evaluation

Editor's note:	This clause will capture the evaluations related to the solutions per KI.

7.X	Evaluation for KI#3: Policies for finer granular sets of UEs

In KI#3 the following aspects are to be studied:

-	how to identify set of UEs at a finer granularity that are associated with a dedicated offload policy, and how to express the set of UE in the offload policy;

-	impacts to 5GS needed to support providing traffic offload policy for such a set of UEs.

Also some examplary scenarios are provided:

There are cases that traffic offload policy aims at a finer sets of UE(s), for instance, UEs satisfying a combination of criteria:

Case a:	UEs within a specific geographical area and have been associated with specific service provided by operator or application service provider;

Case b:	UEs that have been associated with a combination of services provided by operator or application service provider;

Case c:	UEs within a specific geographical area and have been associated with a combination of services provided by operator or application service provider;

Case d:	at specific time, UEs that have been associated with specific service or a combination of services provided by operator or application service provider;

Case e:	UEs belongs to both group-A and group-B, or UEs belongs to group-A and associated with specific service. It is possible that these users belong to pre-defined groups or do not belong to any pre-defined groups.

Group is introduced by 3GPP to manage a set of UEs, and group Id is used for identifying the set of UEs, the group can be defined by operator or by 3rd party. In case the group is defined by operator, the 3rd party AF is allowed to provide offload policy for a set of UEs by providing external group identifier; in case the group is defined by 3rd party, the 3rd party AF could manipulate the group member dynamically.

Also, TS 23.503 [13] Table 6.2-2: “PDU Session policy control subscription information” already defines following subscription data in UDR (information already defined for PCF usage).

| Information name | Description | Category |
| --- | --- | --- |
| Allowed services | List of subscriber's allowed service identifiers | Optional |
| Subscriber categories | List of category identifiers associated with the subscriber | Optional |

The Allowed services, Subscriber categories can be used for identifying a set of UEs, althouth the Allowed services and Subscriber categories themselves don't directly indicate UEs in a finer granularity, but combination of Allowed services / Subscriber categories and other criteria provides a flexible way to identify set of UEs at a finer granularity.

For some cases, one possible way for identifying set of UEs satisfying a combination of criteria is creating a group for the UEs by considerating criteria combination into account, and identifying the group with a group id, the group id based identification satisfies some cases. For example, a 3rd party AF gets UE Ids for UEs satisfying a combination of criteria and store the UE Ids as part of group subscription data in UDM/UDR, then the AF can identify the set of UEs by providing external group identifier.

But there are cases that are not suitable to reorganize UEs into a group. For example,

Example#1: assuming there are two operator defined groups, e.g. group-a and group-b, and a 3rd party AF wants to provide offload policy for UEs belong to both group-a and group-b, in this case some issues could exist if the AF tries to define a new group including UEs belong to both group-a and group-b: 1) 3rd party AF may not be able to be aware of UEs in group-a and group-b; 2) the member group-a and group-b could change and there is no such a mechanism to notify the change of group member to AF.

Example#2: for the the combination including Allowed services and Subscriber categories cases, there are same issues as example#1, i.e., 1）3rd party AF may not be able to be aware of UEs subscribing to Allowed services/ Subscriber categories; 2) the UE’s subscription of Allowed services/ Subscriber categories could change and there is no such a mechanism to notify the change

# * * * * Second change * * * *

8	Conclusions

Editor's note:	This clause will capture the conclusions of the study.

8.X	Conclusions for KI#3: Policies for finer granular sets of UEs

Normative specifications will add following features

- the data stored by NEF in UDR (as part of step 3a of Figure 4.3.6.2-1 of TS 23.502 [9]) and the information sent by UDR to the PCF (as part of step 4 of Figure 4.3.6.2-1of TS 23.502 [9]) may refer to a logical combination of Group Ids and/or "Allowed services" and/or "Subscriber categories" as defined in TS 23.503 [13] table 6.2-2. and/or application decided service information

-	The logical combination may e.g. refer to users members of Group Id = X and of "Subscriber categories" = “Gold”

- Nnef_TrafficInfluence service API may refer to a logical combination of External group Identifiers, "Allowed services" and/or "Subscriber categories" and/or application decided service information. The "Allowed services" and "Subscriber categories" are as defined in TS 23.503 [13] table 6.2-2 (and as defined above for the data stored by NEF in UDR as part of step 3a of Figure 4.3.6.2-1 of TS 23.502 [9]).

- The NEF may, based on local policies determine a logical combination of External group Identifiers, "Allowed services" and/or "Subscriber categories" and/or application decided service information that is to be associated with an incoming Nnef_TrafficInfluence service request . These local policies may relate e.g. with the identity of the AF having issued the Nnef_TrafficInfluence service request and/or relate with the parameters indicated in the Nnef_TrafficInfluence service request (for example the local policies may take into account following information provided in the AF request: the MTC Provider Information, the S-NSSAI and/or the Target category defined above).

- The SMF, EASDF and UPF are not impacted by normative work on this KI. Only the NEF needs to be aware of application decided service information.

# * * * * Third change * * * *

# * * * * Fourth change * * * *

# * * * * Fifth change * * * *

# * * * * End of changes * * * *