3GPP TSG-WG SA2 Meeting #153E e-meeting 	S2-220xxxx

Elbonia, 10 – 17 October 2022

Title:	[Draft] LS on exchange of EDI among PLMNs

Response to:

Release:	Release 18

Work Item:	FS_EDGE_Ph2

Source:	SA2

To:	GSMA OPG

Cc:

Contact Person:

Name:	Dario S. Tonesi

Tel. Number:

E-mail Address:	dtonesi AT qti DOT qualcomm DOT com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	none

1. Overall Description:

According to GSMA OPG.02 v2.0:

2.3 High-Level Security Requirements

The OP architecture shall comply with the following security requirements:

- The OP shall expose operator network functions and resources data (e.g., compute and storage) following the 'need-to-know' principle and only for the legitimate scenarios expected in the PRD.

- The OP shall expose operator network functions and resources data (e.g., compute and storage) following the 'need-to-know' principle and only for the legitimate scenarios expected in the PRD.

- The OP shall not expose its configuration data and internal topology (referred to as topology hiding).

During the FS_EDGE_Ph2 study item, some solutions based on the exchange of EAS Deployment Information (EDI) between PLMNs have been developed to support the deployment of Edge Node Sharing. Currently, the EDI is defined in Table 6.2.3.4-1 of TS 23.548 as:

| Parameters | Description |
| --- | --- |
| DNN | DNN for the EAS Deployment Information. [optional] |
| S-NSSAI | S-NSSAI for the EAS Deployment Information. [optional] |
| External Group Identifier/Internal Group Identifier | Group ID for the EAS Deployment information. [optional] NOTE: The AF may provide External Group Identifier, and NEF can map the External Group Identifier into Internal Group Identifier according to information received from UDM. |
| Application ID | Identifies the application for which the EAS Deployment Information corresponds to. [optional] |
| FQDN(s) | Supported FQDN(s) for application(s) deployed in the Local part of the DN. |
| DNAI(s) | DNAI(s) for the EAS Deployment information. [optional] |
| DNS Server Information | list of DNS server identifier (consisting of IP address and port) for each DNAI. [optional] |
| EAS IP address range Information | IP address(es) of the EASs in the Local part of the DN or the IP address ranges (IPv4 subnetwork(s) and/or IPv6 prefix(es) of the Local part of the DN where the EAS is deployed for each DNAI. [optional] |

2. Actions:

To GSMA OPG:

ACTION: 	SA2 kindly ask GSMA OPG

- if they see any issues with the exchange of EDI among PLMNs; and;

- what information of the EDI, if any, can be exchanged among PLMNs without breaking the topology hiding requirement

3. Date of Next TSG SA WG2 Meetings:

TSG-SA2 Meeting #154		14-20 November 2022	Toulouse, France (hopefully)

TSG-SA2 Meeting Ad Hoc TBD	16-20 January 2022		E-meeting