3GPP TSG-SA WG2#153E e-meeting	S2-220abcd

Elbonia, 10-14 October 2022	(was S2-220xxxx)

Source:	Qualcomm Incorporated

Title:	TR 23.700-58 Conclusions Key Issue #2

Document for:	Discussion/Approval

Agenda Item:	9.5

Work Item / Release:	FS_UAS_Ph2

Abstract of the contribution: the contribution proposes a set of interim conclusions for FS_UAS_Ph2.

# 1.	Text proposal

It is proposed to agree the following changes vs. TS 23.700-58.

- >>>>BEGINNING OF CHANGES<<<<

# 8	Conclusions

Editor's note:	This clause will list conclusions that have been agreed during the course of the study item activities.

## 8.Y	Conclusions for Key Issue #2 - Support of Broadcast Remote ID

It is proposed to progress to normative phase the following:

-	a mechanism leveraging U2X (UAV-to-everything), with the architectural enhancements identified in solution 5, applicable to both UAV UE that registers to the MNO network, and to UAVs that operate out of coverage

-	for scenarios restricted to UAV UEs in coverage and that register to the network, and to UAV-C that are 3GPP UEs, a mechanism based on solution #3 based on MBS.

Editor’s note: the exact limitations for the applicability of an MBS-based solution for key issue #2 are FFS.

It is assumed that security of the U2X solution (beyond PC5-related security already defined in 3GPP for C-V2X) is specified at the application layer outside of 3GPP.

It is assumed that a combination of U2X and network-based solutions can be deployed.

- >>>>END OF CHANGES<<<<