3GPP TSG-SA WG2#153E e-meeting	S2-220abcd

Elbonia, 10-14 October 2022	(was S2-220xxxx)

Source:	Qualcomm Incorporated

Title:	TR 23.700-58 Conclusions Key Issue #3

Document for:	Discussion/Approval

Agenda Item:	9.5

Work Item / Release:	FS_UAS_Ph2

Abstract of the contribution: the contribution proposes a set of interim conclusions for FS_UAS_Ph2.

# 1.	Text proposal

It is proposed to agree the following changes vs. TS 23.700-58.

- >>>>BEGINNING OF CHANGES<<<<

# 8	Conclusions

Editor's note:	This clause will list conclusions that have been agreed during the course of the study item activities.

## 8.Z	Conclusions for Key Issue #3 - Support of Detect and Avoid Mechanism in 3GPP system

It is proposed to progress to normative phase the following:

-	a mechanism leveraging U2X (UAV-to-everything), with the architectural enhancements identified in solution 5, with the following principles:

-	The detection and resolution of collisions is locally performed between UAVs using direct UAV to UAV communication over PC5.

-	The USS can (optionally) be informed of the collision situation.

NOTE:	The UAV may inform the USS of the collision situation over application layer communication. No normative work is expected to enable this

-	Both unicast and broadcast mode direct communication over PC5 is supported for DAA

-	a network-based mechanism based on solution 2 for scenarios where a single USS serves all the potential UAVs, and all the UAVs involved are served by the same PLMN.

It is assumed that a combination of U2X and network-based solutions can be deployed.

It is assumed that security of the U2X solution (beyond PC5-related security already defined in 3GPP for C-V2X) is specified at the application layer outside of 3GPP.

- >>>>END OF CHANGES<<<<