SA WG2 Meeting #153e	S2-220xxxx

October 10 – October 14, 2022, Electronic Meeting

Source:	InterDigital Inc.

Title:	KI #1, New Conclusion: Overall Conclusion for Key Issue

Document for:	Approval

Agenda Item:	9.19

Work Item / Release:	FS_XRM/ Rel-18

Abstract of the contribution: This paper provides a conclusion for Key Issue #1.

- 1.	 Discussion

Solution #1, Solution #36, Solution #38, Solution #39, Solution #40, Solution #62, Solution #63, Solution #65 and Solution #66 are for Key Issue #1.

This paper proposes text for the Key Issue #1 conclusion section.

- 2. Proposals

It is proposed to adopt the following text in TR.23.700-60 v1.1.0.

*** Start of 1st  changes (all new text)***

## 8.X Key Issue #1 Conclusion

It is concluded that, in the normative phase:

- An NEF API will allow the AF to indicate what flows need to be coordinated. In order to allow the AF to configure the flow information before, or after, PDU Sessions are established, and in order to avoid NAT issues, the NEF API will allow the AF to identify the UE with a GPSI.

- An NEF API will provide an additional option where the AF can indicate the allowed delay differences between flows to the PCF. This information and the existing Per QoS Flow per UE QoS Monitoring feature can be used by the PCF to monitor delay differences between flows and take action (i.e. change PCC Rules) if any delay differences are measured to be out of allowed range.

- In order to ensure that the UE selects the correct DNN/S-NSSAI combination for the XRM traffic, a traffic descriptor for the XRM session will be used during URSP evaluation. In order to allow the DNN/S-NSSAI combination to be used by other UEs for non-XRM traffic and other XRM sessions, the network may provide a Coordination Identifier to the AF and the AF may provide the Coordination Identifier to the UE. The Coordination Identifier can then be used as the Traffic Descriptor.

NOTE: 	How the AF provides the Coordination Identifier to UE hosted applications that want to participate in the service is out of SA2's work scope.

- When the PCF is aware that certain flows need to be associated (i.e. based on information provided by the AF via invocation of an NEF API), the PCF may provide a "Handled Together” indication to the SMF so that the SMF can either reject or allow all flows that need to be "Handled Together”; or provide an indication of  necessary or critical flows, so that if any necessary flow is rejected, the remaining flows will be rejected by SMF, thus providing the benefit that the PCF can derive a new set of PCC Rules that can be fulfilled.

*** End of changes ***