SA WG2 Meeting #153E	S2-2208892r07

Elbonia, October 10 – 17, 2022	(revision of S2-220xxxx)

Source:	Intel, OPPO, Qualcomm

Title:	KI#3: Update on Conclusion

Document for:	Approval

Agenda Item:	9.22

Work Item / Release:	FS_eUEPO / Rel-18

Abstract: This contribution proposes evaluation and conclusion for KI#3.

# 1.	Discussion

This paper aims to resolve two ENs in the conclusion for KI#3:

#1：Editor's note: It is FFS how to handle the EPS mobility for a UE that is dual registered (i.e. with a registration in 5GS via N3IWF and attached to EPC/E-UTRAN) and thus whether UE policy association can be maintained in both EPS and 5GS or only in 5GS or EPS.

#2：Editor's note: It is FFS whether an explicit URSP Provisioning Support Indication in EPS in UE Policy Container needs to be included when UE does Initial Attach in EPS.

# 2.	Proposal

It is proposed to capture the following changes into TS 23.700-85.

- * * * * First change * * * *

# 8	Conclusions

Editor's note:	This clause will list conclusions that have been agreed during the course of the study item activities.

## 8.3	Conclusion on KI#3:

-	The ePCO based URSP delivery to UE in EPS is selected as baseline mechanism for normative work.

-	In order to avoid unnecessary signalling overload for the purpose of delivery URSP to pre-Rel-18 UEs in EPS, the UE needs to indicate the URSP Provisioning Support Indication in EPS in the UE Policy Container to UE-PCF in EPS and 5GS.

-	During 5GS to EPS mobility with N26, the AMF doesn't initiate the UE Policy Association termination procedure, instead the AMF will request the UE-PCF to delay the termination of UE Policy Association for a configured period. The UE-PCF can initiate the UE Policy Association Termination procedure either after it receives the UE Policy Association Establishment from SM-PCF or after the configured period. The mobility procedure from 5GS to EPC described in clause 6.16.2.3.3 is selected as the baseline procedure for 5GS to EPS mobility with N26.

-	AMF doesn't provide any new information to MME during 5GS to EPS mobility with N26, the SM-PCF determines whether the UE supports URSP delivery in EPS by checking UE context policy control subscription information in UDR. The SM-PCF discovers the address of UE-PCF serving the UE by querying BSF. The UE-PCF recovers the information about the PSI list in the UE and the subscribed PCRTs in 5GS from former UE Policy Association for the UE after receiving the UE Policy Association Establishment request including a UE Policy Container only including an indication about the trigger for the UE Policy Association Establishment ("5GS to EPS handover").

-	When the UE is attached to EPS, the SM-PCF can retrieve the PCRTs for UE Policy from UE-PCF and subscribe to the applicable PCRTs in EPC to SMF+PGW-C.

-	When the UE returns from EPS to 5GS, the old UE policy association in EPS should be terminated after the new UE policy association established in 5GS.

-	When the UE does Initial Attach to EPC, in the default PDN connection is requested to established, the UE includes the UE Policy Container ePCO in PDN Connectivity Request encapsulated in Attach Request; otherwise, the UE will include the UE Policy Container ePCO in the PDN Connectivity Request during the first request for PDN connectivity as described (i.e. as described in Sol#16). The PDN Connection used by UE and SMF/PGW-C to convey UE Policy Container ePCO shall be kept when the UE is in the CONNECTED mode.

-	The PCF triggers the re-evaluation of applicable URSPs for the UE and determines an update of URSP is needed for the UE when an event as described in clause 6.16.11 happens in PCF. The PCF selects one of the PDN connection/PDU sessions associated to EPC for the delivery of the URSP update. Then the PCF generates the corresponding UPDP message MANAGE UE POLICY COMMAND in a similar way than it is done in 5GC and then includes the message into a new IE for sending it to the SMF over N7, then further to UE as described in clause 6.16.2.2.

-	If the feature "ePCO based URSP delivery to UE in EPS" is supported in a network, during UE registration, the AMF queries BSF to discover the PCF and PCF set that may be already serving a UE policy association for the UE in EPS.

NOTE: BSF needs to be deployed in order to ensure a single UE-PCF is selected for a UE in both EPS and 5GS.

Editor's note:	It is FFS whether URSP mapping in EPS requires any dynamic configuration mechanism (e.g. as proposed in Sol #17) to address the aspects identified in the 2nd bullet of the KI#3 description.

Editor's note:	It is FFS whether an explicit URSP Provisioning Support Indication in EPS in UE Policy Container needs to be included when UE does Initial Attach in EPS.

- * * * * End of changes * * * *