3GPP TSG-WG SA2 Meeting #153e	S2-2208199r02

E-meeting, October 10-17, 2022	(revision of S2-220xxxx)

Source:	InterDigital Inc., Intel

Title:	KI #1, Conclusion Update on Re-evaluation Triggers

Document for:	Approval

Agenda Item:	9.22

Work Item / Release:	FS_eUEPO / Rel-18

Abstract: This contribution updates the Key Issue #1 Conclusion on re-evaluation triggers.

1. Discussion

Editor's note:	It is FFS whether a new re-evaluation trigger (e.g., change of PLMN/Route Selection Validation Criteria or an indication in the URSP Rule) is needed when VPLMN specific URSP Rules or VPLMN specific RSDs are used.

This contribution addresses the following editor’s note from the Key Issue #1 conclusion.

The UE may re-evaluate the URSP rules, to check if the change of the association of an application to a PDU session is needed, when:

NOTE 11:	The time when the UE performs the re-evaluation is up to UE implementation. It is recommended that the UE performs the re-evaluation in a timely manner.

a)	the UE performs periodic URSP rules re-evaluation based on UE implementation;

b)	the UE NAS layer indicates that an existing PDU session used for routing traffic of an application based on a URSP rule is released;

c)	the URSP is updated by the PCF;

d)	the UE NAS layer indicates that the UE performs inter-system change from S1 mode to N1 mode;

e)	the UE NAS layer indicates that the UE is successfully registered in N1 mode over 3GPP access or non-3GPP access;

f)	the UE establishes or releases a connection to a WLAN access and transmission of a PDU of the application via non-3GPP access outside of a PDU session becomes available/unavailable;

g)	the allowed NSSAI or the configured NSSAI is changed; or

h)	the LADN information is changed.

If the re-evaluation leads to a change of the association of an application to a PDU session, the UE may enforce such change immediately or when UE returns to 5GMM-IDLE mode.

NOTE 12:	The time when the UE enforces the change of the association of an application to a PDU Session is up to UE implementation. It is recommended that the UE performs the enforcement in a timely manner.

The URSP handling layer may request the UE NAS layer to release an existing PDU session after the re-evaluation.

Section 4.2.2.2, TS 24.526 lists the following re-evaluation conditions:

The re-evaluation triggers listed above that are highlighted in yellow are good examples of cases where a UE may re-evaluate URSP rules because services are becoming available or are becoming unavailable. For example, a re-evaluation may be triggered by the UE receiving new LADN Information, a new slice being allowed, or a slice being removed from the Allowed NSSAI.

The Key Issue #1 conclusion introduces VPLMN specific URSP Rules which can be used to cause the UE to route traffic to an LBO PDU Session when the UE is in the VPLMN. When the UE leaves the VPLMN, the ability to use the LBO PDU Session is lost. Similarly, when the UE enters the VPLMN, access to the VPLMN’s Data Network(s) becomes available. It is important to consider that some services (e.g. edge computing) require timely re-location of application layer sessions from one data network to another (i.e. closer) data network. Timely re-location of an application layer session from one data network to a newly available (i.e. closer) data network is not possible until the UE detects the availability of the new data network. Thus, it is proposed add the following sentence to the key issue #1 conclusion to ensure that the UE accounts for the availability (or un-availability) of VPLMN services in a timely manner:

When there is a PLMN change, and the UE has PLMN specific URSP Rule(s) for the PLMN that it is leaving or entering, the UE (re-)evaluates URSP rule validities in a timely manner.

This p-CR also deletes the related editor’s note and make some editorial fixes.

The following changes are proposed for TR 23.700-85 v1.0.0.

# * * * * 1st change * * * *

8.1.3	Conclusions on which PLMN determines the VPLMN specific URSP Rules

The scenario where the network provides the UE with URSP rules applicable in the VPLMN It is proposed to adopt the following interim conclusion principles:

-	The H-PCF provides VPLMN specific URSP Rules to the UE.

-	The H-PCF generates VPLMN specific URSP rules by taking Service Parameters either from V-PCF or the V-AF in.

Editor's note:	It is FFS which solution is taken for normative specification, H-PCF generating VPLMN specific URSP rules based on service parameters provided by V-PCF or based on URSP guidance provided by the AF from VPLMN for the case when the H-PLMN generates the VPLMN specific URSP rules.

8.1.4	Conclusions on Re-evaluation Triggers

Upon PLMN change, the UE (re-)evaluates URSP rules.

# * * * * End of changes * * * *