3GPP TSG-SA WG2 Meeting #153E (e-meeting)	S2-2208257

# Elbonia, 10 – 17 October, 2022												 (revision of S2-xxxxxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 3555 | rev | - | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support of RRC_INACTIVE with long eDRX |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_REDCAP_Ph2 |  |  |  |  |  | Date: |  |  | 2022-09-30 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Supports the CN based MT data and signalling handling for UE in RRC_INACTIVE with long eDRX, based on the conclusions for FS_REDCAP_Ph2 in TR 23.700-68. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduction of flow for NG-RAN to request CN based MT data and signalling handling in RRC_INACTIVE state. Introduction of flow for CN trigger NG-RAN paging. Update the UE triggered Connection Resume procedure for reporting RRC_CONNECTED state to CN |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing functionality agreed based on conclusion from the study. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.8.1.1a(new), 4.8.2.2, 4.8.2.2b(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

****************** START CHANGE ***************

#### 4.8.1.1	Connection Inactive procedure

This procedure may be initiated by the serving NG-RAN node when the UE is in CM-CONNECTED with RRC Connected state and has received the "RRC Inactive Assistance Information" from the AMF as defined in clause 5.3.3.2.5 of TS 23.501 [2]. NG-RAN initiates the transition to RRC Inactive state as defined in TS 38.300 [9].

#### 4.8.1.1a	Connection Inactive procedure with CN based MT communication handling

This procedure may be initiated by the serving NG-RAN node when CN based mobile terminating (MT) communication handling is requested for a UE that is configured with extended DRX (i.e. >10.24 seconds) for RRC_INACTIVE state and has at least one PDU session with active user plane as defined in clause 5.31.7 of TS 23.501 [2].

Figure 4.8.1.1a-1: NG-RAN initiated Connection Inactive procedure with CN based MT communication handling

0.	The UE is registered in the network with negotiated eDRX parameters for CM-IDLE state and is in CM-CONNECTED with RRC_CONNECTED state. The AMF provides the eDRX parameter to NG-RAN as part of the RRC Inactive Assistance Information as defined in clause 5.3.3.2.5 of TS 23.501 [2]. The AMF also includes support indication of CN based MT communication handling for UE in CM-CONNECTED with RRC_INACTIVE state.

1.	NG-RAN determines eDRX parameters for UE in RRC_INACTIVE state and decides to transit the UE to RRC_INACTIVE state with CN based MT communication handling.

2.	The NG-RAN sends N2 message to AMF indicating the UE is transitioning to RRC_INACTIVE state and the CN handles MT communication. The NG-RAN also provides the eDRX value for RRC_INACTIVE to AMF.

Editor’s note:	Whether the N2 message to be used is a new message or an existing message, e.g. N2 notification, needs coordination with RAN WG considering also if it’s NGAP class 1 or class 2 procedure.

3.	For each of the PDU sessions with user plane resources have been activated, the AMF invokes Nsmf_PDUSession_UpdateSMContext Request (PDU Session ID, Cause, Operation type, User Location Information, Age of Location Information, N2 SM Information (Secondary RAT usage data)) towards SMF. The Operation Type is set to a value that indicates to stop user plane DL data transmissions towards the UE and enable data buffering. The SMF starts data buffering for MT data if the data buffering is handled in SMF.

Editor’s note:	Whether an existing Operation Type is used, e.g. "UP Suspend", or another value is used needs coordination with CT WG.

4.	If data buffering is handled in the UPF, the SMF updates the UPF with proper rules for MT data handling.

5.	The SMF sends the Nsmf_PDUSession_UpdateSMContext response.

6.	The AMF sends N2 response to NG-RAN acknowleding the indication and the AMF considers the UE is in CM-CONNECTED with RRC_INACTIVE state.

7.	NG-RAN initiates state transition from RRC_CONNECTED to RRC_INACTIVE with RRC configuring eDRX as specified in TS 38.300 [9].

****************** NEXT CHANGE ***************

#### 4.8.2.2	UE Triggered Connection Resume in RRC Inactive procedure

The Connection Resume procedure is used by the UE to perform RRC Inactive to RRC Connected state transition. Triggers for the UE to initiate this procedure are defined in clause 5.3.3.2.5 of TS 23.501 [2].

Figure 4.8.2.2-1: Connection Resume in RRC Inactive

1.	UE to NG-RAN: RRC message (Resume ID).

The UE initiates the transition from RRC Inactive state to RRC Connected state, see TS 38.300 [9]. The UE provides its Resume ID needed by the NG-RAN to access the UE's stored Context.

2.	[Conditional] NG-RAN performs UE Context Retrieval.

UE Context Retrieval is performed when the UE Context associated with the UE attempting to resume its connection is not locally available at the accessed NG-RAN. The UE Context Retrieval procedure via NG-RAN is specified in TS 38.300 [9].

3a.	[Conditional] N2 Path switch procedure.

If the accessed NG-RAN is able to retrieve the UE Context, the accessed NG-RAN node initiates N2 Path Switch procedure, i.e. steps 1 to 8 of clause 4.9.1.2.2 and including Xn data forwarding.

If the Connection Resume procedure is a response to RAN paging which is triggered by 5GC due to an N2 interface procedure, NG-RAN and 5GC handle the N2 interface procedure as a collision described in clause 4.9.1.2.

3b.	[Conditional] N2 Notification

3b.1	If the accessed NG-RAN is the same as the NG-RAN that configured RRC Inactive and still has the UE context, NG-RAN send an N2 Notification to the AMF indicating the UE is in RRC_CONNECTED, if an AMF requested N2 Notification (see clause 4.8.3) or Connection Inactive procedure with CN based MT communication handling (see clause 4.8.1.1a) has been performed previously.

3b.2	The AMF invokes Nsmf_PDUSession_UpdateSMContext Request towards SMF indicating the Downlink data delivery for each PDU session with active user plane, if the AMF has requested data buffering as described in clause 4.8.1.1a.

3b.3	N4 session modification procedure is triggered by the SMF. If data buffering is handled in the UPF, the SMF updates the UPF with appropriate rules to trigger data delivery.

3b.4	The SMF sends the Nsmf_PDUSession_UpdateSMContext response.

3b.5	The AMF sends the N2 response to NG-RAN.

Editor’s note:	Whether the N2 message to be used is a new message or an existing message, e.g. N2 notification, needs coordination with RAN WG considering also if it’s NGAP class 1 or class 2 procedure.

4.	NG-RAN to UE: RRC message.

The NG-RAN confirms to the UE that the UE has entered RRC_CONNECTED state.

NOTE:	Steps 3 and 4 can be executed in parallel.

****************** NEXT CHANGE ***************

#### 4.8.2.2a	Network Triggered Connection Resume in RRC Inactive procedure

The Network Triggered Connection Resume in RRC Inactive procedure is used when the NG-RAN needs to send data (e.g. the N1 NAS PDU and/or downlink user plane PDU) to a UE in the RRC Inactive state. During the procedure, the NG-RAN sends a RAN Paging to the UE in order to trigger the Connection Resume in RRC Inactive procedure in clause 4.8.2.2.

Figure 4.8.2.2a-1: Network Triggered Connection Resume for UE in RRC Inactive

1.	The NG-RAN receives downlink data (e.g. the N1 NAS PDU and/or the user plane PDU) for a UE in RRC Inactive State, the NG-RAN buffers the downlink data and triggers RAN Paging message.

If the NG RAN supports the Paging Cause Indication for Voice Service feature and if the UE context in NG-RAN indicates that the UE supports the Paging Cause Indication for Voice Service feature, the NG RAN shall provide the Voice Service Indication in the RAN Paging message for the UE when it detects that the downlink data which triggers the RAN Paging message is related to voice service, as specified in clause 5.38.3 in TS 23.501 [2].

2.	The NG-RAN sends the RAN Paging message to the UE.

3.	If the UE is in RRC Inactive State, based on the RAN paging and the UE decides to accept the paging, the UE initiates the UE Triggered Connection Resume in RRC Inactive procedure (see clause 4.8.2.2).

#### 4.8.2.2b	Network Triggered Connection Resume in RRC Inactive with CN based MT communication handling

When the UE is in CM-CONNECTED with RRC_INACTIVE state with CN based mobile terminating (MT) communication handling, high latency communication as described in clause 5.31.8 in TS 23.501 [2] is applied.

This procedure may be triggered by MT data, or a N1 procedure from SMF and UPF as shown in Figure 4.8.2.2b-1. When the procedure is triggered by other NFs (e.g., SMSF, LMF, GMLC), the UPF (or SMF) in the following figure should be replaced by the respective NF (the corresponding service operations used by other NFs when they communicate with AMF may also be different from the service operations used by SMF/UPF).

During the procedure, the NG-RAN (i.e. gNB) performs RAN paging towards the UE based on the N2 message from the AMF in order to trigger the UE triggered Connection Resume procedure in clause 4.8.2.2.

Figure 4.8.2.2b-1: Network Triggered Connection Resume for UE in RRC Inactive with CN based MT communication handling

1a.	When downlink data is received and the SMF/UPF is requested to perform buffering as specified in clause 4.8.1.1a, the UPF/SMF checks with AMF for the possibility of data delivery, similar to step 2 of clause 4.24.2 with the following differences:

-	The AMF determines if the UE is reachable based on the stored eDRX information for RRC_INACTIVE state provided from NG-RAN in clause 4.8.1.1a. If the UE is unreachable, the AMF provides the Estimated Maximum Wait time in the response message based on the extended DRX information for RRC INACTIVE in AMF (steps 2-5 are skipped). If the UE is considered reachable, step 2 is triggered.

NOTE: This handling is similar to CM-IDLE with eDRX. When the AMF provides the Estimated Maximum Wait time, it can consider the time needed for RRC level procedures (e.g. RRC RNA update procedure) when UE wakes up from the eDRX cycle.

2.	The AMF sends an N2 message to NG-RAN with the request for the UE to be transitioned to RRC_CONNECTED.

Editor’s note:	Whether the N2 message to be used is a new message or an existing message, e.g. N2 notification, needs coordination with RAN WG.

3.	NG-RAN performs RAN paging towards the UE.

4.	When the UE receives RAN paging, it initiates the UE triggered Connection Resume procedure and NG-RAN notifies CN as specified in clause 4.8.2.2 including the N2 Notification in step 3b.

5.	The UPF triggers downlink data delivery if there is any. The AMF sends downlink NAS messages if there is any.

****************** END CHANGES ***************