SA WG2 Meeting #S2-153e(e-meeting)	S2-2208259

10 - 17 October 2022, Electronic Meeting	(revision of S2-22xxxxx)

Source:	Ericsson, Huawei? HiSilicon?

Title:	KI#7, Sol#20, Update to remove ENs

Document for:	Approval

Agenda Item:	9.9

Work Item / Release:	FS_VMR/Rel-18

Abstract of the contribution: Introduction updates for Sol#20.

# 1. Introduction

This paper introduces updates to solution #20.

# 2. Discussion

There are two ENs in solution#20 currently.

Editor's note:	Whether this is a CAG or whether this will have to be an independent identifier supported from rel-18 or both the CAG and a new identifier could be considered is FFS.

Editor's note:	It is FFS whether there are further enhancements needed for existing CAG procedures, e.g. extended range (max #) of CAG identifiers, specific CAG prioritization (for MBSR), roaming support of the CAG use, etc.

The first EN is related to the consideration that if MBSR access control can be totally separated from CAG. The reason to use CAG functionality as a base is due the consideration that it’s legacy functionality and can be applied to both pre-rel-18 UEs and rel-18 UEs. It also provides the possibility to prevent UE initiate unnecessary signalling at the first place of accessing MBSR (i.e. IAB-node) if it’s not allowed. Using CAG based logic is proper way to move forward.

The second EN is related to the consideration that if there is any enhancement needed on top of current CAG functionality for MBSR with roaming in different PLMNs. The number of CAG IDs that can be delivered to UE is already rather high (61/62 CAG IDs) and there seems no need of further improvement. For all the cells broadcasted by the MBSRs in a PLMN, limited number of CAG IDs can be introduced based on intended service group (e.g. all MBSRs used for public transport can use one CAG ID, all MBSRs used for vehicles for public safety usage can have a separate CAG ID). For the CAG prioritization, there is probably no such needed when it comes CAG cell and non-CAG cell selection at this stage, when RAN WG is also discussing the cell selection on the cell broadcasted by MBSR and the cell broadcasted by gNB. For the roaming support, the solution already indicated subscription data handling can be simplified with common flag. The handling at UE side can be the same as legacy.

# 3. Proposal

This paper proposes the following to solution#20 in clause 6.20

* Start of change *

## 6.20	Solution #20: CAG based UE access control via MBSR

### 6.20.1	Introduction

This solution relates to KI#7 "Control of UE's access to 5GS via a mobile base station relay". The solution proposes to use the existing CAG, Closed Access Group, concept as a base to manage the UE access control via MBSR.

There are certain simplifications on the existing CAG logic to fit into the IAB architecture.

### 6.20.2	Functional Description

When a UE tries to select or connect to 5GS via a MBSR (a mobile IAB-node), the access control of the UE is managed by CN, RAN and UE. During the CM-CONNECTED mobility, the access control via MBSR (mobile IAB-node) is managed by CN and RAN.

6.20.2.1  CAG identifier of MBSR

A CAG identifier shall be assigned for the MBSR node to use and it is unique within the scope of a PLMN ID. The CAG identifier is announced by the cells covered by the MBSR. The CAG identifier of MBSR cell may be different from the CAG identifier of normal cell. If the MBSR is pre-configured with the PLMN list in which the MBSR is allowed to operate as an IAB node, the corresponding CAG Identifier per PLMN is also configured in the MBSR.

6.20.2.2  UE CAG configuration

To access the CAG cell of MBSR, the UE, that supports CAG as indicated as part of the UE 5GMM Core Network Capability, is configured with the CAG information as described in the clause 5.30.3.3 of TS 23.501[2].

Considering the roaming scenario, the CAG information per PLMN ID shall be configured to UE. This CAG Identifier for MBSR in subscription data can be defined as a "common" CAG identifier or a flag which can be indicated so that different PLMNs can use it to determine whether the UE is allowed (or not allowed) to access the cell associated with the MBSR CAG Identifier.

6.20.2.2  Access control

RAN and CN supports the UE access control based on the CAG identifier associated with the MBSR cell and the allowed CAG identifiers for the UE.

For UEs that support CAG, they may select the cells covered by a MBSR based on the configured MBSR CAG identifier using the existing CAG control logic. The legacy UE(s) that support CAG shall also support to access the MBSR cell based on the CAG identifier.

For the UEs that do not support CAG (i.e. the UE is not able to understand the broadcasted CAG information by the cell) but supports the use of IAB architecture, it may select the cell as normal and rely on RAN and CN to control the IAB-node access using network side CAG control logic.

See more details of the CAG concept in clause 5.30.3 of TS 23.501 [2].

Considering the MBSR access use case, additional control (e.g., time duration, specific geographic location) may be linked to the MBSR CAG identifier management, e.g., when user purchase a buss/train ticket, the access to IAB-node/MBSR CAG identifier may be limited to certain time period and specific locations and granted to the user subscription data via O&M or AF. It may also be provisioned to the UE accordingly so that the UE can properly select the cell. Additionally, the range (number) of CAG identifiers may need to be increased, access to a CAG cell may be (de)prioritized, and roaming case of CAG use may need further control.

The serving PLMN may introduce number of CAG Identifiers needed for the cells covered by MBSR based on the targeting group of users (e.g. one CAG Identifier for MBSRs on public transport vehicles used for passengers, another CAG identifier for MBSRs on vehicles used by public safety).

### 6.20.3	Procedures

#### 6.20.3.1	General

The CAG control is well described in different procedures in TS 23.502 [5]. Procedure details may be updated to support the additional control in case of MBSR CAG Identifier.

The CAG specific procedures, involving RAN/AS, are defined in TS 38.300 [20] clause 16.7. These procedures allow the control of:

-	whether new UEs (Rel-18 onward), or also legacy UEs, can access to a mobile-IAB CAG cell;

-	whether non-CAG capable UEs can access, or not, to a mobile-IAB CAG cell (as in TS 38.304 [21]).

In order to control the use of the CAG for MBSR use cases, the MBSR CAG configuration (in the UE and the IAB node) may be associated with the following:

-	validity criteria of the MBSR CAG ID, associated with time duration, geographic area;

A mobile-IAB CAG cell may also be configured with a human-readable network name, to be signalled to the UE for presentation to the user.

### 6.20.4	Impacts on services, entities and interfaces

AMF:

-	Supports baseline CAG control with possible additional controls (e.g. time duration, geographic location) based on MBSR CAG identifier or a flag.

UDM:

-	Supports storing CAG information of MBSR as UE’s Subscription data or common flag with additional controls (e.g. time duration, geographic location).

gNB/IAB-node:

-	Supports CAG function.

UE:

-	Supports CAG function.

NOTE:	Some of the potential CAG extensions and additional control mechanisms may have RAN dependencies.

* End of changes *