SA WG2 Meeting #153e	S2-2208378r02

# October 10th – 14th, 2022; Elbonia               	  			  					(revision of S2-220xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 3561 | rev | - | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Emergency PDU session transfer |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell, MediaTek, Google Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_Ph1, TEI17 |  |  |  |  |  | Date: |  |  | 2022-09-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | LS C1-225230 / S2-2208123 asks for clarity on the SMF processing requirements when the UE is attempting to set up multiple emergency PDU sessions. The UE shall not request multiple emergency PDU sessions over any Access Type. TS 23.501 CR 3659 specifies that the UE shall have only one emergency PDU session at any time. The network is required to enforce this limitation. The UE is expected to request handover when it is transferring an existing emergency PDU session from one Access Type to the other. TS 23.501 requirement for the network to enforce the above requirement by rejecting the second emergency PDU session establishment request from the same UE differs from the PDU Session collision control in stage 3 specifications. TS 29.502 specifies that PDU Session establishment from the same UE on the same PDU session ID shall override the old PDU session. The SMF shall proceed with the new request and abandon the old PDU session locally. TS 24.501 also specifies that the SMF may override the existing PDU session. Over-writing the existing emergency PDU session is a fail-safe approach but redundant SM state must be cleared from the old SMF if the release of the PDU session towards the old SMF fails and subsequently a new emergency PDU session establishment request is processed by another SMF. |  |  |  |  |  |  |  |  |
| Summary of change: |  | UDM needs to de-register the old SMF if emergency PDU session is established in another SMF. Re-use of the duplicate PDU session collision handling from TS 29.502 clause 5.2.2.2.1 covers the collision case when both the old and the new PDU Session are handled by the same SMF. This CR adds the deregistration of the old SMF causing the removal of the redundant PDU Session if the new PDU session is established in a different SMF. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | CT1 cannot specify correct error handling to police the maximum number of emergency PDU sessions. SMF might reject a valid emergency PDU session establishment request. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.3.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

##### 5.2.3.2.2	Nudm_UECM_DeregistrationNotification service operation

Service operation name: Nudm_UECM_DeregistrationNotification.

Description: UDM notifies the NF consumer which has previously registered (using Nudm_UECM_Registration operation) has been deregistered in the UDM. As a result, the consumer is no longer registered in UDM as a serving NF for that UE.

NOTE:	This notification corresponds to an implicit subscription.

Inputs, Required: SUPI, Access Type, PDU Session ID, serving NF deregistration reason.

Inputs, Optional: NF ID in the case of SM Context Transfer.

Outputs, Required: None.

Outputs, Optional: None.

See step 14d of clauses 4.2.2.2.2 and 4.26.5.3 for an example usage of this service operation. The serving NF deregistration reason tells the reason for sending the deregistration notification to the consumer NF.

The reason for AMF deregistration can be one of the following:

-	UE Initial Registration.

-	UE Registration area change.

-	Subscription Withdrawn.

-	5GS to EPS Mobility.

The reason for SMF deregistration can be one of the following:

-	SMF Context Transfer.

-	Removal of duplicated (unused) PDU Sessions in the network via deregistration of the old SMF, e.g. if the release of the old PDU Session failed before new PDU Session was created in another SMF.