3GPP TSG-WG SA2 Meeting #153E e-meeting 	S2-2208870r04

Elbonia, Oct 10th – 17th, 2022	(revision of S2-220xxxx)

Source:	China Mobile, Ericsson

Title:	KI#4 Update the Evaluation and Conclusion

Document for:	Approval

Agenda Item:	9.23

Work Item / Release:	FS_eNA_Ph3 / Rel-18

Abstract of the contribution: This contribution proposes to update evaluation and conclusion for KI#4.

# 1. Background

This paper proposes to update an evaluation on the key aspects of the Enhance Data collection and Storage discussed in solutions adopted in the TR for Key Issue 4.

# 2. Text Proposal

It is proposed to adopt the following text within the TR.

# * * * * First change * * * *

## 7.4	Key Issue #4: How to Enhance Data collection and Storage

According to the Table 6.0-1, solution#12, #41 to #46, #61 and #64 to #68 are proposed for Key Issue 4.

Table 7.4-1 gives the comparison of candidate solutions and the corresponding solutions can be classified into four categories:

Table 7.4-1: Comparison of solutions for Key Issue#4

| Solutions | Aspects addressed for the Key Issue |  |  |  | Impacts on services, entities and interfaces |
| --- | --- | --- | --- | --- | --- |
|  | Category-1: ADRF stores types of data other than historical data and analytics | Category-2: DCCF relocation | Category-3: Dealing with buffer overflow of data producer due to muting notifications | Category-4: ADRF/NWDAF data management enhancement |  |
| 12 |  | √ |  |  | Data consumer, DCCF, MFAF. For DCCF and MFAF relocation, new services defined. |
| 41 |  |  | √ |  | Data producer. New parameters defined for existing services. |
| 42 | √ |  |  |  | ADRF, NWDAF. For ML model storage, new services defined. |
| 43 | √ |  |  |  | ADRF,DCCF, NWDAF. For ML model storage, new services defined. |
| 44 |  | √ |  |  | DCCF. New parameters defined for existing services. |
| 45 |  |  | √ |  | Event Consumer NFs (DCCF, NWDAF), Event Producer NFs (AMF, SMF, etc.). New parameters defined for existing services. |
| 46 |  |  |  | √ | NWDAF, DCCF, MFAF, ADRF. New services defined. |
| 61 | √ |  |  |  | NWDAF, ADRF. New services defined. |
| 64 |  |  |  | √ | Data consumer,Data producer. New parameters defined for existing services. |
| 65 |  |  |  | √ | NWDAF. Registration all Analytics IDs in UDM. |
| 66 |  | √ |  |  | DCCF. New parameters defined for existing services. |
| 67 |  |  |  | √ | NWDAF, DCCF, MFAF, ADRF. New parameters defined for existing services. |
| 68 |  |  |  | √ | NWDAF, ADRF. New parameters defined for existing services. |

The solution evaluation for Category-1 is the following:

Models are trained and retrieved from NWDAF containing MTLF. ADRF is suggested as an intermediate storage of models, before NWDAF containing AnLF retrieves them. Nothing prevents an NWDAF containing AnLF to retrieve the models directly from an NWDAF containing MTLF, this service is already available in Rel-17. Adding ADRF as an intermediate storage adds multiple ways to retrieve models, increase number of NFs included and thereby complexity and increase signalling between NFs. Models are viewed as business secrets and shall be protected. Model privacy when storing models in ADRF may be a concern, therefore, SA3 needs to investigate how to securely store the model without leaking privacy information.

In Rel-17, MTLF provides the model storage URL and FQDN information to the AnLF. In other words, the MTLF manages the repository where models are stored, obviously this repository is standardized in R17. Therefore, since the ADRF function is built to maintain a centralized repository for AIML operations across the network, it is logical that ADRF can be reused to store the ML models. In addition, saving the ML model generates the additional signalling, in R17 it is hidden under the implementation, now it has been shown due to the standardization of the procedure.

The solution evaluation for Category-2 is the following:

Solution#12 proposes that the source DCCF initiates the DCCF and MFAF reselection due to UE mobility. Target DCCF (MFAF) gets the data subscription and pending outputs from the source DCCF (MFAF). This solution is aligned with the purpose of deploying a DCCF, which is managing the complexity of subscriptions on behalf of the data consumer. Indeed, the benefit of the solution is that the data consumer does not need to resend the data subscription to the target DCCF and data/notifications are not lost during the process. Also, the solution provides a procedure to cover MFAF re-location, in case MFAFs are deployed separately from DCCF. The solution works when the data subscription is for single UE, not for group UE or any UE.

Solution#44 argues that it is good enough to terminate the data subscription to the old DCCF and the data consumer does the DCCF selection and subscribes to the new DCCF due to UE mobility. Ulike NWDAF relocation, where the target NWDAF may need the same data and ML model from the source NWDAF in order to have consistent analytics performance, nothing is really needed by the target DCCF from the source DCCF. Therefore, Solution#44 proposes a simple and clean solution that the old DCCF terminates the subscription and the data consumer select and subscribe to a new DCCF, which has no impact to the current standards.

Solution#66 proposes that when the data consumer subscribes to the source DCCF directly, upon change of a source DCCF instance, it is required to inform the related data consumer of the event and possibly to update subscriptions between the DCCF and the data consumer. If the central DCCF is deployed, data consumer collects UE data via the central DCCF, data consumer may not need to be notified of the UE mobility events and DCCF reselection events.

We observe the following:

1.	While the Solution #44 procedure has fewer steps compared to Solution #12, it is not necessarily more message efficient. In Solution #44, when a UE moves to an area served by another DCCF, the entire procedure needs to be repeated for each DCCF consumer, including possible discovery of the new DCCF via queries by each consumer to the NRF. In Solution #12 the procedure is executed once with consumers updated with a new Subscription Correlation ID in the last step.

The solution evaluation for Category-3 is the following:

Solution#41 proposes that when the buffer of the data producer is about to overflow, the data producer informs the data consumer that it cannot keep the muted notifications and send all the muted notifications to the data consumer.

Solution#45 covers Solution#41 and it also proposes that the data consumer can provide exception instruction to the data provider and the data provider will make the final decision on how to treat the muted notifications when its buffer is overflow.

The solution evaluation for Category-4 is the following:

Solution#46 proposes enhancement for the ADRF/NWDAF data management. In particular, data consumer can provide Storage Handling Information to ADRF/NWDAF. In addition, ADRF/NWDAF is also provisioned with the storage policy of the operator. Note that, despite all the data belongs to the operator, it may not be enough to only have the storage policy applied at the data storage. Enabling Storage Handling Information from individual data consumer:

-	allows an operator to deploy flexible rules and to gain more granular control over the desired policies. For example, ADRF storage of analytics for an OA&M consumer may be longer than for an NF consumer that requires the same analytics only for transient 5GS optimization

-	supports e2e use case dependent requirements for storing the same analytics. The same analytics/data consumer (e.g. NF) may have longer or shorter analytics storage requirements depending on how the analytics/data are used by the consumer. The ADRF/NWDAF/DCCF has no insight into the use case supported by the analytics, and hence can not alter storage duration without further information (Storage handling information) from the consumer.

Also, it is possible for the data consumer to be informed if some data is to be deleted.

Furthermore, Solution#46 proposes the option in which DCCF maintains the data storage information, so that it executes tasks towards the data storage on behalf of the data consumer and informs the data consumer about the data deletions on behalf of the data storage.

Solution#64 proposes enhancement for data collection and reporting of network data for NWDAF. In order to optimize the performance and accuracy of data collection and reduce the impact on data producers, data can be reported in a specified idle period of the device and data collection frequency can be set to the dynamic Non-fixed sampling ratio. However, notifications are generated when events occur, and it is not clear how the consumer (e.g. NWDAF) will know the best time for the producer to report events. It appears better to let the NF decide when it is too busy to send a report immediately and delay sending a report (which can be done via implementation). The non-fixed sampling ratio, which adjusts according to NF loading seems a more useful mechanism, though data consumer can adjust the sampling ratio dynamically via data subscription updates.

Solution#65 proposes that the NWDAF to register in the UDM for the served UE, also for Analytics IDs that are not UE-related. The NWDAF may do this when the NWDAF is collecting data specific to the UE for the indicated Analytics ID. This further reduces signaling due to data collection and storage volume.

Solution#67 proposes enhancement for the ADRF/NWDAF data management. In particular, data records stored at ADRF are associated to a DataSetTag which enables the service consumer to store and retrieve data sets by specifying such tag in the service request. The advantage of this solution is not only to group data that should belong together using a quick reference to the data set as a whole, but it also enables an analytics consumer (respectively an AnLF) to indicate what exact input data to be used by AnLF (respectively by MTLF) for inference (respectively for training). However, it is not clear what is the reason why analytics consumer should indicate what input data should be used by AnLF..

Solution#68 proposes enhancement for the ADRF/NWDAF data management to enable NWDAF and ADRF to employ data synthesis tools and compression techniques respectively to produce input data for inference and/or training, and to reduce the volume of transferred and stored data. In particular, the solution introduces the DSC Indicator as an attribute included in the Nadrf_DataManagement service operations which indicates the data processing executed over the stored/fetched data, let such processing be data synthesis or compression. The benefit of this solution is that it permits the re-use of synthetic data for privacy preserving purposes, as well as the re-use compressed data to reduce the volume of transferred and stored data. The impact of using synthetic data in model training or other use case needs further study. .

# * * * * End of changes * * * *