3GPP TSG-WG SA2 Meeting #153-e	S2-2209140r05

# Electronic Meeting, 2022 Oct 10-17														rev S2-2205903

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.228 | CR | 1257 | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | IMS cross border mobility with Home routed IMS calls |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Vodafone, Nokia, Nokia Shanghai Bell, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | S8HR, TEI18 |  |  |  |  |  | Date: |  |  | 2022-09-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | If there is no IMS level roaming agreement, the P-CSCF is always in the HPLMN, so called IMS home routed roaming. In such cases when a UE moves between PLMNs, the PLMNs may support keeping the PDN connection/PDU session instead of re-establishing it. In this case there may be a requirement e.g. for the VPLMN to get access to IMS call related information like user identities due to Lawful Interception, see the explanation in LS S2-2203662. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding procedure for mobility between PLMNs for IMS Home routed roaming scenarios. Editorial updates |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Mobility between PLMNs may need re-establishment of PDN connection/PDU session depending on regulatory requirements. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, W.1, W.2.1 (new), W.2.2 (new), W3, W4 (new), W.4.1 (new), W.4.2 (new), W.4.3 (new), Y.9.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

******************** 1st changes ********************

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TS 23.002: "Network Architecture".

[2]	CCITT Recommendation E.164: "Numbering plan for the ISDN era".

[3]	CCITT Recommendation Q.65: "Methodology – Stage 2 of the method for the characterisation of services supported by an ISDN".

[4]	ITU Recommendation I.130: "Method for the characterization of telecommunication services supported by an ISDN and network capabilities of an ISDN".

[5]	3GPP TS 33.310: "Network Domain Security (NDS); Authentication Framework (AF)".

/… unmodified text not shown …./

[99]	3GPP TS 36.300: "Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Overall description".

[100]	3GPP TS 36.321: "Evolved Universal Terrestrial Radio Access (E-UTRA); Medium Access Control (MAC) protocol specification".

[101]	3GPP TS 38.300: "NR; NR and NG-RAN Overall Description".

[102]	3GPP TS 38.321: "NR; Medium Access Control (MAC) protocol specification".

******************** 2nd changes ********************

Annex W (normative):
Support of IMS Services for roaming users in deployments without IMS-level roaming interfaces

# W.1	General

This clause describes the functions that are used to support IMS services for roaming users in deployments without IMS-level roaming interfaces. This annex is applicable to UEs connected to EPS and 5GS. In this roaming model the mobility anchor (i.e., PGW or UPF) is located in the home PLMN and therefore UE IMS signalling and user plane traffic are routed to home PLMN. It is assumed that confidentiality protection is not used in HPLMN and VPLMN for IMS media plane traffic.

For the LBO roaming model where P-CSCF is in VPLMN, see Annex M.

# W.2	Architecture

## W.2.1	EPS

The architecture to support IMS services for roaming users, including Voice over IMS, in deployments without IMS-level roaming interfaces is shown in figure W.2-1

The following architecture requirements apply:

-	P-CSCF (at HPLMN) identifies the serving network (VPLMN) where the UE is located using the procedure defined in clause W.3.

Figure W.2-1: Home Routed IMS architecture

## W.2.2	5GS

See clause Y.9.2.

# W.3	VOID

NOTE: The content of this clause was moved to clause W.4.1.

# W.4	Procedures related to PLMN ID change

## W.4.1	Subscription to changes of PLMN ID at IMS Initial Registration

-

In deployments without IMS-level roaming interfaces, the home network determines the serving PLMN of the UE using procedures defined in TS 23.203 [54], TS 23.503 [95] or TS 29.214 [11], where the P-CSCF requests the PCRF/PCF to report the PLMN identifier where the UE is currently located. The received PLMN ID information is then forwarded in the SIP REGISTER request.

This procedure shall be applied by the P-CSCF at initial UE IMS registration.

The below procedure relates to EPS, the corresponding procedure for 5GS is defined in Annex Y.

Figure W.4.1-1: Subscription by P-CSCF to changes in PLMN ID during initial IMS Registration

1.	The UE sends a SIP REGISTER request to the P-CSCF.

2.	If this is for initial IMS registration then the P-CSCF subscribes to the PCRF/PCF to be notified of the PLMN ID where the UE is currently attached. The subscription to PLMN changes is active as long as the UE is IMS registered.

NOTE:	P-CSCF cancels the subscription to PLMN changes at the PCRF/PCF when the UE is IMS de-registered.

3.	The PCRF/PCF forwards the PLMN ID to the P-CSCF. The P-CSCF stores the PLMN ID.

4.	The P-CSCF includes the received PLMN ID in the SIP REGISTER request before forwarding the request to the I-CSCF.

5.	Normal IMS registration procedure is then completed.

## W.4.2	UE is not active in an IMS voice session

The following procedure shall be applied by IMS at reception of PLMN change when the UE is not active in an IMS voice call, and when the PDN connection/PDU session is kept at PLMN change.

H-PCRF/PCF

I/S-CSCF

:

,

CSCF

-

P

UE

2. NOTIFY (PLMN ID)

SIP NOTIFY (re-register)

SIP NOTIFY (re-register)

5. Re-registration Procedure

SIP NOTIFY (re-register)

1. PLMN change detected

3. SIP MESSAGE (PLMN ID)

4. SIP NOTIFY (re-authenticate)

Figure W.4.2-1: Procedure for PLMN change

1.	EPC/5GC detects PLMN change.

2.	H-PCRF/PCF notifies P-CSCF that PLMN change has occured and indicates the new PLMN ID.

3.	P-CSCF sends SIP MESSAGE with new PLMN ID to S-CSCF.

4.	S-CSCF sends SIP NOTIFY with re-authentication request to the UE.

NOTE 1:	Regulatory requirements in the country of the new PLMN could require the possibility to get access to IMS signalling information. If the source PLMN uses encryption for IMS signaling, the Re-Registration procedure can trigger the HPLMN to change encryption to use null encryption which would enable the new PLMN to get access to future IMS signalling information. Also, the Re-registration can trigger to turn on encryption in the new PLMN if  the HPLMN requires it.

5.	UE initiates re-registration procedure. During the re-registration procedure, the P-CSCF may update the SIP signalling encryption depending on roaming agreements. E.g., moving from HPLMN to VPLMN can result in turning off IMS encryption while retaining integrity protection of IMS signalling.

## W.4.3	UE is active in an IMS voice session

The following procedure shall be applied by IMS at reception of PLMN change when the UE is active in an IMS voice call, and when the PDN connection/PDU session is kept at PLMN change.

6. SIP MESSAGE (Re-INVITE request)

UE

P

-

CSCF

I/S-CSCF

H-PCRF/PCF

TAS

5.  re-Registration Procedure

0. UE active in a voice call

1. PLMN change detected

2. NOTIFY (PLMN ID)

3. SIP MESSAGE (PLMN ID)

3a. SIP MESSAGE (PLMN ID)

4. SIP NOTIFY (re-authenticate)

5. IMS Re-Registration procedure

8. SIP 200 OK

7. SIP Re-INVITE

Figure W.4.3-1: PLMN change detected by EPC/5GC

0.	UE is active in an IMS voice call.

1.	EPC/5GC detects PLMN change.

2.	H-PCRF/PCF notifies the P-CSCF that PLMN change has occurred and indicates the new PLMN ID.

3.	P-CSCF sends SIP MESSAGE with the new PLMN ID to the S-CSCF

3a.	P-CSCF sends SIP MESSAGE with new PLMN ID to the TAS so that the TAS may update any charging records with the new PLMN ID.

4.	S-CSCF sends SIP NOTIFY with re-authentication request to the UE.

NOTE 1:	Regulatory requirements in the country of the new PLMN could require the possibility to get access to IMS signalling information. If the source PLMN uses encryption for IMS signaling, the Re-Registration procedure can trigger the HPLMN to change encryption to use null encryption which would enable the new PLMN to get access to future IMS signalling information. Also, the Re-registration can trigger to turn on encryption in the new PLMN  if the HPLMN requires it.

5.	UE initiates re-registration procedure. During the re-registration procedure, the P-CSCF may update the SIP signalling encryption depending on roaming agreements. E.g., moving from HPLMN to VPLMN can result in turning off IMS encryption while retaining integrity protection of IMS signalling.

6.	When the re-registration procedure is completed, the P-CSCF sends a SIP MESSAGE with "re-INVITE request" to the TAS.

7.	TAS sends SIP Re-INVITE to UE including call related information (e.g., calling and called party information, media endpoints and used codecs). Codec endpoints are translated in the P-CSCF if needed.

8.	UE sends SIP 200 OK to TAS.

NOTE 2:	Whether to trigger (in step 6) sending the re-INVITE request to the UE is based on regulatory requirements in the new PLMN.The signalling sent in step 7 would be encrypted (or not) according to the result of the re-registration procedure in step 5.

******************** 3rd changes ********************

## Y.9.4	Subscription to changes in PLMN ID at IMS Initial Registration

In IMS local breakout where P-CSCF is located in VPLMN (see Annex M.1 and Annex M.3), the home network determines the serving PLMN of the UE from the location of the P-CSCF during initial IMS Registration, using the P-CSCF network identifier.

In deployments without IMS-level roaming interfaces, the home network determines the serving PLMN of the UE using procedure defined in TS 23.503 [95], where P-CSCF requests the PCF to report the PLMN identifier where the UE is currently located. The received PLMN ID information is then forwarded in the SIP REGISTER request.

This procedure shall be applied by the P-CSCF at initial UE IMS registration.

Figure Y.9.4-1: Subscription by P-CSCF to changes in PLMN ID during initial IMS Registration

1.	The UE sends a SIP REGISTER request to the P-CSCF.

2.	If this is initial IMS registration then the P-CSCF subscribes to the PCF to be notified of the PLMN ID where the UE is currently attached. The subscription to PLMN changes is active as long as the UE is IMS registered.

NOTE:	P-CSCF cancels the subscription to PLMN changes at the PCF when the UE is IMS de-registered.

3. The PCF forwards the PLMN ID to the P-CSCF. The P-CSCF stores the PLMN ID.

4.	The P-CSCF includes the received PLMN ID in the SIP REGISTER request before forwarding the request to the I-CSCF.

5.	Normal IMS registration procedure is then completed.

******************** End changes ********************