SA WG2 Meeting #154AHE (e-meeting)	S2-2300889

Jan 16 – 20, 2023	(revision of S2-2210458)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.548 | CR | 0096 | rev | 0 | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | ECS Address Configuration Information delivery in roaming |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo, Huawei, Nokia, Nokia Shanghai Bell, CMCC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | EDGE_ph2 |  |  |  |  |  | Date: |  |  | 2023-01-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | As the conclusion in TR 23.700-62[x], in the UDM and UDR, the subscription data of EACI is stored per PLMN ID. The V-SMF (for the LBO case) or the H-SMF via the V-SMF (for the HR case) provides the UE with the EACI as indicated in step 11 in figure 4.3.2.2.1-1 of TS 23.502 [9]. For the HR case, the V-SMF does not modify, but just delivers the EACI provided by the H-SMF. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add PLMN ID in ECS Address Configuration Information provided by AF, that the 5GC can identify which PLMN the ECAI associated to. Add a new clause 6.5.2.X to describe the ECS Address Configuration Information provisioning in roaming case. Add a new clause 6.5.2.X to support ECS Address Configuration Information provisioning from VPLMN in roaming case. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The UE in HR case cannot receive ECS Address Configuration Information, and H-SMF in HPLMN cannot decide in certain roaming PLMN, which ECS Address Configuration Information should be delivered to UE. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.5.2.1, 6.5.2.2, 6.5.2.X (new text) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Change 1 * * *

6.5.2.1	ECS Address Configuration Information

The ECS Address Configuration Information consists of one or more ECS Configuration Information as defined in clause 8.3.2.1 of TS 23.558 [5]. The ECS Configuration Information may contain Spatial Validity Conditions, which includes one of the following alternatives:

-	a Geographical Service Area (see TS 23.558 [5]);

-	a list of TA(s); or

-	a list of countries (list of MCC).

-	a list of PLMN IDs (see Table 4.15.6.3d-1 of TS 23.502[x]).

A UE may receive multiple instances of ECS Address Configuration Information e.g., corresponding to different ECSPs (e.g., the MNO or a 3rd party service provider).

The ECS Address Configuration Information is sent to the UE on a per PDU Session basis. The same PDU session can be used by multiple ECS providers.

The SMF does not need to be aware of the internal structure of the ECS Address Configuration Information.

* * * Start of Change 2, all new text * * *

6.5.2.X	ECS Address Provisioning in Roaming

For both LBO and HR case, the subscription data of the ECS Address Configuration Information in UDM or UDR is stored per PLMN ID.

For the LBO case, an AF in the visited PLMN may provide the EACI via External Parameter Provisioning procedure as described in clause 4.15.6.3d in TS 23.502[3] to UDM via H-NEF. This ECS Address Configuration Information is further provided to SMF as part of subscription information.

For the HR case when access to EHE in VPLMN is allowed:

-	The HPLMN has the knowledge of EACI in the VPLMN: For this scenario, the AF is able to interact with NEF in HPLMN. The AF may provide the VPLMN EACI to H-NEF via External Parameter Provisioning procedure as described in clause 4.15.6.3d in TS 23.502[3], and further to the UDM. During the HR PDU session establishment procedure, the H-SMF sends the VPLMN EACI to V-SMF and then to UE. The V-SMF does not modify, but just delivers the EACI provided by the H-SMF.

-	HPLMN does not have the knowledge of EACI in VPLMN: For this scenario, the AF can’t interact with NEF in HPLMN. The AF deployed in the VPLMN provides the VPLMN EACI to the V-SMF via the V-NEF as defined in clause 6.5.2.x. During the HR PDU Session establishment, the V-SMF sends the VPLMN EACI obtained from V-NEF to the H-SMF, and H-SMF decides the VPLMN ECS Address Configuration Information sent to V-SMF and then to UE according to the PLMN ID additionally.

* * * Start of Change 3, all new text * * *

6.5.2.X	ECS Address Configuration Information Provisioning via VPLMN

6.5.2.X.1	General

This procedure describes the provisioning of ECS Address Configuration Information from an AF via NEF in VPLMN.

For provisioning of ECS Address Configuration Information from AF to NEF in HPLMN, External Parameter Provisioning procedure described in clause 4.15.6.3d in TS 23.502[3] shall be used.

Editor’s note: it is FFS whether a new API is used for V-AF to provide ECAI information for roamers or whether External Parameter Provisioning is used, defining e.g. an extra parameter that tells whether Any UE means a) Any subscriber of the serving PLMN only (NEF configures UDM only) or b) Any UE of another (h)PLMN in HR-SBO (NEF configures UDR only) Or c) both (NEF configures UDM and UDR). The section 6.5.2.X will be changed according to the final decision of V-AF to provide ECAI.

6.5.2.X.2	ECS Address Configuration Information Provision from AF via NEF in VPLMN

The AF provides non-PDU Session specific ECS Address Configuration Information via NEF in VPLMN to 5GC is defined in this clause.

UDR

NEF

AF

1.Nnef_ECSAddress_Create /Update/Delete Request

2.NEF Handling

3.Nudr_DM_Create/Update/Delete Request

4.Nudr_DM_Create/Update/Delete Response

5.Nnef_ECSAddress_Create /Update/Delete Response

Figure 6.2.3.X.1-1 ECS Address Configuration Information provisioning to UDR

1.	The AF invokes the Nnef_ECSAddress_Create /Update/Delete service operation to provide ECS Address Configuration Information to the NEF in VPLMN.

2.	NEF checks whether the AF is authorized to perform the, ECS Address Configuration Information provisioning request based on the operator policies.

3.	The NEF invokes the Nudr_DM_Create/Update/Delete to the UDR in VPLMN if it is authorized.

4.	The UDR stores/updates/removes the corresponding information (and responds a Nudr_DM_Create/Update/Delete Response to the NEF).

5.	The NEF sends Nnef_ECSAddress_Create/Update/Delete Response to the AF.

6.5.2.X.3	ECS Address Configuration Information Provision to the SMF in VPLMN

The SMF in VPLMN (i.e. V-SMF for HR roaming case) may receive the ECS Address Configuration Information from NEF in VPLMN via Subscribe/Notify procedure defined in this clause.

SMF

NEF

3.Nnef_ ECSAddress _Notify Request

4.Nnef_ ECSAddress _Notify Response

1.Nnef_ ECSAddress _Subscribe Request

2.Nnef_ ECSAddress _Subscribe Response

Figure 6.2.3.X.2-1: ECS Address Configuration Information provisioning to SMF in VPLMN

1-2.	As pre-requisite condition, the SMF subscribes to ECS Address Configuration Information Change Notification from the NEF by sending Nnef_ECSAddress_Subscribe message. The SMF may indicate that the current status of ECS Address Configuration Information shall be notified immediately (if available). The SMF may indicate for which (list of) DNN and/or S-NSSAI and/or Internal Group Identifier (if available) it subscribes.

3-4.	The NEF invokes Nnef_ECSAddress_Notify (ECS Address Configuration Information) to the SMF if the ECS Address Configuration Information is updated. If there is ECS Address Configuration Information available and immediate report is required, the NEF notifies the SMF(s) with such information immediately. NEF may retrieve the ECS Address Configuration Information from UDR using Nudr_DM_Query/Notify.

* * * End of Change * * *