3GPP TSG-WG SA2 Meeting #154  	S2-2210496

Toulouse, France, Nov 14 – 18, 2022	(revision of S2-220xxxx)

Source:	Samsung

Title:	Conclusion for FS_5TRS_URLLC KI#1

Document for:	Approval

Agenda Item:	9.7

Work Item / Release:	FS_5TRS_URLLC / Rel-18

Abstract: Providing Conclusion for KI#1.

1. Introduction/Discussion

There's an EN like the following.

Editor's note:	Whether both UE and AF need to be informed about time synchronization status details simultaneously, and in which cases only UE or only AF may receive it is FFS.

According to KI#3 solutions, there can be cases where 5GS provides Sync services based on UE subscription information without AF requests. In those cases, only UE receives time synchronization status. In cases where 5GS provides Sync services based on AF request with UE subscription information, both AF and UE need to be informed about time synchronization status.

2. Text Proposal

It is proposed to capture the following changes to TR 23.700-25.

# * * * * First change * * * *

8.5	Conclusion for KI #1: 5GS network timing synchronization status and reporting

The following bullet points summarize the principles for the way forward:

-	Detecting and reporting RAN and UPF timing synchronization status to TSCTSF.

Editor's note:	What the RAN and UPF timing synchronization status consists of is FFS.

-	NG-RAN and UPF/NW-TT can detect timing synchronization degradation/failure/improvement locally.

NOTE:	The detection is performed based on information provided by time synchronization protocols used in the transport network for both RAN and UPF, or, in the case of NG-RAN, using information provided by a local GNSS receiver. However, in any case, the details on how exactly NG-RAN/UPF detects timing synchronization degradation/failure/improvement locally are beyond the scope of 3GPP.

-	TSCTSF may receive network timing synchronization status information of RAN and UPF/NW-TT directly from OAM.

-	Alternatively, TSCTSF may receive network timing synchronization status information of RAN and UPF/NW-TT using control plane signalling at node level. For UPF/NW-TT case the TSCTSF may use UMIC. For NG-RAN case the TSCTSF may obtain NG-RAN information via the AMF (i.e. AMF uses NGAP signalling to configure the NG-RAN reporting).

Editor's note:	How to support additional methods (NGAP, control plane signalling, etc.) to obtain timing synchronization status from NG-RAN requires RAN feedback.

-	UE determining that the RAN timing synchronization status changed using:

-	SIB broadcast information to enable UEs in RRC_IDLE and RRC_INACTIVE and in case of RRC_CONNECTED UEs, dedicated RRC signalling, to enable UEs to determine that:

-	the timing synchronization status of the cell that the UE is camping on has changed;

-	the timing synchronization status of the new cell the UE is camping on after cell reselection is different compared to the timing synchronization status of the cell that the UE was previously camping on.

-	If the UE has determined that the RAN timing synchronization status has changed and the UE has been requested by the TSCTSF to connect to the network in case the RAN timing synchronization status changes, the UE performs a registration (if the UE is in RRC_IDLE) or the UE Triggered Connection Resume in RRC Inactive procedure (if the UE is in RRC_INACTIVE).

Editor's note:	Whether the UE performs a Registration also in Inactive is FFS.

Editor's note:	The details of which existing/new SIB information the UE uses to determine that the RAN timing synchronization status has changed is FFS and will be coordinated with RAN WGs.

Editor's note:	How to provide RAN timing synchronization status information to the UE is FFS and will be coordinated with RAN WGs. Details of which RAN timing synchronization status information to provide to the UE will be determined based on feedback from ITU-T.

-	Determining UEs impacted by RAN timing synchronization status degradation/improvement:

-	TSCTSF subscribes to receive location information (RAN node granularity) from AMF for UEs that AF requested time synchronization for or which are configured for (g)PTP-based time synchronization based on subscription.

-	When activating time synchronization for a UE, TSCTSF requests the UE to connect to the network via AMF (i.e. to perform a registration if the UE is in RRC_IDLE or the UE Triggered Connection Resume in RRC Inactive (if the UE is in RRC_INACTIVE) in case when  the UE later detects that the RAN timing synchronization status has changed while the UE is in RRC_IDLE or RRC_INACTIVE.

-	TSCTSF correlates information about impacted RAN nodes and the UE location information received from AMF to determine the UEs impacted by RAN timing status degradation/failure/improvement.

Editor's note:	Whether alternatively NG-RAN can be responsible for determining the impacted UE(s) and sending the NG-RAN timing synchronization status reports to the AMF via NG-AP signalling, together with the impacted UE(s) is FFS.

-	Determining UEs impacted by UPF timing synchronization status degradation or improvement (only for the case when UPF/NW-TT is involved in providing time information to DS-TT):

-	TSCTSF determines the UEs for which an impacted UPF/NW-TT is configured to send (g)PTP messages.

-	Informing AFs about network timing synchronization status degradation or improvement:

-	If TSCTSF has determined UEs impacted by RAN or UPF timing synchronization status degradation or improvement or failure then TSCTSF informs the AF about the timing synchronization status for those UEs if the AF was the requester of the time synchronization service.

-	The AF may subscribe to time synchronization service status for a UE (or group of UEs) for which the AF requests or has requested time synchronization service (for ASTI or (g)PTP services).

-	For the subscribed AFs the TSCTSF provides time synchronization service status.

Editor's note:	Which information to provide to AF is FFS.

NOTE:	According to KI#3 solutions, there can be cases where 5GS provides Sync services based on UE subscription information without AF requests. In those cases, only UE receives time synchronization status. In cases where 5GS provides Sync services based on AF request with UE subscription information, both AF and UE need to be informed about time synchronization status.

-	Deactivating/reactivating/updating time synchronization services based on RAN/UPF timing synchronization status changes:

-	PTP case: For UEs that are part of a PTP instance and which are impacted by RAN or UPF time synchronization status degradation or improvement:

-	If TSCTSF determines that the Time synchronization error budget provided by AF can still be met, then TSCTSF may update the clockQuality information sent in Announce messages (see clause 7.6.2 of IEEE 1588 [8]) for the PTP instance using existing procedures and existing PMIC/UMIC information. The handling of Announce messages follows existing procedures as described in TS 23.501 [2].

-	If TSCTSF determines that the Time synchronization error budget provided by AF cannot be met (see above) then TSCTSF temporarily removes the UE/DS-TT from the PTP instance using existing procedures in clause K.2.2.1 and clause K.2.2.4 of TS 23.501 [2]).

-	If TSCTSF determines that the Time synchronization error budget provided by AF can be met again then TSCTSF adds the DS-TT PTP port to the PTP instance again and also re-activates the Grandmaster functionality.

-	ASTI case: TSCTSF updates the access stratum time distribution indication to "enable" or "disable" and forwards the attribute to the serving NG-RAN nodes for the impacted UEs via AMF depending on whether the Time synchronization error budget can or cannot be met (following Rel-17 operations as described in clause 4.15.9.4 of TS 23.502 [3]).

Editor's note:	It is FFS whether AF can also decide to deactivate/reactivate/update time synchronization services based on RAN or UPF timing synchronization status degradation or improvement.

# * * * * End of changes * * * *