3GPP TSG-WG SA2 Meeting #154 	S2-2210558

Toulouse, France, November 14 – 18, 2022

Source:	Samsung

Title:	Update Conclusion on KI#1

Document for:	Approval

Agenda Item:	9.14

Work Item / Release:	FS_eNS_Ph3 / Rel-18

Abstract: this paper is to propose partial conclusion of key issue #1.

1. Introduction/Discussion

This paper is to resolve the Editor’s Notes in the conclusions for KI#1 of FS_eNS_Ph3.

2. Text Proposal

Based on the analysis above, it is proposed to capture the following changes vs. TR 23.700-41 110.

# * * * * First change * * * *

8	Conclusions

8.1	Conclusions for KI#1

The following principles are concluded for the normative work.

NOTE:	The IE names for the alternative S-NSSAI and other new information in the conclusion is to be determined during normative phase.

0)	The trigger to replace a currently used S-NSSAI with an alternative S-NSSAI is either based on local configuration (e.g. based on trigger from OAM) or based on AM PCF notification.

NOTE 1:	What the SBI service is used for AM PCF notification will be determined in normative phase.

1)	The AMF determines the alternative S-NSSAI to be used to replace the old S-NSSAI. The AMF may interact with the AM PCF and/or with the NSSF to determine the alternative S-NSSAI. The alternative S-NSSAI may or may not be part of the Subscribed S-NSSAIs of the UE. The Subscribed S-NSSAIs of the UE are not updated for the purpose of KI#1.

NOTE 2:	The use of the alternative S-NSSAI not part of the Subscribed S-NSSAIs of the UE is for the roaming case.

2)	In cases where the old S-NSSAI is replaced by an alternative S-NSSAI, the following principles apply:

a)	The AMF performs UE MM configuration update (e.g. UCU procedure) to include the alternative S-NSSAI in the Allowed NSSAI and/or in the Configured NSSAI, if not included yet. The AMF provides information that the alternative S-NSSAI is associated with a replaced/mapped value to the old S-NSSAI in both roaming or non-roaming case; when the UE receives the information from the AMF, the UE does not locally release the old S-NSSAI and does not request to establish PDU sessions using the old S-NSSAI.

NOTE 3: Whether the existing Mapping Of Allowed NSSAI can be re-used, or an alternative IE (e.g. Replaced NSSAI) has to be specified will be determined in the normative phase.

-

b)	For a new PDU Session establishment request from the UE on the old S-NSSAI (e.g., when the UE has not received the information to prevent this), the principle 4c) can be applied.

c)	For an existing PDU Session transfer from the old S-NSSAI to an alternative S-NSSAI:

-	The AMF notifies the current SMF, e.g. by triggering Nsmf_PDUSession_UpdateSMContext service operation, that the PDU Session is to be relocated to an alternative S-NSSAI and indicates the alternative S-NSSAI.

NOTE 4: How to handle the home-routed case will be determined in the normative phase.

-	If the old S-NSSAI and the alternative S-NSSAI are associated with the different NSI, the SMF sends to the UE either PDU Session Modification Command (if the PDU Session is of SSC mode 3) or PDU Session Release/Modification Command (if the PDU Session is of SSC mode 1 or 2) and indicates to the UE that the PDU Session can be re-established on the alternative S-NSSAI.

-  If the old S-NSSAI and the alternative S-NSSAI are associated with the same NSI, the SMF sends to the UE PDU Session Release/Modification Command and indicates to the UE that the S-NSSAI of the PDU Session can be changed to the alternative S-NSSAI without the PDU Session re-establishment.

-  If AMF indicated the alternative S-NSSAI the SMF provides the alternative S-NSSAI to the UE.

--

# * * * * End of changes * * * *