3GPP TSG-WG SA2 Meeting #154 	S2-2210609

Toulouse, France, November 14 – 18, 2022	(revision of S2-221xxxx)

Source:	Nokia, Nokia Shanghai Bell

Title:	KI#2: Update of the Conclusion

Document for:	Approval

Agenda Item:	9.19.1

Work Item / Release:	FS_XRM / Rel-18

Abstract: This document updates KI#2 conclusion agreed in the previous meeting.

# 1. Introduction/Discussion

The conclusions for Key Issue #2 agreed at SA2#153-e refer to most part of the conclusions about Key Issue #1. There is however an Editor’s Note about an issue that is specific to KI #2, where the traffic flows may be transmitted by multiple UEs. This paper proposes a resolution to that EN. The rationale for the proposed resolution is given below:

Editor's note: Whether multiple PCFs may be involved in the procedures for multiple UEs and how the coordination across PCFs works in that case is determined in the normative phase.

It is in principle possible that when multiple UEs are involved, they or their PDU sessions are served by separate PCFs. In that case, any policy enforcement across the UEs and their traffic flows (for synchronized delivery, admission control or QoS fulfilment and retainment) will require coordination across the involved PCFs. It is argued here this is best facilitated by NEF as outlined in Solution #64.

NOTE: If the method for coordination across PCFs cannot be agreed, the next option is to conclude that in this release the common policies across UEs can be requested from and enforced by the 5GS only in the special case that the UEs are served by the same PCF. This can still be a valid option for many deployments.

# 2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-60.

# * * * * First change * * * *

## 8.X	Conclusions for Key issue#2

The following aspects are concluded as principles for the normative work:

-	For multi-modality flows transmitted by multiple UEs, the AF manages the flows in the following ways:

-	The AF procedure is extended with QoS requirements for individual service flows and treatment requirements and policies according to the conclusions for Key Issue #1.

-	In case multiple UEs are involved, AF provides the information related to all UEs to NEF either using a single procedure or using multiple UE specific procedures.

-	In either case, the AF may indicate that specific service data flows belong to the same multimodal service by associating them with a common ID. PCF uses the individual service data flow QoS requirements and requirements and policies in the same way as concluded for Key Issue #1.

-	If the PDU Sessions of the multiple UEs involved are served by separate PCFs, the coordination among the PCFs is facilitated by NEF.

# * * * * End of changes * * * *