3GPP TSG-WG SA2 Meeting #154	S2-2210703

# Toulouse, France, November 14 – 18, 2022		(revision of S2-220xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0580 | rev | - | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | NWDAF-assisted decision of E2E Redundant Transmission |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | DUMMY |  |  |  |  |  | Date: |  |  | 2022-11-04 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | It is concluded that NWDAF can be enhanced to expose redundant transmission experience related analytics to PCF, to assist the PCF to decide on the URSP for E2E redundant transmission. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The redundant transmission experience related analytics are updated to support the adjustment of URSP for E2E redundant transmission in the PCF. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Conclusion of TR cannot be specified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.13 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR is part of outcome of SI FS_eNA_Ph3 and the WI code for the work is not assigned yet & will replace WI code DUMMY when available. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

6.13	Redundant Transmission Experience related analytics

6.13.1	General

This clause describes the Redundant Transmission Experience related analytics. These analytics may be used by the SMF to determine whether redundant transmission on N3/N9 interfaces (see clause 5.33.2.2 of TS 23.501 [2]) shall be performed, or (if it had been activated) shall be stopped. Alternatively, these analytics may also be used by the PCF to determine whether redundant transmission on E2E redundant user plane paths (see clause 5.33.2.1 of TS 23.501 [2]) shall be performed, or (if it had been activated) shall be stopped.

The service consumer may be a NF (e.g. SMF or PCF).

The consumer of these analytics may indicate in the request:

-	Analytics ID = "Redundant Transmission Experience".

-	Target of Analytics Reporting: a single UE (SUPI), any UE, or a group of UEs (an Internal Group ID).

-	Analytics Filter Information optionally containing:

-	Area of Interest;

-	S-NSSAI;

-	DNN;

-	PDU Session ID;

-	optional list of analytics subsets that are requested (i.e. UL/DL packet drop rate GTP-U, UL/DL packet delay GTP-U, see clause 6.13.3).

-	An Analytics target period indicates the time period over which the statistics or predictions are requested.

-	Preferred level of accuracy of the analytics;

-	Preferred order of results for the list of Redundant Transmission Experience:

-	ordering criterion: "time slot start"; and

-	order: ascending or descending;

-	In a subscription, the Notification Correlation Id and the Notification Target Address are included.

6.13.2	Input Data

The NWDAF supporting data analytics on Redundant Transmission Experience shall be able to collect UE mobility information from OAM, 5GC and AFs and service data from AF, as described in clause 6.7.2.2. In addition, NWDAF shall be able to collect the information for PDU session which is established with redundant transmission from the SMF. UE mobility information is specified in Table 6.7.2.2-1 and service data from AF related to UE mobility in Table 6.7.2.2-2. In addition, the NWDAF shall be able to collect performance measurements on user data congestion as specified in Table 6.8.2-1 for user data congestion analytics.

Additionally, the NWDAF collects the following input (see Table 6.13.2-1 and Table 6.13.2-2) according to existing measurements defined in clause 5.33.3 QoS Monitoring to Assist URLLC Service of TS 23.501 [2].

Table 6.13.2-1: Packet drop and/or packet delay measurement per QFI or GTP level

| Information | Source | Description |
| --- | --- | --- |
| UL/DL packet drop rate GTP-U | OAM (see NOTE 2) | UL/DL packet drop rate measurement on GTP path on N3. |
| UL/DL packet delay GTP | OAM (see NOTE 2) | UL/DL packet delay measurement round trip on GTP path on N3. |
| E2E UL/DL packet delay | SMF/UPF (see NOTE 2, NOTE 4) | End-to-End (between UE and UPF) packet delay measurements. |
| E2E UL/DL packet drop rate | OAM (see NOTE 2, NOTE 4) | End-to-End (between UE and UPF) packet drop rate measurements. |
| UL/DL packet delay RAN | SMF/UPF (see NOTE 3 and NOTE 4) | RAN part of UL/DL packet delay measurement. |
| NOTE 1: The information in this table is provided both as the base to compare with the redundant transmission performance as well as when redundant transmission is enabled. NOTE 2: Refer to clause 5.1 of TS 28.552 [8] for the performance measurement in NG-RAN and clause 5.4 1 of TS 28.552 [8] for the performance measurement in UPF. In addition, Annex A of TS 28.552 [8] describes various performance measurements. NOTE 3: It is assumed UL/DL packet delay RAN includes the measurements for both primary and secondary paths. As specified in clause 5.33.3 of TS 23.501 [2], if the redundant transmission on N3/N9 interfaces is activated, the UPF and NG-RAN performs QoS monitoring for both UP paths. The UPF reports the packet delay of the two UP paths respectively to the SMF. NOTE 4: Only applicable for redundant transmission experience related analytics for E2E redundant transmission. |  |  |

Table 6.13.2-2: The information related to PDU Session established

| Information | Source | Description |
| --- | --- | --- |
| DNN | SMF | Data Network Name associated for URLLC service. |
| UP with redundant transmission | SMF | Redundant transmission is setup or terminated. |

6.13.3	Output Analytics

The NWDAF supporting data analytics on Redundant Transmission Experience shall be able to provide Redundant Transmission Experience analytics as defined in Table 6.13.3-1 and Table 6.13.3-2.

Table 6.13.3-1: Redundant Transmission Experience statistics

| Information |  | Description |  |
| --- | --- | --- | --- |
| UE group ID or UE ID, any UE |  | Identifies a UE, any UE, or a group of UEs. |  |
| DNN |  | Data Network Name associated for URLLC service. |  |
| Spatial validity |  | Area where the Redundant Transmission Experience statistics applies. If Area of Interest information was provided in the request or subscription, spatial validity should be the requested Area of Interest. |  |
| Time slot entry (1..max) |  | List of time slots during the Analytics target period. |  |
| > Time slot start |  | Time slot start within the Analytics target period. |  |
| > Duration |  | Duration of the time slot (average and variance). |  |
| > Observed Redundant Transmission Experience |  | Observed Redundant Transmission Experience related information during the Analytics target period. |  |
| >> UL/DL packet drop rate GTP-U (NOTE 2) |  | Observed UL/DL packet drop rate on GTP-U path on N3 (average, variance). |  |
| >> UL/DL packet delay GTP-U (NOTE 2) |  | Observed UL/DL packet delay round trip on GTP-U path on N3 (average, variance). |  |
| >> E2E UL/DL packet drop rate (NOTE 4) |  | Observed End-to-End (between UE and UPF) UL/DL packet drop rate (average, variance). |  |
| >> E2E UL/DL packet delay (NOTE 4) |  | Observed End-to-End (between UE and UPF) UL/DL packet delay (average, variance). |  |
| >> Primary RAN Path Redundant Transmission Experience (UE to Primary NG-RAN) (NOTE 4) |  | Redundant Transmission Experience value of primary RAN path. |  |
| >> Secondary (Redundant) RAN Path Redundant Transmission Experience (UE to secondary NG-RAN) (NOTE 4) |  | Redundant Transmission Experience value of secondary RAN (redundant) path. |  |
| >> Primary N3 tunnel Path Redundant Transmission Experience (NG-RAN to UPF) (NOTE 3, NOTE 4) |  | Redundant Transmission Experience value of primary N3 tunnel. |  |
| >> Secondary N3 Tunnel Path Redundant Transmission Experience (NG-RAN to UPF) (NOTE 3, NOTE 4) |  | Redundant Transmission Experience value of secondary N3 tunnel. |  |
| > Redundant Transmission Status |  | Redundant Transmission Status, i.e. redundant transmission was activated or not activated for the time slot entry. |  |
| > Ratio |  | Percentage on which UE, any UE, or UE group experience the packet drop rate and packet delay. |  |
| NOTE 1: The Observed Redundant Transmission Experience can be further derived by SMF from the observed UL/DL packet drop rate GTP-U and UL/DL packet delay GTP-U. NOTE 2: This information element is an analytics subset that can be used in "list of analytics subsets that are requested" and only applicable when Target of Analytics Reporting is for a single UE. NOTE 3: In case of I-UPF, this value includes both N3 and N9 experience. NOTE 4: Only applicable for redundant transmission experience related analytics for E2E redundant transmission. |  |  |  |

Table 6.13.3-2: Redundant Transmission Experience predictions

| Information | Description |
| --- | --- |
| UE group ID or UE ID, any UE | Identifies a UE or, any UE, a group of UEs. |
| DNN | Data Network Name associated for URLLC service. |
| Spatial validity | Area where the estimated Redundant Transmission Experience predictions applies. If Area of Interest information was provided in the request or subscription, spatial validity should be the requested Area of Interest. |
| Time slot entry (1..max) | List of predicted time slots. |
| >Time slot start | Time slot start time within the Analytics target period. |
| > Duration | Duration of the time slot. |
| > Predicted Redundant Transmission Experience | Predicted Redundant Transmission Experience related information during the Analytics target period. |
| >> UL/DL packet drop rate GTP-U (NOTE 2) | Predicted UL/DL packet drop rate on GTP-U path on N3 (average, variance). |
| >> UL/DL packet delay GTP-U (NOTE 2) | Predicted UL/DL packet delay round trip on GTP-U path on N3 (average, variance). |
| >> E2E UL/DL packet drop rate (NOTE 4) | Predicted End-to-End (between UE and UPF) UL/DL packet drop rate (average, variance). |
| >> E2E UL/DL packet delay (NOTE 4) | Predicted End-to-End (between UE and UPF) UL/DL packet delay (average, variance). |
| >> Primary RAN Path Redundant Transmission Experience (UE to Primary NG-RAN) (NOTE 4) | Predicted Redundant Transmission Experience value of primary RAN path during the Analytics target period. |
| >> Secondary (Redundant) RAN Path Redundant Transmission Experience (UE to secondary NG-RAN) (NOTE 4) | Predicted Redundant Transmission Experience value of secondary RAN (redundant) path during the Analytics target period. |
| >> Primary N3 Tunnel Path Redundant Transmission Experience (NG-RAN to UPF) (NOTE 3, NOTE 4) | Predicted Redundant Transmission Experience value of primary N3 tunnel during the analytics target period. |
| >> Secondary N3 Tunnel Path Redundant Transmission Experience (NG-RAN to UPF) (NOTE 3, NOTE 4) | Predicted Redundant Transmission Experience value of secondary N3 tunnel during the analytics period. |
| > Ratio | Percentage on which the UE, any UE, or UE group may efficiently use the PDU session with redundant transmission. |
| > Confidence | Confidence of this prediction. |
| NOTE 1: The Predicted Redundant Transmission Experience can be further derived by the SMF from the predicted UL/DL packet drop rate GTP-U and UL/DL packet delay GTP-U and based on which the SMF can decide to start redundant transmission or not. NOTE 2: This information element is an analytics subset that can be used in "list of analytics subsets that are requested" and only applicable when Target of Analytics Reporting is for a single UE. NOTE 3: In case of I-UPF, this value includes both N3 and N9 experience. NOTE 4: Only applicable for redundant transmission experience related analytics for E2E redundant transmission. |  |

6.13.4	Procedures

6.13.4.1	Analytics Procedure for N3/N9 redundant transmission

Figure 6.13.4.1-1 shows the analytics procedure. The NWDAF can provide analytics, in the form of statistics or predictions or both.

Figure 6.13.4.1-1: Redundant Transmission Experience analytics for N3/N9 redundant transmission provided to an NF

1.	The analytics consumer sends a request to the NWDAF for analytics on a specific UE, any UE, or a group of UEs, using either the Nnwdaf_AnalyticsInfo or Nnwdaf_AnalyticsSubscription service. The NF can request statistics or predictions or both. The type of analytics is set to Redundant Transmission Experience. The NF provides the UE id or Internal Group ID in the Target of Analytics Reporting. Analytics Filter Information optionally contains DNN, S-NSSAI, Area of Interest, etc.

2.	If the request is authorized and in order to provide the requested analytics, the NWDAF may subscribe to events with all the serving AMFs for notification of location changes and may subscribe to events with SMFs serving PDU Session on URLLC service for notification of redundant transmission related information.

The NWDAF may subscribe the service data from AF(s) by invoking Naf_EventExposure_Subscribe service or Nnef_EventExposure_Subscribe (if via NEF).

The NWDAF collects UE mobility information and/or packet measurement information from OAM, following the procedure captured in clause 6.2.3.2. The NWDAF collects redundant transmission status from SMF.

NOTE:	The NWDAF determines the AMF serving the UE, any UE, or the group of UEs as described in clause 6.2.2.1.

This step may be skipped when e.g. the NWDAF already has the requested analytics available.

3.	The NWDAF derives requested analytics.

4.	The NWDAF provide requested Redundant Transmission Experience analytics to the NF, using either the Nnwdaf_AnalyticsInfo_Request response or Nnwdaf_AnalyticsSubscription_Notify, depending on the service used in step 1.

5-7.	If at step 1, the NF has subscribed to receive notifications for Redundant Transmission Experience analytics, after receiving event notification from the AMFs, SMF, AFs and OAM subscribed by NWDAF in step 2, the NWDAF may generate new analytics and provide them to the NF.

If a service consumer is SMF, the Redundant Transmission Experience analytics can be used to make decision if that redundant transmission shall be performed or (if activated) shall be stopped regarding the PDU session for URLLC service.

6.13.4.2	Analytics Procedure for E2E redundant transmission

Figure 6.13.4.2-1 shows the procedure how the NWDAF can be triggered to provide the Redundant Transmission analytics to the PCF for the UE and how the Redundant Transmission analytics can be utilized by the PCF to trigger the establishment of Redundant end to end user plane paths.

Figure 6.13.4.2-1: Redundant Transmission Experience analytics for E2E redundant transmission provided to an NF

1.	The PCF for the UE receives dynamic redundant transmission request from the AF, via the Service specific information provisioning procedure as defined in step 0 to step 5 in figure 4.15.6.7-1 of 23.502 [3]. The request message includes the target AF-Service-Identifier, target UE(s) identified by the user identity or External Group ID or any UE, dynamic redundant transmission request indication, and optionally SLA requirements for the application service.

2.	The PCF for the UE sends a request to the NWDAF for analytics on the target UE, using either the Nnwdaf_AnalyticsInfo or Nnwdaf_AnalyticsSubscription_Subscribe service. The type of analytics is set to Redundant Transmission Experience. Analytics Filter Information optionally contains DNN, S-NSSAI, PDU Session ID, Area of Interest, etc. The DNN, S-NSSAI and PDU Session ID can be determined by the PCF based on local configuration or URSP rules.

3.	If the request is authorized, the NWDAF, in order to provide the requested analytics, may collect the service data information from OAM and/or the SMF/UPF.

NOTE 1: How the UE is triggered to initiate redundant transmission based on updated URSP rules is up to UE implementation.

4.	The NWDAF derives requested analytics.

5.	The NWDAF provides requested Redundant Transmission Experience analytics to the PCF for the UE, using either the Nnwdaf_AnalyticsInfo_Request response or Nnwdaf_AnalyticsSubscription_Notify, depending on the service used in step 1.

6.	Based on the received analytics result, the PCF for the UE may decide to update the URSP rules of the UE to activate E2E Redundant Transmission.

7.	The PCF for the UE provides the updated URSP rules to the UE via UE Configuration Update procedure as specified in clause 4.2.4.3 of TS 23.502 [3].

8.	The application on the UE initiates redundant transmission based on UE's re-evaluation of the updated URSP rules.

NOTE 2: How the UE is triggered to initiate redundant transmission based on updated URSP rules is up to UE implementation.

9.	The UE initiates Redundant PDU Session Establishment/Modification procedure to establish E2E Redundant user plane paths for URLLC services:

-	If the PDU Session parameters (e.g. DNN, S-NSSAI, SSC mode) of the existing PDU Session for URLLC service are not updated and the corresponding URSP rule contains RSN and/or PDU Session Pair ID, the UE initiates PDU Session Modification procedure by indicating the RSN/PDU Session Pair ID to the SMF, and initiates another Redundant PDU Session Establishment procedure by indicating the corresponding RSN/PDU Session Pair ID to the SMF.

-	If PDU Session parameters of the existing PDU Session for URLLC service are updated, the UE initiates the PDU Session Establishment procedure for both Redundant user planes, and may provide the corresponding RSN/PDU Session Pair ID, if available in the URSP, to the SMF.

# * * * * End of changes * * * *