3GPP TSG-SA WG2 Meeting #154	S2- 2210716

# Toulouse, France, Nov 14 – 18, 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 3641 | rev | - | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | A New Service for Selection of Common DNAI |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NTT DOCOMO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | EDGE_Ph2 |  |  |  |  |  | Date: |  |  | 2022-10-31 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | As concluded in TR 23.700-48, the multiple SMF case should be supported, and the common DNAI selection should be independent of one single PDU session. For that purpose, a new service for a common DNAI selection is proposed, which is central for all PDU Sessions, i.e. can decide the DNAI for any PDU session in the collection. The service gets all necessary data as input arguments from the SMFs that use this service. |  |  |  |  |  |  |  |  |
| Summary of change: |  | A new SMF service is defined to determine a common DNAI by updating Clauses 4.3.6.2, 4.3.6.x (new), 5.2.8.1 and 5.2.8.x (new) for any type of groups. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | If the proposed changes are not made, the common DNAI selection depends on one single session. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.6.2, 4.3.6.x (new), 5.2.8.1, 5.2.8.x (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*** Start change ***

#### 4.3.6.2	Processing AF requests to influence traffic routing for Sessions not identified by an UE address

Figure 4.3.6.2-1: Processing AF requests to influence traffic routing for Sessions not identified by an UE address

NOTE 1:	The 5GC functions used in this scenario are assumed to all belong to the same PLMN (HPLMN in non-roaming case or VPLMN in the case of a PDU Session in LBO mode).

NOTE 2:	Nnef_TrafficInfluence_Create or Nnef_TrafficInfluence_Update or Nnef_TrafficInfluence_Delete service operations invoked from an AF located in the HPLMN for local breakout and home routed roaming scenarios are not supported.

1.	To create a new request, the AF invokes a Nnef_TrafficInfluence_Create service operation. The content of this service operation (AF request) is defined in clause 5.2.6.7. The request contains also an AF Transaction Id. If it subscribes to events related with PDU Sessions the AF indicates also where it desires to receive the corresponding notifications (AF notification reporting information).

To update or remove an existing request, the AF invokes a Nnef_TrafficInfluence_Update or Nnef_TrafficInfluence_Delete service operation providing the corresponding AF Transaction Id.

The Nnef_TrafficInfluence_Create (initiated by target AF) or Nnef_TrafficInfluence_Update (initiated by source AF or target AF) service operation may be used for the case of AF instance change. If Nnef_TrafficInfluence_Update service operation is invoked, the NEF is required to update the subscription resource. The Nnef_TrafficInfluence_Update service operation may include an updated notification target address. The updated subscription resource is used by the target AF.

NOTE 3:	If the source AF transfers the application context to the target AF, then target AF may create new subscription via Nnef_TrafficInfluence_Create operation or update existing subscription via Nnef_TrafficInfluence_Update. However, whether and how the application context transfer is done is out of this specification.

2.	The AF sends its request to the NEF. If the request is sent directly from the AF to the PCF, the AF reaches the PCF selected for the existing PDU Session by configuration or by invoking Nbsf_management_Discovery service.

The NEF ensures the necessary authorization control, including throttling of AF requests and, as described in clause 4.3.6.1, mapping from the information provided by the AF into information needed by the 5GC.

3.	(in the case of Nnef_TrafficInfluence_Create or Update): The NEF stores the AF request information in the UDR (Data Set = Application Data; Data Subset = AF traffic influence request information, Data Key = AF Transaction Internal ID, S-NSSAI and DNN and/or Internal Group Identifier or SUPI).

NOTE 4:	Both the AF Transaction Internal ID and, S-NSSAI and DNN and/or Internal Group Identifier or SUPI are regarded as Data Key when the AF request information are stored into the UDR, see Table 5.2.12.2.1-1.

(in the case of Nnef_TrafficInfluence_delete): The NEF deletes the AF requirements in the UDR (Data Set = Application Data; Data Subset = AF traffic influence request information, Data Key = AF Transaction Internal ID).

The NEF responds to the AF.

4.	The PCF(s) that have subscribed to modifications of AF requests (Data Set = Application Data; Data Subset = AF traffic influence request information, Data Key = S-NSSAI and DNN and/or Internal Group Identifier or SUPI) receive(s) a Nudr_DM_Notify notification of data change from the UDR.

5.	The PCF determines if existing PDU Sessions are potentially impacted by the AF request. For each of these PDU Sessions, the PCF updates the SMF with corresponding new policy information about the PDU Session by invoking Npcf_SMPolicyControl_UpdateNotify service operation as described in steps 5 and 6 in clause 4.16.5.

If the AF request includes a notification reporting request for UP path change, the PCF includes in the PCC rule(s) the information required for reporting the event, including the Notification Target Address pointing to the NEF or AF and the Notification Correlation ID containing the AF Transaction Internal ID.

The PCF may, optionally, use service experience analytics per UP path, as defined in clause 6.4.3 of TS 23.288 [50], to provide an updated list of DNAI(s) to the SMF.

6.	When the updated policy information about the PDU Session is received from the PCF, the SMF may take appropriate actions to reconfigure the User plane of the PDU Session. The SMF may consider service experience analytics and/or DN Performance analytics per UP path (i.e. including UPF and/or DNAI and/or AS instance) as defined in clauses 6.4.3 and 6.14.3, respectively, of TS 23.288 [50] before taking such actions. Examples of actions are:

-	Determining a target DNAI.

- 	Determining if a common DNAI needs to be used as a target DNAI. In this case the SMF determines the common DNAI as described in clause 4.3.6.x.

-	Based on the target DNAI, adding, replacing or removing a UPF in the data path to e.g. act as an UL CL or a Branching Point e.g. as described in clause 4.3.5.

-	Allocate a new Prefix to the UE (when IPv6 multi-Homing applies).

-	Updating the UPF in the target DNAI with new traffic steering rules.

-	Subscribe to notifications from the AMF for an Area of Interest via Namf_EventExposure_Subscribe service operation.

-	Determining whether to relocate PSA UPF considering the user plane latency requirements provided by the AF (see clause 6.3.6 of TS 23.548 [74]).

When the updated policy information about the PDU Session is received from the PCF, the SMF may take appropriate actions to assist the EAS discovery and re-discovery for PDU Session with Session Breakout connectivity model such as:

-	Retrieve the EAS deployment information as defined in clause 6.2.3.4.1 of TS 23.548 [74].

-	Providing DNS message handling rule to forward DNS messages of the UE and/or report when detecting DNS messages as defined in clause 6.2.3.2.2 of TS 23.548 [74].

7.	The SMF may decide whether it is required to send the target DNAI to the AMF for triggering SMF/I-SMF (re)selection and then inform the target DNAI information for the current PDU session or for the next PDU session to AMF via Nsmf_PDUSession_SMContextStatusNotify service operation.

*** Second change ***

#### 4.3.6.x	Selection of the common DNAI

Figure 4.3.6.x-1 describes a procedure for selection of common DNAI.

Figure 4.3.6.x-1: Selection of the common DNAI

1.	SMF receives the user subscription data and PCC Rules as described in step 5 in Figure 4.3.6.2-1. Based on the "Indication of traffic correlation" and "influence-id" associated with a list of DNAIs in the PCC Rules, SMF knows that it needs to determine a common DNAI for the "influence-id". The SMF uses NRF to discover a service instance for Nsmf_CommonDNAI service for a selection of a common DNAI for the given influence-id. The SMF discovers one service instance for each "influence-id" associated with a list of DNAIs in the PCC Rules.

2.	The SMF invokes the Nsmf_CommonDNAI service and indicates the "influence-id", SUPI, and a list of DNAIs for the given "influence-id" as received in the PCC Rules.

3.	For the selection of the common DNAI for a collection of PDU Sessions associated with the same "influence-id", the Nsmf_CommonDNAI service may consider the UE locations, network topology, or the current Analytics from NWDAF for service experience between the gNB and candidate UPFs. The Nsmf_CommonDNAI service can subscribe for UE location info (UE mobility events) for the SUPI from the AMF.

4.	Nsmf_CommonDNAI service determines the common DNAI per each "influence-id" and notifies the SMF(s) serving the PDU Sessions for the result. Based on a trigger, e.g. UE mobility or analytics update, the Nsmf_CommonDNAI service can reselect a new common DNAI, and notify the SMF(s) serving the PDU Sessions for the change of the common DNAI.

5.	The SMF(s) serving the PDU Sessions use the common DNAI as a target DNAI for the traffic as indicated by the PCC Rule for the given "influence-id". The SMF(s) may take the necessary actions as described in step 6 in Figure 4.3.6.2-1 based on the target DNAI.

6.	Whenever a new PDU Session is established that matches with the given "influence-id", the steps 2-5 are performed for the new PDU Session.

If the Nsmf_CommonDNAI service determines a new common DNAI for the "influence-id", the other SMF(s) of the existing PDU Sessions are notified by Nsmf_ CommonDNAI_Notify service operation for a new common DNAI. This may be done e.g. due to an establishment of a new PDU Session, or one of the existing PDU Sessions are released, or one or more of the UEs are moving.

*** Third change ***

#### 5.2.8.1	General

The following table shows the SMF Services and SMF Service Operations.

Table 5.2.8.1-1: NF services provided by the SMF

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Nsmf_PDUSession | Create | Request/Response | V-SMF/I-SMF |
|  | Update | Request/Response | V-SMF/I-SMF, H-SMF |
|  | Release | Request/Response | V-SMF/I-SMF |
|  | CreateSMContext | Request/Response | AMF |
|  | UpdateSMContext | Request/Response | AMF |
|  | ReleaseSMContext | Request/Response | AMF |
|  | SMContextStatusNotify | Subscribe/Notify | AMF |
|  | StatusNotify | Subscribe/Notify | V-SMF/I-SMF |
|  | ContextRequest | Request/Response | AMF, V-SMF/I-SMF, SMF |
|  | ContextPush | Request/Response | SMF |
|  | SendMOData | Request/Response | AMF |
|  | TransferMOData | Request/Response | V-SMF/I-SMF |
|  | TransferMTData | Request/Response | SMF, H-SMF |
| Nsmf_EventExposure | Subscribe | Subscribe/Notify | NEF, AMF, NWDAF |
|  | Unsubscribe |  | NEF, AMF, NWDAF |
|  | Notify |  | NEF, AMF, NWDAF |
|  | AppRelocationInfo |  | AF |
| Nsmf_NIDD | Delivery | Request/Response | NEF |
| Nsmf_CommonDNAI | Subscribe | Subscribe/Notify | SMF |
|  | Unsubscribe |  | SMF |
|  | Notify |  | SMF |

*** Fourth change (new)***

##### 5.2.8.x	Nsmf_ CommonDNAI service

##### 5.2.8.x.1	General

Service Description: This service is used to determine a common DNAI for a collection of UEs. The SMF that serves the PDU session in the UE collection discovers the service instance of “Nsmf_ CommonDNAI” from NRF. The SMF passes the necessary input parameters (e.g., SUPI, influence-id, notification correlation ID) in the Subscribe request.

The service returns a common DNAI for the UE collection identified by the "influence-id". The SMF that serves the PDU session uses the common DNAI for the PDU session.

Any SMF that serves the PDU session in the UE collection invokes the Nsmf_ CommonDNAI service, with the same influence-id.

The service may determine a new common DNAI, e.g. when the service is invoked for a new PDU Session in the UE collection; the service notifies the SMFs in the UE collection for the new common DNAI. The SMFs that serve the PDU sessions use the new DNAI for their PDU sessions. This may result to EAS re-discovery procedures in the SMFs and in the UEs.

##### 5.2.8.x.2	 Nsmf_ CommonDNAI_Subscribe service

Service operation name: Nsmf_CommonDNAI _Subscribe

Description: subscription for the notifications for the change of the common DNAI event.

Inputs, Required: SUPI, influence-id, list of candidate DNAIs, Notification Target Address (+ Notification Correlation ID).

Inputs, Optional: Subscription Correlation ID (in the case of modification of the event subscription).

Outputs, Required: When the subscription is accepted: Subscription Correlation ID.

Outputs, Optional: None.

##### 5.2.8.x.3	 Nsmf_ CommonDNAI_Unsubscribe service

Service operation name: Nsmf_CommonDNAI _Unsubscribe

Description: Enable NF consumers to unsubscribe the notification of events related to Nsmf_CommonDNAI _Subscribe operation.

Inputs, Required: Subscription Correlation ID.

Inputs, Optional: None.

Outputs, Required: Success or Failure.

Outputs, Optional: None.

##### 5.2.8.x.4	 Nsmf_ CommonDNAI_Notify service

Service operation name: Nsmf_CommonDNAI _Notify

Description: notification for the change of the common DNAI event.

Inputs, Required: Common DNAI, influence-id.

Inputs, Optional: None.

*** End of change ***