SA WG2 Meeting #154	S2-2210806

Toulouse, 14 - 18 November, 2022

Source:	NEC

Title:	KI#6: Update of conclusion for KI#6

Document for:	Approval

Agenda Item: 	9.4

Work Item / Release:	FS_eNPN_Ph2 / Rel-18

Abstract of the contribution: This paper proposes to update the conclusion for KI#6 based on the evaluation criteria in Section 7.6.

# Discussion

In the SA2#153e meeting, the evaluation criteria for the solutions for KI#6 was discussed and agreed. The evaluation criteria are:

- To mitigate overload at home network when a high number of UEs attempt to return from a local hosting networks in a short period of time,

- To reduce unnecessary waiting times of UEs which are to re-register to their home network,

- To utilize the available resources in the home network efficiently, and

- To allow UEs to leave the localized hosting network on their own at any time to their home network.

This contribution proposes to update the conclusion of KI #6 based on the evaluation criteria captured in Section 7.6.

# Proposal

It is proposed to capture the following changes in TR 23.700-08.

* * * * Start of Changes * * * *

## 8.6	Key Issue #6: Support for returning to home network

The following principles from the evaluation are recommended to be considered during the normative work:

-	The home/serving network can re-use existing mechanisms for Control Plane Load Control, Congestion and Overload Control described in clause 5.19 of TS 23.501 [3] when load level reaches a certain threshold and overload control mechanism are triggered.

-	The home network can negotiate/communicate with a localized hosting network to i) mitigate an overload at the home network in an adaptive manner, ii) utilize the available home network AMF resources efficiently, iii) reduce the overall waiting time for UEs to re-register to their home network, and iv) allow UEs to leave the hosting network on their own at any time.

-	Additional mechanisms can be implemented to ensure spreading of return of the UEs to home network without any normative impacts to 3GPP specifications.

NOTE:	Means to describe how to spread returning UEs can be captured as informative description, during normative specification work.

* * * * End of Changes * * * *