3GPP TSG-SA WG2 Meeting #154	S2-2210866

# Nov 14 – 18, 2022, Toulouse

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.273 | CR | 0260 | rev | - | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | LCS Continuity During UE Mobility |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |  |
| Work item code: | eLCS_Ph3 |  |  |  |  |  |  | Date: |  | 2022-10-14 |  |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-18 |  |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |  |  |
| Reason for change: |  | - The LCS continuity support during UE mobility feature in TS 23.700-71 has been concluded. This paper introduces the feature into the TS 23.273. |  |  |  |  |  |  |  |  |  |
| Summary of change: |  | The basic framework for the different mobility scenarios is introduced. |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | LCS continuity during UE mobility feature in the study is not merged into the TS 23.273. |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |  |

* * * * Start of Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 22.071: "Technical Specification Group Systems Aspects; Location Services (LCS)".

[3]	3GPP TS 22.261: "Service requirements for the 5G system; Stage 1".

[4]	3GPP TS 23.271: "Functional stage 2 description of Location Services (LCS)".

[5]	3GPP TS 43.059: "Functional Stage 2 description of Location Services in GERAN".

[6]	Void.

[7]	3GPP TS 36.305: "Stage 2 functional specification of User Equipment (UE) positioning in E-UTRAN".

[8]	3GPP TS 23.032: "Universal Geographical Area Description (GAD)".

[9]	3GPP TS 38.305: "Stage 2 functional specification of User Equipment (UE) positioning in NG-RAN".

[10]	3GPP TS 23.167: "IP Multimedia Subsystem (IMS) emergency sessions".

[11]	3GPP TS 24.501: "Non-Access-Stratum (NAS) protocol for 5G System (5GS); Stage 3".

[12]	3GPP TS 29.572: "5G System; Location Management Services; Stage 3".

[13]	OMA MLP TS: "Mobile Location Protocol", [http://www.openmobilealliance.org].

[14]	Void.

[15]	3GPP TS 38.455: "NG-RAN; NR Positioning Protocol A (NRPPa)".

[16]	3GPP TS 29.518: "5G System; Access and Mobility Management Services; Stage 3".

[17]	3GPP TS 25.305: "Stage 2 functional specification of User Equipment (UE) positioning in UTRAN".

[18]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[19]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[20]	3GPP TS 37.355: "LTE Positioning Protocol (LPP)".

[21]	3GPP TS 23.316: "Wireless and wireline convergence access support for the 5G System (5GS)".

[22]	Void.

[23]	ITU-T Recommendation E.164: "The international public telecommunication numbering plan".

[24]	3GPP TS 23.222: "Common Application Programming Interface (API) framework for 3GPP northbound APIs".

[25]	RFC 2396: "Uniform Resource Identifiers".

[26]	RFC 3261: "SIP: Session Initiation Protocol".

[27]	3GPP TS 23.228: "IP multimedia subsystem (IMS)".

[28]	3GPP TS 23.003: "Numbering, addressing and identification".

[29]	3GPP TS 29.002: "Mobile Application Part (MAP) specification".

[30]	3GPP TS 32.271: "Telecommunication management; Charging management; Location Services (LCS) charging".

[31]	3GPP TS 32.298: "Telecommunication management; Charging management; Charging Data Record (CDR) parameter description".

[32]	Void.

[33]	3GPP TS 29.571: "5G System; Common Data Types for Service Based Interfaces; Stage 3".

[34]	Void.

[35]	3GPP TS 29.122: "T8 reference point for Northbound APIs".

[36]	3GPP TS 24.571: "5G System (5GS); Control plane Location Services (LCS) procedures; Stage 3".

[37]	OMA-TS-ULP-V2_0_6: "UserPlane Location Protocol Approved Version 2.0.6".

* * * * Next Change * * * *

### 4.3.3	Gateway Mobile Location Centre, GMLC

The Gateway Mobile Location Centre (GMLC) contains functionality required to support LCS. In one PLMN, there may be more than one GMLC.

A GMLC is the first node an external LCS client accesses in a PLMN (i.e. the Le reference point is supported by the GMLC). AFs and NFs may access GMLC directly or via NEF. The GMLC may request routing information and/or target UE privacy information from the UDM via the Nudm interface. After performing authorization of an external LCS Client or AF and verifying target UE privacy, a GMLC forwards a location request to either a serving AMF using Namf interface or to a GMLC in another PLMN using the Ngmlc interface in the case of a roaming UE.

The target UE's privacy profile settings shall always be checked in the UE's home PLMN prior to delivering a location estimate.

The "Visited GMLC" (VGMLC) is the GMLC, which is associated with the serving node of the target UE.

The "Home GMLC" (HGMLC) is the GMLC residing in the target UE's home PLMN, which is responsible for the control of privacy checking of the target UE.

Additional functions which may be performed by a GMLC to support location services include the following.

-	At an HGMLC, determine the serving AMF for a target UE when there is more than one serving AMF.

-	At an HGMLC, determine whether to attempt a second location request for a target UE from a different AMF when location information returned by a first AMF does not meet QoS requirements and there is more than one serving AMF.

-	At an HGMLC, support location requests from an external LCS client or NEF for a 5GC-MT-LR and deferred 5GC-MT-LR for periodic, triggered and UE available location events.

-	At an HGMLC, forward location requests for a roaming UE to a VGMLC or serving AMF in the VPLMN based on deployment configurations.

-	At an HGMLC, receive event reports from a VGMLC or LMF for a deferred 5GC-MT-LR for periodic or triggered location and return to an external LCS Client or NEF.

-	At an HGMLC, support cancelation of a periodic or triggered location.

-	At an HGMLC, receive location information from an VGMLC for a 5GC-MO-LR and forwards to an LCS Client or an AF (via NEF) if requested by the UE.

-	At a VGMLC, receive location requests from an HGMLC for a roaming UE and forward to a serving AMF.

-	At a VGMLC, receive event reports from an LMF for a deferred 5GC-MT-LR for periodic or triggered location for a roaming UE and forward to an HGMLC.

-	At a VGMLC, receive location information from an AMF for a 5GC-MO-LR and forwards to an HGMLC.

-	At an HGMLC, reject the LCS request coming from a LCS client, e.g. when the number of Target UEs in the LCS request exceeds the Maximum Target UE Number of such client.

-	At an HGMLC, allocate the reference number for each location request from an external LCS client for LDR.

-	At an HGMLC, assign the pseudonym if pseudonym indicator is received in the service request and transfer it to external LCS client, e.g. when core network provides the UE's location information to LCS client. Resolve the verinym from the pseudonym, if it is received from the LCS client.

-	At an HGMLC, resolve group identifier to identifier of individual UEs and aggregate responses to LCS Client or NEF during bulk operation procedure.

-  During UE mobility between 5GS and EPS, the GMLC shall transfer the LCS request from the source to target RAT, map the LCS QoS and initiate the LCS session on the target RAT from 5GS to EPS

* * * * Next Change * * * *

### 4.3.7	Access and Mobility Management Function, AMF

The AMF contains functionality responsible for managing positioning for a target UE for all types of location request. The AMF is accessible to the GMLC and NEF via the Namf interface, to the RAN via the N2 reference point and to the UE via the N1 reference point.

Functions which may be performed by an AMF to support location services include the following.

-	Initiate an NI-LR location request for a UE with an IMS emergency call or to know a UE geographical area with NR satellite access for PLMN selection verification.

-	Receive and manage location requests from a GMLC for a 5GC-MT-LR and deferred 5GC-MT-LR for periodic, triggered and UE available location events.

-	Receive and manage location requests from a UE for a 5GC-MO-LR.

-	Receive and manage Event Exposure request for location information from an NEF.

-	Select an LMF.

-	Receive updated privacy requirements from a UE and transfer to a UDR via UDM.

-	Support cancelation of periodic or triggered location reporting for a target UE.

-	Support change of a serving LMF for periodic or triggered location reporting for a target UE.

-	When assistance data is broadcast by 5GS in ciphered form, the AMF receives ciphering keys from the LMF and forwards to suitably subscribed UEs using mobility management procedures.

-	Store UE Positioning Capability received from an LMF and send the UE Positioning Capability along with the received location request to an LMF.

NOTE:	Details of UE Positioning Capability is defined in TS 37.355 [20].

- Provide the target MME ID to GMLC as part of Namf_Location_Response and sending the LCS user plane server assistance data to the UE during 5GS to EPS handover to enable LCS session continuity

* * * * Next Change * * * *

## 6.X	LCS Continuity During UE Mobility

### 6.X.1 Inter-RAT Mobility

#### 6.X.1.1 5GS to EPS Mobility

Figure 6.33.3-1 : LCS Continuity Solution for 5GS to EPS Mobility

1-4. This is the normal MT-LR LCS session on 5GS (same as steps 1.a. to 12 in Figure 6.1.2-1)

5-14. This is the normal 5GS to EPS HO procedure (same as steps 1 to 12.b in Figure 4.11.1.2.1-1 in TS 23.502 [19])

15. AMF requests the LMF to cancel the LCS session on 5GS.

16. LMF cancels the LCS session with the UE and NG-RAN

17. LMF responds to the 5GC-GMLC with Namf_Location_ProvidePositioningInfo Resp indicating failure of the LCS session with cause = HO to EPC while also including the target MME ID for the UE

NOTE-1 : The LCS session cancellation with the UE and NG-RAN is to be completed before the UE is handed over to LTE. To enable this, the AMF could initiate steps 11, 15 and 17 in parallel.

8. The 5GC-GMLCforwards the location request to the EPC-GMLC including all the parameters of the LCS Request and the target MME ID.

19-24. The EPC-GMLC  forwards the LCS Request to the Target MME received. The LCS session is started in EPS (same as steps 1-10 in Figure 9.18 in TS 23.271 [4]).

NOTE-2 : EPC-GMLC could do a mapping of LCS QoS (from the 5GS defined values to EPS defined one). For QoS attributes that are common between 5GS and EPS, the original LCS QoS defined values can be used as is in EPS. For QoS attributes that are specific to 5GS, the QoS mapping could be done to the most stringent QoS value with best effort handling in QoS Class.

25-26.  The UE location estimate is returned by the EPC-GMLC to the 5GC-GMLC which then forwards the same to the LCS Client.

NOTE-3:  If an LCS session was active over user plane in 5GS when the handover is triggered, the AMF/LMF share assistance data to the UE that minimizes the TLS handshake workload according to clause 6.1.1.4. of [37], once the UE moves to EPS.

#### 6.X.1.2 EPS to 5GS Mobility

Figure 6.33.3-2: LCS Continuity Solution for EPS to 5GS Mobility

1-4. This is the normal MT-LR LCS session on EPS (same as steps 1 to 3 in Figure 6.1.2-1 in TS 23.271 [4].

5-14 This is the normal EPS to 5GS HO procedure (similar to steps 1 to 16 in Figure 4.11.1.2.2.2-1 and steps 1-4 in Figure 4.11.1.2.2.3-1 in TS 23.502 [19]).

15. The MME initiates the cancellation of the LCS session over EPS

16. e-SMLC cancels the LCS session with the UE and E-UTRAN

17. The MME indicates failure to the EPC-GMLC with reason "HO to 5GC" and including the target AMF ID.

NOTE-1:  The MME could initiate steps 15 and 17 in parallel to step 11 to ensure that the LCS session cancellation happens before the handover completes while the UE is in EPS.

18.  The EPC-GMLC forwards the location request to the 5GC-GMLC including all the parameters of the LCS Request and the target AMF ID.

19-24. The LCS Session is started on 5GS (similar to steps 1-23 in Figure 6.1.2-1).

NOTE-2:  There could be a mapping of LCS QoS (from the values defined for EPS to values defined in 5GS - this could be done by the GMLC.  For QoS attributes that are common between 5GS and EPS, the original LCS QoS defined values can be used as is in 5GS.  For QoS attributes that are specific to EPS, the QoS mapping could be done to the most stringent QoS value with best effort handling in QoS Class.

25-26. The UE location estimate is returned by the 5GC-GMLC to the EPC-GMLC which then forwards the same to the LCS Client.

NOTE-3:  If the LMF is configured to use LCS over user plane in 5GS, then LMF will choose to initiate the LCS session over user plane at step 24.

### 6.X.2 Mobility between NG-RAN nodes

#### 6.X.2.1 Mobility in RRC-Connected

##### 6.X.2.1.1 Xn based Handover

### The high-level approach is as per the conclusion for key issue #8 in TR 23.700-71.

### Source AMF shares LCS session details to T-AMF as part of HO preparation.

### Target AMF notifies to LMF after HO completion.

### LMF continues with the LCS Session with new configuration

### LMF responds with UE location to the T-AMF which responds to the LCS Client via the GMLC(s).

Editor’s Note : Details are FFS

##### 6.X.2.1.2 NG based HO

### - The high-level approach is as per the conclusion for key issue #8 in TR 23.700-71Source AMF shares LCS session details to T-AMF as part of HO preparation.

### - Target AMF notifies to LMF after HO completion.

### - LMF continues with the LCS Session with new configuration

### - LMF responds with UE location to the T-AMF which responds to the LCS Client via the GMLC(s).

### Editor’s Note : Details are FFS

#### 6.X.2.2 Mobility in RRC-INACTIVE

The current solution defined clauses 6.7.3, 6.7.4, 6.7.5 shall be used.

#### 6.X.2.3 Mobility in RRC-Idle

### The high-level approach is as per the conclusion for key issue #8 in TR 23.700-71.

### Source AMF shares LCS session details to T-AMF as part of HO preparation.

### Target AMF notifies to LMF after HO completion.

### LMF continues with the LCS Session with new configuration

### LMF responds with UE location to the T-AMF which responds to the LCS Client via the GMLC(s).

Editor’s Note : Details are FFS

* * * * End of Changes * * * *