3GPP TSG-WG SA2 Meeting #154  	S2-2210349

Toulouse, France, November 14 – 18, 2022	(revision of S2-220xxxx)

Title:	[DRAFT] LS Reply on UE Policy Control with PCF re-selection during AMF relocation

Response to:	LS (S2-2208132/C3-224697) on UE Policy Control with PCF re-selection during AMF relocation rom CT3

Release:	Release 17

Work Item:	5GS_Ph1-CT

Source:	[Huawei to be] SA2

To:	CT3, CT1

Cc:

Contact Person:

Name:	Haiyang, Sun

Tel. Number:

E-mail Address:	sunhaiyang3 AT huawei DOT com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	 None

1. Overall Description:

Thanks CT3 for the questions on UE Policy Control with PCF re-selection during AMF. Please find SA2’s answers as following:

Question 1: At the reception of UE Policy Association creation request, when no indication about the list of stored PSIs, ANDSP support, and UE OSId is received, shall the PCF interpret that the UE is triggering initial registration and the UE does not have UE Policies stored? If not, how does the PCF determine it is a different scenario?

SA2 Answer 1: As described in clause 6.1.2.2.2 of TS 23.503, “When the PCF (i.e. the (H-)PCF as well as the V-PCF) receives a list of PSIs associated to the PLMN of the PCF from the UE, the PCF compares the list of PSIs provided by the UE and the list of PSIs retrieved from the UDR. In addition, the PCF checks whether the list of PSIs provided by the UE or its content needs to be updated according to operator policies, e.g. change of Location and/or time.” So the PCF does not need to determine it is a different scenario from stage 2 perspective. Besides, it is noticed in CT1 there is an ongoing discussion on PSI report, it is assumed CT1 can provide related information.

Question 2: Additionally, since the new selected PCF does not receive information about the UE's support for ANDSP or UE’s support of OSIds, which affect to the proper determination of the ANDSP,

how does the ANDSP policies procedure work in this scenario (considering also roaming and V-PCF determination of ANDSP)?

SA2 Answer 2: As described in clause 6.1.2.2.2 of TS 23.503, The (H-)PCF maintains UE's support for ANDSP or UE’s support of OSIds and stores them in the (H-)UDR using the Nudr_DM_Update including DataSet "Policy Data" and Data Subset "Policy Set Entry". The new selected PCF can retrieve them from the UDR by using Data Key described in clause 5.2.12.2.1 of TS 23.502 in both roaming and non-roaming scenario. However, in roaming case, V-PCF does not need to be aware of UE’s support of OSIds as V-PCF does not need to determine URSP.  Thus, UE’s support of OSIds does not need to be stored in V-UDR.

2. Actions:

To CT3 group.

ACTION: 	SA2 asks CT3 group to take the above information into account and update their specifications if necessary.

3. Date of Next TSG SA WG2 Meetings:

TSG-SA2 Meeting #154 Ad Hoc	Jan 16 – 20, 2023				eMeeting

TSG-SA2 Meeting #155		Feb 20 – 24, 2023				Athens, GR