3GPP SA WG2 Meeting #154	S2-2210440

14 - 18 November, 2022, Toulouse, France

Source: 	Samsung

Title: 	KI#4: Update to conclusions related to PVS address in Localized service information

Document for: 	Approval

Agenda Item: 	9.4.1

Work Item / Release:	FS_eNPN_Ph2 / Rel-18

Abstract of the contribution: This paper provides discussions and update to include PVS address related information in the Localized service information.

# Discussion

We address the following EN in the KI#4 conclusions and propose changes to the clause 8.4.2.

Editor's note 2-1 a:	It is FFS whether a PVS address / captive portal information need to be provided as an optional information to the UE. In the Localized service information. If not How will UE be provided this address is FFS.

For the scenarios where business relationship does not exist between Hosting Network (HOST-NO) and UE’s Home network (HOME-NO) (Business Models II and IV in Ericsson’s evaluation update on Business Models), following points can be observed:

- UE cannot use Home network credentials for accessing the Hosting Network (which is an SNPN)

- UE may be provided credentials by the LSP (Localized Service Provider).

- UE may use user plane of the serving network and performs remote provisioning to obtain SNPN subscription/credential, or UE may disconnect from the serving network and performs SNPN onboarding procedure to obtain SNPN subscription/credentials.

Thus in order to perform remote provisioning for obtaining credentials for accessing Hosting Network for Localized services, UE need to know the PVS address(es) related to the Localized Service which UE want to receive. UE may use the PDU Session of it’s serving network to be provisioned with the required credentials.

Thus we propose to remove the said EN from the KI#4 conclusion and include the bullet related to PVS address in Localized Service Information.

# 2	Proposal

It is proposed to agree the following changes in TR 23.700-08.

*** BEGIN CHANGES ***

# 8	Conclusions

### 8.4.2	Conclusion for the content of the localized service information

The following interim conclusions are reached. Final conclusions for normative work are expected for SA2#154 i.e. whether to proceed with Alt.1 and/or Alt.2.

Alt.1.	The localized service information is provided to UE for UE to discover and select hosting network to receive desired localized service. The content of the localized service information can include the following elements:

a.	Identifier/name of the localized service.

b.	Validity conditions for the localized service, e.g. duration of remaining service operation ,time and location.

c.	PVS information (PVS IP address(es) and/or PVS FQDN(s)) (optional)

d.	Hosting network related information, per hosting network in the case of SNPN as hosting network:

i.	hosting network identifier, e.g. SNPN ID, GIN;

ii.	Validity condition, e.g. time and/or location (optional)

Editor's note 2-1b:	It is FFS whether Validity condition is to be possible to be set per localized service, hosting network or both.

e.	Hosting network related information, per hosting network in the case of PNI-NPN as hosting network:

i.	PLMN ID of the PNI-NPN.

ii.	Optionally, if hosting network is associated with CAG IDs, a list of CAG IDs corresponding to the localized service.

Editor's note 2-2:	It is FFS whether it is beneficial to provide CAG information within the localized service information, given that there is existing procedure to update UE with CAG information, and whether CAG ID is used to identify hosting network. It is also FFS whether CAG IDs needs to be associated with localized service when sent to the UE.

Editor's note 2-3:	It is FFS whether more hosting network related information can be sent as to better enable a user selection of hosting network, e.g. balance/service rate information to access the hosting network for the localized service, quality of the service.

Alt.2.	The UE is provided with:

a.	a list of prioritized hosting networks (i.e. SNPN ID or GIN) for localized services, in the case of SNPN as hosting network.

b.	allowed CAG ID list, in the case of PNI-NPN as hosting network and the PNI-NPN is associated with CAG ID.

NOTE:	The home network / hosting network map localized services to hosting network ID (e.g. SNPN ID, GIN, CAG ID).

Editor's note 2-4:	It is FFS whether the UE receives list a/list b per localized service ID, or the lists are used for localized services in general.

Editor's note 2-5:	It is FFS whether necessary to associate validity conditions with the list in a and b above, and what are the conditions, e.g. time and/or location.

Editor's note 2-6:	It is FFS what localized service information needs to be provided to UE in b above, in the case PNI-NPN as hosting network does not support CAG.

Editor's note 2-7:	Alt. 1 provides UE with necessary information as a first step, and then the UE based on the received information derives how to perform other activities in later steps, e.g. network selection, credential determination and provisioning, etc. Alt.2 provides UE with information directly used for network selection, but requires that UE/network knows beforehand whether the desired localized service is provided by the hosting networks in the lists.

*** END CHANGES ***