3GPP TSG-WG SA2 Meeting #154	S2-2210621

# Toulouse, France, November 14 – 18, 2022		(revision of S2-221xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.304 | CR | 0149 | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | 5G ProSe Remote UE traffic handling for multipath transmission via UE-to-Network Relay |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_ProSe_Ph2 |  |  |  |  |  | Date: |  |  | 2022-11-04 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Based on the KI#5 conclusion in TR 23.700-33, to support multi-path transmission via direct Uu path and Layer 3 U2N Relay UE without N3IWF, the RSD is extended with additional components to indicate multipath transmission via Layer 3 U2N Relay UE without N3IWF and Uu. The KI#6 conclusion presents that backward compatibility with Rel-17 shall be considered when concluding this key issue for Rel-18 ProSe. In Rel-17 if "5G ProSe Layer-3 UE-to-Network Relay Offload indication" is present in a Route Selection Descriptor, no other components shall be included in the Route Selection Descriptor. The RSD is extended by adding a new Multi-Path ProSe Layer-3 UE-to-Network Relay Offload indication. This new indication is used by the UE to determine if the matching application should be routed via a PDU Session of this UE and a Layer-3 UE-to-Network Relay outside of a PDU Session. The ProSe Layer-3 UE-to-Network Relay Offload indication and Multi-Path ProSe Layer-3 UE-to-Network Relay Offload indication are separated indications and are not used together. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 5G ProSe Remote UE traffic handling for multipath transmission via UE-to-Network Relay is added. The existing ProSe Layer-3 UE-to-Network Relay Offload indication term is aligned with 23.503. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | 5G ProSe Remote UE traffic handling for multipath transmission via UE-to-Network Relay is not supportted. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.5.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

### 6.5.4	5G ProSe Remote UE traffic handling for 5G ProSe UE-to-Network Relay support

For the 5G ProSe Remote UE to access the service via 5G ProSe UE-to-Network Relay, the following apply:

-	The application traffic on the 5G ProSe Remote UE is managed by URSP rules (with consideration of local configurations), following the procedure defined in clauses 6.1.2.2.1 and 6.6.2.3 of TS 23.503 [9]. The URSP rules defined in clause 6.6.2.1 of TS 23.503 [9] applies for the 5G ProSe Remote UE, with RSD enhanced to include:

-	a "ProSe Layer-3 UE-to-Network Relay Offload indication".

-	a "Multi-Path ProSe Layer-3 UE-to-Network Relay Offload indication".

-	If an application or application traffic matches a URSP rule, corresponding RSDs shall be used to evaluate the existing PDU sessions, or establish a new PDU session, or determine to offload outside of a PDU session, or multipath transmission via 5G ProSe Layer-3 UE-to-Network Relay.

-	If the selected RSD contains "ProSe Layer-3 UE-to-Network Relay Offload indication":

-	The 5G ProSe Remote UE will route the traffic to the 5G ProSe Layer-3 UE-to-Network Relay connection without establishing a PDU session, when such connection is available.

This may trigger the 5G ProSe Remote UE to start 5G ProSe UE-to-Network Relay discovery if it is not yet started. The discovery and establishment of the connection with the 5G ProSe Layer-3 UE-to-Network Relay is controlled by the ProSe Policy (pre-) configured on the 5G ProSe Remote UE.

-	If the matched URSP rule contains both a RSD with ''Non-Seamless Offload indication" and a RSD with "ProSe Layer-3 UE-to-Network Relay Offload indication", whether to offload the traffic to non-3GPP access or the 5G ProSe Layer-3 UE-to-Network Relay connection depends on the priority of the RSDs, and the availability of the connections, as specified in the clause 6.6.2.3 of TS 23.503 [9].

-	If the selected RSD does not contain the "ProSe Layer-3 UE-to-Network Relay Offload indication" or, "Non-Seamless Offload indication", the 5G ProSe Remote UE shall use a PDU session to route the corresponding application traffic.

If configured in the ProSe Policy, the 5G ProSe Remote UE may attempt the discovery of a Relay Service Code corresponding to a 5G ProSe Layer-3 UE-to-Network Relay with N3IWF support in the discovery procedure, when the non-3GPP access type is preferred in the selected RSD. The 5G ProSe Remote UE may attempt the discovery of a Relay Service Code corresponding to a 5G ProSe Layer-2 UE-to-Network Relay in the discovery procedure, when the 3GPP access type is preferred in the selected RSD.

-	If the selected RSD contains "Multi-Path ProSe Layer-3 UE-to-Network Relay Offload indication":

-	The 5G ProSe Remote UE can route the traffic over an existing PC5 connection with 5G ProSe Layer-3 UE-to-Network Relay and existing PDU Session matched to other components (e.g., Network Slice Selection) in the selected RSD, when such connections are available.

-	If the PC5 connection with 5G ProSe Layer-3 UE-to-Network Relay is not available, this may trigger the 5G ProSe Remote UE to start 5G ProSe UE-to-Network Relay discovery and connection establishment, controlled by the ProSe Policy configured on the 5G ProSe Remote UE.

-	If the PDU Session matched to other components in the selected RSD is not available, this will trigger to establish a new PDU Session using the values specified by the selected RSD.

-	If the 5G ProSe Remote UE has an indirect connection via a 5G ProSe Layer-2 UE-to-Network Relay connection available, it will be treated as the "3GPP" access type. If the 5G ProSe Remote UE has an indirect connection via a 5G ProSe Layer-3 UE-to-Network Relay with N3IWF support available, it will be treated as the "non-3GPP" access type. The URSP handling as defined in TS 23.503 [9] applies.

For the 5G ProSe Layer-3 UE-to-Network Relay and 5G ProSe Layer-2 UE-to-Network Relay, the URSP handling does not apply to the relayed traffic from the 5G ProSe Remote UE.

For the 5G ProSe Layer-3 UE-to-Network Relay, the PDU session established for relaying the 5G ProSe Remote UE's traffic is controlled by the ProSe Policy.

# * * * * End of changes * * * *