3GPP TSG-WG SA2 Meeting #154 	S2-2210626

Toulouse, France, Nov 14 – 18, 2022	(revision of S2-220xxxx)

Source:	Huawei, HiSilicon

Title:	Update the conclusion for KI#4

Document for:	Approval

Agenda Item:	9.23

Work Item / Release:	FS_eNA_Ph3 / Rel-18

Abstract: This contribution proposes to update the conclusion for KI#4.

1. Introduction/Discussion

During the SA2#153E meeting, a FFS is left in the conclusion for KI#4 (S2-2208238):

Editor's note:	Whether the MTLF sends the ML model address or ML model to ADRF is FFS.

The concern behind this FFS is that, some companies believe it is possible for MTLF directly send the ML model to the ADRF by triggering ADRF’s ML model storage service operation and it is stage 3’s responsibility to define such ML model.

However, they has been persuaded by the clarification that it is not possible to define the ML model in SA2 and the future stage 3 (out of 3GPP), only the ML model address (such as FQDN, URL) can be defined and used in the ADRF’s ML model storage service operation.

So this FFS can be removed.

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-81.

# * * * * First change * * * *

8.4	Key Issue #4: How to Enhance Data collection and Storage

1.	For Managing Impact of storing data in NFp during muting, Solutions #41 and #45 are adopted for normative work.

2.	For ADRF / NWDAF Data Storage Management, a consumer may request data deletion alert and provide life time of the data to the ADRF. NWDAF or ADRF may be configured with operator policies for data storage as defined in Solution#46.

3.	Data records can be stored at ADRF with an associated DataSetTag. The DataSetTag can be used to retrieve the whole set of data records associated to such tag. The DataSetTag may be included in the service requests to AnLF and MTLF to indicate the data set to be used by the AnLF, respectively by MTLF.

4.	Data can be stored at and retrieved from ADRF using techniques of data compression and/or data synthesis. The DSC indicator is used to represent the technique of data compression and/or data synthesis. The value of DSC is up to vendor agreement and up to implementation. How to support data transportation using the techniques of data compression and/or data synthesis is out of the scope of 3gpp.

NOTE 1:	How DSC is conveyed in the SBI message is up to Stage-3 design.

5.	To optimize the reporting of network data, data can be reported according to a non-fixed sampling ratio. Data consumer can indicate the non-fixed sample ratio in the data subscription request. Data consumer can update the data sampling ratio dynamically by updating the data subscription. Data provider can use different sampling ratio according to local configuration and provide the sampling ratio info in the data notifications to the data consumer.

6.	Normative change (only impacting stage 2) is recommended to allow NWDAF to register in the UDM for the served UE, also for Analytics IDs that are not UE-related, as defined in Solution #65.

7.	For storing ML models in ADRF, MTLF can store ML model in ADRF based on MTLF policy. ADRF shall not duplicate the functionality provided by MLModelProvision Service.

8.	MTLF sends the ML model address to ADRF by triggering ADRF’s ML model storage service, according to Rel-17, with no additional parameters to ADRF. A URL where the model file is stored in the ADRF is included in the response from ADRF to MTLF.

NOTE 2:	Whether the ADRF further downloads the ML model based on the ML model address and locally stores the ML model is left for implementation.

NOTE 3:	When storing ML models at the ADRF, whether and which additional security-related parameters need to be sent by MTLF to ADRF are to be determined with SA3.

# * * * * End of changes * * * *