3GPP TSG-WG SA2 Meeting #154	S2-2210785

# Toulouse, France, November 14 – 18, 2022		(revision of S2-220xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.273 | CR | 0252 | rev | - | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Multiple location report for MT-LR regulatory location service |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | [Ericsson?]Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI18_MLR |  |  |  |  |  | Date: |  |  | 2022-11-04 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | S2-2208249 was prospoed in S2#152 meeting and most changes on top of rel-17 design is fine. However, one aspect can be further optimized that when AMF receives the request from GMLC, and AMF determines the MT-LR is for multiple location estimate, then AMF immediately respond to GMLC with an ACK indication, which is different from a normal MT-LR handling. Same implementation is applicable to LMF, when receives the request from AMF. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Proposal in S2-2208249 can be further optimizaed when AMF receives the request from GMLC, and AMF determines the MT-LR is for multiple location estimate, AMF immediately respond to GMLC with an ACK indication, which is different from a normal MT-LR handling. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No solution for TEI18 MLR work. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

6.1.x	5GC-MT-LR multiple location procedure for the regulatory location service

- Steps 1-7 for 5GC-MT-LR procedure for the regulatory location service clause 6.1.1 are performed with the following differences:

  - At step 1 in clause 6.1.1 the request from external location services client may include additionally the number of requested locations, and the time interval.

  - At step 4 in clause 6.1.1 Namf_Location_ProvidePositioningInfo service operation invoked by GMLC may include additionally the number of requested locations, the time interval, GMLC contact address and LIR reference number. The GMLC contact address and LIR reference number must be provided if the number of requested locations is available and higher than one. The LIR reference number is allocated by GMLC based on predefined rule.

  - At step 7 in clause 6.1.1 Nlmf_Location_DetermineLocation service operation invoked by AMF may include additionally include the number of requested locations and the time interval and GMLC contact address and LIR reference number.

When AMF receives the request from GMLC, and AMF determines the MT-LR is for multiple location estimate, AMF immediately respond to GMLC with an ACK indication, which is different from a normal MT-LR handling.

- LMF performs positioning procedures based on Steps 8 for 5GC-MT-LR procedure for the regulatory location service clause 6.1.1

When LMF receives the request from AMF, and LMF determines the MT-LR is for multiple location estimate, LMF immediately respond to AMF with an ACK indication, which is different from a normal MT-LR handling.

- This step is executed if number of requested locations is available and higher than one. The LMF invokes an Nlmf_Location_EventNotify service operation towards GMLC and provides the current location of the UE and timestamp of the location estimate, and LIR reference number,.

- This step is executed if step 2 was executed. The GMLC sends the current location of the UE and timestamp of the location estimate to external location services client.

- If step 3 and step 4 were executed, then the LMF repeats what is described in clause 6.1.1 step 8 and determines again a geographical location and optionally a location in local coordinates. If the time interval was provided, LMFexecutes the positioning procedure based on the time interval.. If the time interval is not provided, it is up to LMF implementation to determine the time interval.

# * * * * End of changes * * * *