SA WG2 Meeting #154	S2-2210858

November 14th – 18th, 2022; Toulouse, France

Source:	Samsung

Title:	KI#1: Conclusion updates

Document for:	Approval

Agenda Item:	9.12.1

Work Item / Release:	FS_5GSATB / Rel-18

Abstract of the contribution: This contribution proposes to update KI#1 conclusion.

Discussion

There is a NOTE that packet out-of-order evaluation and conclusion are left for further discussion.

In show of hands made in SA2#153e-meeting, the majority was that it is up to higher layer protocols (e.g., TCP, QUIC) to re-order packets, and therefore no normative work is needed. And during the discussions it was also expressed that the packet out-of-order issue is not only for the satellite backhaul but it is more like a general issue. Hence, it is proposed not to proceed any normative works on packet out-of-order issue.

Proposal

It is proposed to update TR 23.700-27 as follows:

**** First Change ****

8.1	Conclusions on PCC/QoS control enhancement considering dynamic satellite backhaul

For the determination of packet delivery latency of the satellite backhaul on the UP path, following conclusions are proposed:

-	The PCF may determine the packet delivery latency on N3 interface, if it detects dynamic satellite backhaul is used to serve the PDU session. The PCF can determine the N3 latency based on configuration or by using QoS monitoring triggered by PCF or estimate the N3 latency based on the received satellite backhaul category (LEO, GEO, etc.);

-	The SMF activates the QoS monitoring to measure the packet delivery latency on N3 interface according to the PCC rule;

-	The measured delay can be used by PCF for policy control, and can be considered for UPF selection (e.g. when the different types of backhaul network can be used in CP and UP).

For exposure of backhaul information to the AF, there are solutions #1, #2, #3, following conclusions are proposed:

-	The PCF can report packet delivery latency to the AF using existing QoS monitoring solution;

-	The AF can request the PCF to report the packet delivery latency using existing QoS monitoring solution.

The reporting of the satellite backhaul category and the observed backhaul delay to CHF as proposed in Solution #1 can be supported but this is to be determined by SA WG5.

For handling of out-of-sequence packets that can occur at N3, it is concluded that it is up to higher layer protocols (e.g., TCP, QUIC) to re-order packets, and no normative work is needed.

**** End of Changes ****