3GPP SA WG2 Meeting #154  			   	   S2-2211206

Toulouse, France, November 14-18, 2022	(was S2-2210752)

Source:	Ericsson, Huawei, NEC

Title:	KI#3: Conclusion for KI#3

Document for:	Discussion/Approval

Agenda Item:	9.14.1

Work Item / Release:	FS_eNS_PH3

Abstract of the contribution: This contribution provides a conclusion to KI#3

# 1.	Discussion

This paper proposes a conclusion for KI#3.

# 2 Proposal

It is proposed to include the above information in an interim conclusion for KI#2

FIRST CHANGE

## 8.3	Conclusions for KI#3

Editor's note:	These are interim conclusions.

For support of limited AoS slices not matching deployed TAs it is proposed that:

-	Reconfiguration of TAs while keeping the uniform support of S-NSSAIs in cells within the TA unchanged, but if an operator do not want to change the TA borders, the operator configures the cells of a TA that are outside AoS to have no or limited resources using existing NG-RAN OAM configuration.

-	S-NSSAI availability policies with the validity set to location information (e.g. a set of cells, or combination of cells and TAs), that are sent to the UE. The UE uses the policies and when the availability are not valid, the UE considers the S-NSSAI to be 1) not registered or 2) registered while no UP are allowed to be activated based on information in the received policy.

NOTE 1:	Whether S-NSSAI validity policies is sent to the UE in the Configured NSSAI or separate is to be determined during normative phase.

NOTE 2:	How the handover can be optimized to prevent the UE from leaving the slice service area (or entering into the slice service area) will be considered during normative phase based on RAN WG feedback.

NOTE 3:	The AMF enforcement of the S-NSSAI availability policies e.g. when the UE does not support the policies will be described during normative phase based on AMF subscribing to AoI.

For improved support of temporary network slices:

-	AMF is configured with S-NSSAI availability policies that the AMF sends to the UE. Availability validity can be time and location. The UE uses the policies and when the availability are not valid, the UE considers the S-NSSAI to be 1) not registered or 2) registered while no UP are allowed to be activated based on information in the received policy.

NOTE 4:	Whether S-NSSAI validity policies is sent to the UE in the Configured NSSAI or separate is to be determined during normative phase.

NOTE 5:	The AMF enforcement of the S-NSSAI availability policies e.g. when the UE does not support the policies will be described during normative phase.

NOTE:	Temporary network slices does not mean that the network slices are decommissions and created as per the timing information, but the network slices are not meant to be available for use by the UE.

For the graceful and gradual termination aspect:

Option 1:

-	For decommission of a network slice, OAM and NF implementation (e.g. by updating URSP rules and to deregister not used S-NSSAIs) can handle the functionality without further standardization. Whether OAM provides timing information to an NF as to allow the NF to apply a change of resource utilization in advance, e.g. a time for how long time a Shutting Down state is to be valid, would be up to SA WG5.

-	UE can be aware of the validity timer of the slice or applicable area of the service in the validation criteria in existing URSP and release the PDU Sessions for the related S-NSSAI gracefully based on the current design as described in clause 6.6.2.3 of TS 23.503 [12].

Option 2:

-	When a slice is decommissioned, the PDU Sessions of the slice should be gracefully (for supporting UEs of the timing information) and gradually released (for no supporting UEs of the timing information):

-	If the UE supports the timing information indicating time of network slice availability, the network may provide the timing information to the UEs so the UE knows in advance when a network slice ceases to be supported. In this case, the UE can take the necessary actions to prepare for the slice not becoming available.

-	In addition, the AMF, for non-supporting UEs and for the case of UE not performing any actions despite of the timing information provided by the network, may be triggered by the OAM to start gradually terminating PDU Session(s) associated with S-NSSAI subject to be terminated. The AMF releases PDU Session(s), associated with the S-NSSAI subject to be terminated, based on operator’s policy available at the AMF.

Editor's note:	Whether UDM need to signal anything to AMF is FFS.

Editor's note:	The above bullet is FFS and depends on the conclusion of temporary network slices.

Editor's note:	It is FFS which option are adopted for normative work.

END OF CHANGES