3GPP TSG SA WG2 Meeting #155	S2-2302230

# February 20 - 24, 2023, Athens											revision S2-2301449)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.228 | CR | 1264 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | IMS SBA Services |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RTC |  |  |  |  |  | Date: |  |  | 2023-02-04 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | To include Support for IMS AS SBA Services |  |  |  |  |  |  |  |  |
| Summary of change: |  | Include IMS AS serices/operations and the attributes required |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annex AA (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | x |  | Other core specifications |  |  |  | TS 23.228 CR#1263 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Annex XX is from CR1263 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

**** First Change ****

Annex AA (normative):
Support of Data Channel services in IMS

## AA.1 General

This section details the SBA the IMS architecture of Annex XX. The main Network Functions subject to SBA services in the IMS architecture are the IMS AS, DCMF, and ARMF.

### AA.1.1 IMS AS Services

#### AA.1.1.1 General

The following table shows the IMS AS Services and IMS AS Service Operations.

Table XX.3.1.2-1: NF services provided by the IMS AS

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Nimsas_SessionEventControl | Subscribe (Not specified in this Release) | Subscribe/Notify | DCSF |
|  | Notify | Subscribe/Notify | DCSF |
| Nimsas_MediaControl | MediaInstruction | Request/Response | DCSF |

#### AA.1.1.2	Nimsas_SessionEventControl

##### AA.1.1.2.1 General

Service description: This service enables the consumer to be notified about session events when served IMS subscribers takes part in IMS sessions.

The following operations are available for this service:

-	Explicit subscription to receive session events. This is not specified in this Release. In this Release an implicit subscription is assumed where the IMS AS notifies a configured or discovered DSCF of a call event through a Notify.

-	Notifying IMS session control events of a specific IMS subscriber to NFs.
This service operation reports the event to the consumer that has subscribed implicitly via configuration in the IMS AS.

##### AA.1.1.2.2	Nimsas_SessionEventControl_Notify service operation

Service operation name: Nimsas_SessionEventControl_Notify

Description: This service operation enables IMS AS to notify consumers of session events related to a specific served IMS subscriber requesting use of IMS data channel media.

Inputs, Required: SessionID, EventID,

SessionID is the identity of the IMS session for which the event relates to.

EventID is the event triggered within the IMS session.

Inputs, Optional:  CallingID, CalledID, SessionCase, EventInitiator, MediaInfoList.

CallingID is the public identity of the calling IMS subscriber. CalledID is the public identity of the called IMS Subscriber. SessionCase indicates if this is an originating or terminating IMS session. EventInitiator indicates initiator of the event, i.e. ‘served IMS subscriber’ vs ‘remote IMS subscriber’. MediaInfoList includes for each media in the list;

- 	mediaID : uniquely identifies this media item within the list. The identity is allocated by IMS AS,

- 	mediaSpecification:  This dependends on media type including relevant media attributes of interest to the consumer. The media specification includes the following media description attributes:

-	Media Type :  Data_Channel.

-	Data Channel Mapping and Configuration Information : This attribute is applicable to Data Channel and includes relevant configuration Information.

-	Maximum Message Size: This attributes defines the maximum size to be expected.

-	Data Channel Port : This attribute identifies the port for the Data Channel.

-	Security Setup : This attribute identifies the security setp up.

-	Security Certificate Fingerprint: This attribute identifies the security certificate fingerprint.

-	Security Transport Identity : This attribute identifies transport layer  identity.

Outputs, Required: Result indication.

Outputs, Optional: None

The table below presents supported EventIDs and related parameters.

Table AA.1.1.2.2-1 : List of events and Related Optional parameters

| EventID | Parameters |
| --- | --- |
| SessionEstablishmentRequestEvent | CallingID, CalledID, SessionCase, MediaInfoList |
| SessionEstablishmentProgressEvent | MediaInfoList |
| SessionEstablishmentAlertingEvent | MediaInfoList |
| SessionEstablishmentSuccessEvent | MediaInfoList |
| SessionEstablishmentFailureEvent |  |
| MediaChangeRequestEvent. | EventInitiator, MediaInfoList |
| MediaChangeSuccessEvent | MediaInfoList |
| MediaChangeFailureEvent | MediaInfoList |
| SessionTerminationEvent | SessionCase |

#### AA.1.1.3	Nimsas_MediaControl Service

##### AA.1.1.3.1	General

Service description: This service enables the consumer to control IMS AS handling of media flow within an IMS session. The service can be used by the consumer before responding to a Nimsas_SessionEventControl_Notity request. If the service is not used by the consumer when having received a Nimsas_SessionEventControl_Notity request, then this means that IMS AS will decide how listed media flows shall be handled.

##### AA.1.1.32	Nimsas_MediaControl_MediaInstruction service operation

Service operation name: Nimsas_MediaControl_MediaInstruction

Description: This operation provides instructions to the IMS AS for all media flows a consumer wants to control based on its policies for the received IMS session event, and that may require resource reservation in media resource e.g DCMF or enhanced MRF.

For the case when a specific media flow needs be terminated in DCMF media resource  (i.e. termination of a Data Channel media descriptor offered by one of the IMS subscribers) or originated by the DCMF or enhanced MRF (i.e. origination of a data channel media descriptor to be offered towards one of the IMS subscribers), the consumer must provide a complete DCMF media specification including information required by DCMF to know how to terminate or originate the media flow.

Inputs, Required: SessionID, MediaInstructionSet

SessionID specifies the IMS session for which the MediaInstruction operation applies.

MediaInstructionSet includes a set of instructions for each media flow to control. Each instruction includes:-

- mediaID:  used by the producer (IMS AS) and the consumer for referencing purposes. The consumer reuses the mediaID it received from the IMS AS for referencing the same media. This field will be null for instructions related to originating new media.

- mediaResourceCapability: Identify the Media Resource capabilities the mediaInstruction is intended for (e.g. DCMF, ARMF)

- mediaInstruction:  includes instuctions to the producer (IMS AS) for handling the media. The following instructions are supported:

-	“TerminateMedia”: Terminate the offered media descriptor of the UE in the mediaResource, i.e. this media descriptor will not be exposed to the other UE.

-	“OriginateMedia”: Originate and offer a media descriptor from the mediaResource to the UE. The mediaID representing the new media flow will be provided by the IMS AS in the response.

-	“TerminateAndOriginateMedia”: Terminate the offered media flow in the mediaResource from the UE and originate the same media flow from the mediaResource to the other UE, i.e. the offered media descriptor of the originating UE will be replaced by the mediaResource provided media descriptor, which is sent towards the other UE

-	 “UpdateMedia”: Update a media flow of the mediaResource previously allocated by the instructions “TerminateMedia’,”OriginateMedia”,”TerminateAndOriginateMedia”

-	 “DeleteMedia”: Delete a media flow of the mediaResource previously allocated by the instructions “TerminateMedia’,”OriginateMedia”,”TerminateAndOriginateMedia”.

-	 “RemoveMedia”: Remove an offered media flow, i.e the offered media descriptor will be removed both from the offer is sent to the remote UE and from the answer returned to the initiator of the offer.

Inputs, Conditional:  None.

Inputs, Optional:

mediaResourceData:  is an optional element of each media instruction including information intended for consumption by the media resource.  For mediaResourceCapability ‘DCMF’:

-	 Media proxy configuration (HTTP or UDP) applicable to the media flow,

-	Remote MDC1/MDC2 media endpoint address,

-	The replacement HTTP URL allocated by the consumer representing the application list offered to the IMS subscriber via the MDC1 interface,

-	Data Channel Mapping and Configuration information when originating/terminating data channel media flows on the Mb interface.

Outputs, Required: Atomic operation result indication

Outputs, Optional: mediaResourceInformationSet:

This set includes entries correspdoing to each instruction in the received media instruction set. Each entry in the set includes the mediaID (same as received or new for new media) , and may include the mediaResourceCapability ‘DCMF’, which includes information about the allocated resource being of interest for the consumer. Example of such media resource information is the allocated local media address for the MDC2 interface when an offer must be provided to the Data Channel Application Server.

**** End of Changes ****