3GPP TSG-SA2 Meeting # 155	S2-2302320

Athens, GR, Februrary 20 - 24, 2023

Source: 	Ericsson

Title:	Discussion on RRC_INACTIVE UE receiving MBS data and MBS assistant information

Document for:	Discussion/Agreement

Agenda Item:	9.10.2

Work Item / Release:	5MBS_Ph2

Abstract of the contribution: This paper discusses how to resolve the ENs and address the missing use case related to the MBS Assistance Information.

# Discussion

## 1.1 Introduction

During the normative work of 5MBS for Rel-18 in SA2#154AHe (Jan 2023), when specifying MBS assistance information for KI#1 (MBS session reception in RRC Inactive) based on TR 23.700-47 v18.0.0, a few ENs are added in the related CRs, e.g. S2-2301592 includes the following ENs:

EN#1:

Editor's Note:	Whether the "UE level" prefix is needed in the MBS assistance information final naming is FFS.

EN#2:

Editor's Note:	Whether the MBS assistance information need directly mention the RRC state is FFS.

## 1.2 Resolving ENs

For EN#1,

Per clause 8.1 of TR 23.700-47, use of "UE level" is to differentiate from the "MBS session level " parameter which are the existing MBS QoS parameters, see excerpt below:

8.1	Key Issue #1: MBS session reception in RRC Inactive

…

-	For MBS session level assistance information:

-	The existing MBS session QoS parameters (e.g. ARP, 5QI) can be used as the MBS session level assistance information by NG-RAN to differentiate different MBS sessions.

-	For UE level MBS assistance information:

-	The UE level MBS assistance information is an optional new parameter and set per MBS session. It is indicated by the AF to inform the network whether from the expected traffic pattern of the UE the indicated UE is preferred to be kept in the RRC Connected state even if it is able according to its radio capabilities to receive the MBS session data in RRC_INACTIVE state, e.g. a frequent talker.

As the MBS session level parameters are existing ones, there is no need to introduce a new name (i,e. MBS session level assistance information) for the existing parameters. With that, we can use MBS assistance information instead of UE level MBS assistance information to describe the new parameter provided by the AF to 5GC.

[Proposal-1] It is proposed to use MBS assistance information instead of UE level MBS assistance information for the new parameter provided by the AF to the 5GC.

For EN#2,

RRC states (RRC_CONNECTED or RRC_INACITVE) are mechanisms implemented in Access Stratum level in the RAN. It is our view that the assistance information provided by an AF should not be tied to an implementation mechanism in RAN.

During the TR phase, there are different used cases discussed (e.g., “frequent talker”, “group leader”). To address different use cases, the definition of the MBS assistance information needs to be generic.

To define the assistance information in a more generic way as well as to avoid mentioning the implementation mechanism in RAN, the following is proposed:

[Proposal-2] It is proposed that the MBS assistance information indicate that a UE is preferred to be kept connected in the duration of receiving MBS data from a specific MBS Session.

[Proposal-3] It is proposed to add examples of frequent talker and group leader in the procedure of “AF provisioning MBS assistance information”.

# 2 Proposal

[Proposal-1] It is proposed to use MBS assistance information instead of UE level MBS assistance information for the new parameter provided by the AF to the 5GC.

[Proposal-2] It is proposed that the MBS assistance information indicate that a UE is preferred to be kept connected in the duration of receiving MBS data from a specific MBS Session.

[Proposal-3] It is proposed to add examples of frequent talker and group leader in the procedure of “AF provisioning MBS assistance information”.

See updates in TS 23.247 CR0168 (S2-2302321), TS 23.247 CR0149 (S2-2302322), TS 23.247 CR0159 (S2-2302323), and TS 23.502 CR3705 (S2-2302324)