3GPP TSG-SA2 Meeting # 155	S2-2302344

# February 20 – 24, 2023, Athens, Greece									 (revision of S2-2301475)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.256 | CR | 0075 | rev | 2 | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support for direct C2 communication |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | [InterDigital,] LG Electronics, Qualcomm Incorporated, Ericsson, Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | UAS_Ph2 |  |  |  |  |  | Date: |  |  | 2023-02-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | One of the work item (UAS_Ph2) objectives is to support C2 communication over PC5, including the authorization of C2 communication over PC5. The conclusion for KI#1 of the study item (FS_UAS_Ph2) has been captured in 8.1 of TR 23.700-58, which is summarized below: - Authorization for C2 over PC5 may be based on provisioned policy or existing C2 authorization procedure through 3GPP network. - Existing unicast PC5 communication procedure in TS 23.304 is used as the baseline for C2 communication. This CR introduces the normative text for this functionality based on the above conlcusion. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add a new clause 5.x for support of Direct C2 communicaiton. - Update the existing C2 authorization procedure in 5.2.5 to support authorization for C2 over PC5 In SA2#155, Rev#2 resolves the following ENs, replaces TS 23.304 with TS 23.287 for establishing C2 communication over PC5, and captures enhancements for C2 authorization for Direct C2 Communication under new clause instead of inserting them to clauses 5.2.5.2.3 and 5.2.5.3.1: - Editor's note: Whether other information needs to be preconfigured in the UAV is FFS. - Editor's note: It is FFS whether a dynamic A2X service type for C2 communication can be used by the UAV and broadcasted in DCR so that only the interested/paired UAV-C responds to the DCR carrying the dynamic A2X service type, and how such dynamic A2X service type is derived is FFS. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | C2 over PC5 is not supported and the WI objective is not fulfilled. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 3.1, 5.2.5 (only title), 5.x (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | In SA2#154AH-E, Rev#1 approved. In SA2#155, Rev#X submitted. |  |  |  |  |  |  |  |  |

FIRST CHANGE

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System architecture for the 5G System (5GS)".

[3]	3GPP TS 23.502: "Procedures for the 5G System (5GS)".

[4]	3GPP TS 23.222: "Common API Framework for 3GPP Northbound APIs".

[5]	3GPP TS 22.125: "Unmanned Aerial System (UAS) support in 3GPP".

[6]	3GPP TS 23.401: "General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access".

[7]	3GPP TS 36.300: "Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Overall description; Stage 2".

[8]	3GPP TS 23.273: "5G System (5GS) Location Services (LCS); Stage 2".

[9]	3GPP TS 23.503: "Policy and charging control framework for the 5G System (5GS); Stage 2".

[10]	3GPP TS 33.256: "Security aspects of Uncrewed Aerial Systems (UAS)".

[x]	3GPP TS 23.287: "Architecture enhancements for 5G System (5GS) to support Vehicle-to-Everything (V2X) services".

NEXT CHANGE

3	Definitions and abbreviations

3.1	Definitions

For the purposes of the present document, the terms and definitions given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1] or TS 23.501 [2].

3GPP UAV ID: Identifier assigned by the 3GPP system and used by external AF (e.g. USS) to identify the UAV. GPSI is used as the 3GPP UAV ID.

Application Layer ID: An identifier identifying an entity, e.g. a UAV, a UAV-C, within the context of a specific A2X application. This identifier is used for Direct C2 Communication. The format of this identifier is outside the scope of 3GPP.

NOTE 1:	The Application Layer ID could be e.g. CAA-Level UAV ID for UAV defined by other SDOs e.g. ASTM, etc.

Broadcast Remote ID: The capability of providing Remote Identification and Tracking over broadcast radio links.

NOTE 2:	In the scope of this release, the radio link for Broadcast Remote ID is assumed to utilize radio technologies outside the scope of 3GPP.

CAA (Civil Aviation Administration)-Level UAV Identity: a UAV identity assigned by USS/UTM, and uniquely identifies a UAV at least within the scope of a USS.

Command and Control (C2) Communication: the user plane link to deliver messages with information of command and control for UAV operation from a UAV controller or a UTM to a UAV or to report telemetry data from a UAV to its UAV controller or a UTM. C2 communication may be over Uu reference point or PC5 reference point.

C2 Aviation Payload: Contains application layer information sent by the UAS to the USS containing UAV pairing information and/or flight authorization information that is transparent to the 3GPP System.

C2 Authorization Payload: Contains application layer information sent by the USS to the UAV containing e.g. C2 pairing information and/or C2 security information that is transparent to the 3GPP System.

C2 Pairing Information: Contains UAV-C Addressing Information which may e.g. include the UAV-C IP Address.

Direct C2 Communication: the UAV controller and UAV establish a direct C2 link over PC5 reference point to communicate with each other.

Networked UAV Controller: a UAV Controller connected to the 3GPP network and connected to the UAV via a 3GPP network.

Non-Networked UAV Controller: a UAV Controller not connected to the 3GPP network and connected to UAV via a transport outside the scope of 3GPP, e.g. internet connectivity or direct wireless communication over a technology outside the scope of 3GPP.

Networked Remote ID: The capability of providing Remote Identification and Tracking to a USS over 3GPP network.

Remote Identification (Remote ID) of UAS: The ability of a UAS in flight to provide identification and tracking information that can be received by other parties, to facilitate advanced operations for the UAS (such as Beyond Visual Line of Sight operations as well as operations over people), assist regulatory agencies, air traffic management agencies, law enforcement, and security agencies when a UAS appears to be flying in an unsafe manner or where the UAS is not allowed to fly. The Remote ID information payload may include Serial Number or Session ID assigned to the UAV, location of the ground-station controller, emergency status indication, etc.

Third Party Authorized Entity: is either a privileged Networked UAV Controller, or a privileged Non-Networked UAV Controller, or another entity which gets information on sets of UAV controllers and UAVs from the 3GPP network, and may be connected to the UAV via the Internet; it may be authorized by the UTM to interface with sets of UAV(s).

UAS NF: a 3GPP UAS Network Function for support of aerial functionality related to UAV identification, authentication/authorization and tracking, and to support Remote Identification.

UAS Service Supplier (USS): An entity that provides services to support the safe and efficient use of airspace by providing services to the operator / pilot of a UAS in meeting UTM operational requirements. A USS can provide any subset of functionality to meet the provider's business objectives (e.g. UTM, Remote Identification). In the scope of this specification, the term USS refers to both USS and USS/UTM.

UAS Traffic Management (UTM): a system that can safely and efficiently integrate the flying UAV along with other airspace users. It provides a set of functions and services for managing a range of autonomous vehicle operations (e.g. authenticating UAV, authorizing UAS services, managing UAS policies, and controlling UAV traffics in the airspace).

UAV controller: The UAV controller of a UAS enables a drone pilot to control an UAV.

UAV operator: the entity owning and operating a UAV.

UAS Container: A container to the 3GPP system that includes UUAA Aviation/Authorization Payload and/or C2 Aviation/Authorization Payload. The internal content of the individual payloads is transparent to the 3GPP system.

UAS Services: refers to establishment of connectivity for a UAS for communication with USS, for C2, for remote identification, and for UAV location and tracking.

USS communication: A communication between a UAV and a USS other than C2 communication, by means of user plane data transmission for some UAS Services.

NOTE 3:	The PDU session/PDN connection for C2 communication and the PDU session/PDN connection for USS communication can be common or separate.

UUAA Authorization Payload: Contains application layer information optionally including UUAA result for UAV consumption provided by the USS to the UAS which is transparent to the 3GPP System.

UUAA Aviation Payload: Contains application layer information provided by the UAS to USS and is transparent to the 3GPP System

Uncrewed Aerial System (UAS): Composed of Uncrewed Aerial Vehicle (UAV) and related functionality, including command and control (C2) links between the UAV and the control station, the UAV and the network, and for remote identification. An UAS may comprise of a UAV and a UAV controller.

Unknown UAVs: A list of the UAVs to be identified in the target area and served by the PLMN as the result of the UAV tracking requested by USS/UTM.

UUAA: UAV USS authentication and authorization procedure of the UAV to ensure that the UAV has successfully registered with a USS and has therefore been authorized for operations by the USS. An UAV is authenticated and authorized by USS via a UUAA procedure with the support of the 3GPP system before connectivity for UAS services is enabled.

UUAA-MM: the UUAA procedure optionally performed during registration to a 5GS.

UUAA-SM: the UUAA procedure performed during the establishment of a PDU session and performed during the establishment of a PDN connection.

NEXT CHANGE

5.2.5	Authorization for C2 over Uu

5.2.5.1	General

Authorization for C2 is required when a UAV establishes a user plane connection for C2 operations, i.e. to deliver messages with information of command and control for UAV operations from a UAV-C or USS to a UAV or to report telemetry data from a UAV to its UAV-C. Two sides of C2 communication, i.e. UAV and UAV-C, belong to the same UAS.

A UAV shall be authorized by the USS to use a PDU Session/PDN connection for C2. Authorization for C2 includes the following:

-	UAV to UAV-C pairing authorization: Authorization for pairing with a networked UAV-C or a UAV-C that connects to the UAV via Internet connectivity, before the UAV and the UAV-C can exchange C2 communication. One UAV can be paired with only one UAV-C at the any time. One UAV-C may be paired with one or more UAVs at the same time.

-	Flight Authorization: Authorization for flight when UAV also provides Flight Authorization information.

C2 authorization may be carried out:

-	During the UUAA procedure (if UUAA is carried out at PDU session/PDN connection establishment) as described in clause 5.2.3 when the UAV requests establishment of PDU Session/PDN connection for connectivity.

-	During PDU Session Modification/ UE requested bearer resource modification when the UAV requires to use an existing PDU session/PDN connection to exchange C2 communication related messages.

-	During a new PDU Session/PDN connection establishment, if the UAV requires to use a separate PDU Session/PDN connection for C2 communication.

NEXT CHANGE

5.x	Direct C2 Communication

5.x.1	General

A UAV that supports Direct C2 Communication may establish direct PC5 link with a UAV-C. The UAV that is engaged in Direct C2 Communication may or may not be capable of connection with 3GPP network. The UAV is authorized by the USS for establishing Direct C2 Communication with the UAV-C. The UAV-C with which the UAV performs Direct C2 Communication may be pre-configured in the UAV or provided by the network as described in clause 5.x.3. The UAV may be preconfigured with the A2X service type for direct C2 Communication, direct C2 pairing information (e.g., Application Layer ID of the UAV-C), a default Destination Layer-2 ID for initial signalling to establish unicast connection, authorization policy for direct C2 Communication.

5.x.2	Authorization policy for Direct C2 Communication

The UAV that supports Direct C2 Communication may be provisioned with the following authorization policy:

-	When the UAV is "served by NG-RAN":

-	PLMNs in which the UAV is authorized to perform Direct C2 Communication over PC5 reference point when "served by NG-RAN".

-	When the UE is "not served by NG-RAN":

-	Indicates whether the UE is authorized to perform Direct C2 Communication over PC5 reference point when "not served by NG-RAN".

5.x.3	Procedure for C2 authorization for Direct C2 Communication by the USS

If the UAV is capable of 3GPP network connection, the UAV performs the C2 authorization as described in clause 5.2.5. If the UAV supports Direct C2 Communication and the authorization policy for Direct C2 Communication is absent, it shall include an indication for Direct C2 Communication authorization in the authorization request.

In 5GS, the procedure for UE initiated PDU Session Establishment for C2 Communication as specified in clause 5.2.5.2.3 is used to request C2 authorization for Direct C2 Communication to the USS, with the following enhancements:

-	In step 1: When the UAV needs to establish a direct PC5 link required for connectivity to UAV-C (i.e. Direct C2 Communication), the C2 Aviation Payload sent by the UAV includes an indication that the authorization is also for Direct C2 Communication.

-	In step 4: If the authorization request for Direct C2 Communication was included in step 1 and the C2 authorization is successful, the USS may include direct C2 pairing information containing the UAV-C's Application Layer ID in the C2 Authorization Payload in the Naf_Authentication_AuthenticateAuthorize response.

-	In step 6: If received from the USS, the direct C2 pairing information included in the PDU Session Accept is sent by the SMF to the UAV.

In EPS, the procedure for UE requested PDN connectivity for C2 authorization as specified in clause 5.2.5.3.1 is used to request C2 authorization for Direct C2 Communication to the USS, with the following enhancements:

-	In step 1: When the UAV needs to establish a direct PC5 link required for connectivity to UAV-C (i.e. Direct C2 Communication), the C2 Aviation Payload sent by the UAV includes an indication that the authorization is also for Direct C2 Communication.

-	In step 4: If the authorization request for Direct C2 Communication was included in step 1 and the C2 authorization is successful, the USS may include direct C2 pairing information containing the UAV-C's Application Layer ID in the C2 Authorization Payload in the Naf_Authentication_AuthenticateAuthorize response.

-	In step 6: If received from the USS, the direct C2 pairing information included in the PDN Connectivity Accept is sent by the SMF+PGW-C to the UAV.

5.x.4	Procedure for Direct C2 Communication establishment

The procedure for Unicast mode V2X communication over PC5 reference point as defined in clause 6.3.3.1 of TS 23.287 [x] is used for establishing C2 communication over PC5 reference point, with the following enhancements:

-	In Step 3,

-	the V2X Service Info is set to A2X service type for direct C2 Communication. The A2X service type for direct C2 Communication may be preconfigured in the UAV.

-	the Source User Info is set to the Application Layer ID of the UAV.

-	the Target User Info is set to the Application Layer ID of the UAV-C. If the Application Layer ID of the UAV-C is provided by the network as described in clause 5.x.3, that Application Layer ID is used. Otherwise, the preconfigured Application Layer ID of the UAV-C is used.

-	The destination Layer-2 ID is set to the Layer-2 ID of the UAV-C if known in the UAV. Otherwise, the destination Layer-2 ID is set to a default Destination Layer-2 ID for initial signalling to establish unicast connection as preconfigured in the UAV.

In this Release, only Unicast mode communication over PC5 reference pointis supported for Direct C2 Communication.

NOTE:	Security aspects of direct C2 Communication(e.g. including the privacy protection of UAV-C identifier)  will be defined by SA WG3.

END of CHANGES