3GPP TSG-WG SA2 Meeting #155	S2-2302373

# Athens, Greece, Feb 20th – 24th, 2023      								(revision of S2-2302003)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0634 | rev | 2 | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Enhance NWDAF to enable Federated Learning |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph3 |  |  |  |  |  | Date: |  |  | 2023-02-06 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | According to the conclusion of KI#8, FS_eNA_ph3 in TR 23.700-81, a new Federated Learing mechanism has been introduced to do model training for some scenarios that faces user data privacy or security issues. In the conclusion, two new functionalities i.e. FL server and FL client are defined, some FL related capability information should be registered to NRF for FL server and FL client discovery. This CR is to generally describe the FL mechanism and refine registration and discovery procedure for FL server and FL client in TS 23.288 based on the conclusion. The NWDAF containing MTLF determines to train a ML model under the followding two conditions: - NWDAF containing AnLF requests NWDAF containing MTLF to provide an ML model - or based on local configuration Therefore, if the ML model should be trained via FL method but the NWDAF containing MTLF can not act as a FL server for the ML model training, it should discover an FL server NWDAF and the FL server NWDAF may deterimine to initiates FL procedure before providing the ML model. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Adding a general paragraph in clause 5 to describe the FL mechanism - Clarifying that based on local configuration the NWDAF containing MTLF may also determine to train a ML model - Clarifying that the NWDAF containing MTLF can not act as a FL server for the ML model training, it should discover an FL server NWDAF and the FL server NWDAF may deterimine to initiates FL procedure before providing the ML model - Removing the EN: Whether and how the NWDAF containing MTLF should relocate the ML model provision request is FFS. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is not clear what is FL and how to register and discover FL server or FL client |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.X (all new text) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS/TR ... CR ... TS/TR … CR… |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Changes (all new text) * * *

5.X 	Federated Learning (FL) among multiple NWDAFs

Federated learning among multiple NWDAFs is a machine learning technique in core network that trains an ML Model across multiple decentralized entities holding local data set, without exchanging/sharing local data set. This approach stands in contrast to traditional centralized machine learning techniques where all the local datasets are uploaded to one server, thus allowing to address critical issues such as data privacy, data security, data access rights.

NOTE: Horizontal Federated Learning is supported among multiple NWDAFs, which means the local data set in different FL client NWDAFs have the same feature space for different samples (e.g. UE IDs).

For Federated Learning supported by multiple NWDAFs containing MTLF, there is one NWDAF containing MTLF acting as FL server (called FL server NWDAF for short) and multiple NWDAFs containing MTLF acting as FL client (called FL client NWDAF for short), the main functionality includes:

FL server NWDAF:

- discovers and selects FL client NWDAFs to participant in an FL procedure

- requests FL client NWDAFs to do local model training and to report local model information.

- generates global ML model by aggregating local model information from FL client NWDAFs.

- sends the global ML model back to FL client NWDAFs and repeats training iteration if needed.

FL client NWDAF:

- locally trains ML model that tasked by the FL server NWDAF with the available local data set, which includes the data that is not allowed to share with others due to e.g. data privacy, data security, data access rights.

- reports the trained local ML model information to the FL server NWDAF.

- receives the global ML model feedback from FL server NWDAF and repeats training iteration if needed.

FL server NWDAF or FL client NWDAF register to NRF with their FL capability information as described in clause 5.2.

When NWDAF containing AnLF requests NWDAF containing MTLF to provide an ML model or based on local configuration to train an ML model, the NWDAF containing MTLF determines whether the ML model should be trained via FL mechanism based on Analytic ID, Service Area/DNAI or data can not be obtained directly from data producer NF (e.g., due to data privacy, data security).  The NWDAF containing AnLF is not aware whether the ML model is trained based on FL or not.

If the NWDAF containing MTLF can act as an FL server for the ML model training, then FL procdedure is initiated by the NWDAF containing MTLF as FL server NWDAF directly. Otherwise, the NWDAF containing MTLF should discover an FL server NWDAF as described in clause 5.2, and then the NWDAF containing MTLF requests the FL server NWDAF to provide the ML model and the FL server NWDAF may deterimine to initiates FL procedure before providing the ML model.

Before FL procedure is initiated by FL server NWDAF, appropriate FL client NWDAFs should be discovered by FL server NWDAF as decribed clause 5.2.

When starting an FL procedure, the FL server NWDAF is to provide an initial model to each FL client NWDAF, and then each FL client NWDAF is to perform local model training using their local data set. The detailed procedure for FL among Multiple NWDAFs is described in clause 6.2C.

* * * End of Changes * * *