3GPP TSG-WG SA2 Meeting #155	S2-2302483

# Athens, Greece,  2023-02-20 – 2023-02-24		(revision of S2-2301376)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 3923 | rev | 3 | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Network exposure support for XR services |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | [Tencent], [Tencent Cloud], [Xiaomi], [MediaTek], [vivo], [CATT], [Huawei], [China Mobile], [Meta USA], [ZTE], Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | XRM |  |  |  |  |  | Date: |  |  | 2023-02-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | According to the conclusions of KI#3 in TR23.700-60, certain network information could be exposed to the XR applications via API. These kind of exposure information and capabilities need to be added in the related clauses. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In new clause 5.37.X, the general descriptions of API based network exposure support for XR/Media service is added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The network exposure solutions of XRM is incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.37.X(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | There are two related discussion papers S2-2302481 and S2-2302482. and has a dependency on CR 3887 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | This CR is a revision of CR 3923 rev 1 |  |  |  |  |  |  |  |  |

# * * * * First change (all text new, update to show that for final approved version) * * * *

#### 5.37.X. Network Exposure of 5GS information

5GS and XR/media services cooperate to provide a better user experience using External Network Exposure.

Based on the AF request, the 5GS can report QoS Monitoring per QoS Flow information based on the QoS Monitoring as defined in clause 5.33.3 and/or clause 5.X:

-      For the congestion information, based on the PCC rule from PCF, the SMF requests the NG-RAN to report via GTP-U header to UPF and to PSA UPF to expose the information described in 5.37.Y.3 (this NG-RAN information can be provided to support ECN marking for L4S in PSA UPF as described in 5.37.Y.3) via Nupf_EventExposure service or via SMF/PCF/NEF as described in 5.8.2.X.

-    Data rate information may be

measured and reported by PSA UPF to AF via Nupf_EventExposure service or via SMF/PCF/NEF as described in 5.8.2.X based ob SMF request.

-      Round trip delay for multiple QoS Flows of the XR service (e.g. the UL and DL are separated into two flows) is determined based on the information provided by the PSA UPF on the QoS Monitoring for round trip delay measurement of individual QoS Flow as described in clause 5.33.3. PSA UPF exposes the information via Nupf_EventExposure service or via SMF/NEF/PCF as described in 5.8.2.X.

Editor’s Note: How the PCF correlates the QoS Montoring measurements to provide this information is FFS.

- PSA UPF exposes the information via Nupf_EventExposure service or via SMF/NEF/PCF as described in 5.8.2.X.

Editor’s Note: It is FFS whether all QoS monitoring requests can be addressed with the limitation of one QoS Monitoring control information per PCC.

-      Round trip delay for multiple QoS Flows of the XR service (e.g. the UL and DL are separated into two flows) is determined based on the information provided by the PSA UPF on the QoS Monitoring for round trip delay measurement of individual QoS Flow as described in clause 5.33.3. PSA UPF exposes the information via Nupf_EventExposure service or via SMF/NEF/PCF as described in 5.8.2.X.

Editor’s Note: How the PCF correlates the QoS Montoring measurements to provide this information is FFS.

Editor’s Note: It is FFS whether all QoS monitoring requests can be addressed with the limitation of one QoS Monitoring control information per PCC.

The AF may provide the Alternative QoS parameter set requirements and Averaging Window to the NEF/PCF for the GBR QoS Flow as specified in clause 4.15.6.6 of TS 23.502 [3].

Estimated bandwidth for 5QI may be exposed by NWDAF (according to information described in clause 6.9.2 in TS 23.288 [59]) to AF.

# * * * * End of changes * * * *