3GPP TSG SA WG2 Meeting #155	S2-2302635

# February 20 - 24, 2023, Athens, Greece									(Revision of S2-230xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.228 | CR | 1267 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | DCMF services |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile, Huawei, Hisilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RTC |  |  |  |  |  | Date: |  |  | 2023-02-04 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The services of DCMF is agreed in S2-230174 and captured in solution #20 in TR 23.700-87, which enables DCMF managing a media flow in one service invocation. In such case, multiple service invocations are required to manage e.g. orginating/terminating bootstrap DCs and application DC in a single service invocation. A more efficient way is to allow DCMF to manage one or multiple media flows in one service invocation. It is proposed to define new DCMF services on top of the contents captured in the TR to enabling DCMF managing one or multiple media flows in one service invocation. Moreover, according to the conclusion in the TR, DCMF may also support the function of ARMF, which will be also supproted in DCMF service. The architecture to support SBA in IMS is also updated to add related NFs and reference points. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The architetrure to support SBA in IMS is updated to add related NFs and reference points. The services of DCMF is proposed in Annex AA, enabling DCMF managing one or multiple media flows in one service invocation and fucntion of ARMF. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annex AA |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

**** First Change ****

Annex AA (normative):
Support of SBA in IMS

# AA.1	General

## AA.1.0	Overview

This Annex AA describes support for SBA for IMS nodes. This Annex is intended to be used in conjunction with 5GC.

## AA.1.1	Architectural Support

Figure AA.1.1-1 shows the architecture to support SBA interactions between IMS entities.

# Figure AA.1.1-1: System Architecture to support SBA in IMS

NOTE 1:	The DCSF and enhanced MRF supports NF registration and discovery, but they do not provide services in this release of the specification.

Figure AA.1.1-2 shows the architecture using the reference point representation.

# Figure AA.1.1-2: System Architecture to support SBA in IMS in reference point representation

NOTE 2:	In this release of the specification the SBI capable IMS AS supporting data channel services is collocated with the TAS.

## AA.1.2	Reference point to support SBA in IMS

Following reference points are realized by service-based interfaces in IMS:

N5:	Reference point between the PCF and an AF.

NOTE:	P-CSCF acts as an AF from PCF point of view. N5 Reference point is defined in TS 23.501 [93].

N70:	Reference point between an SBI capable I/S-CSCF and an SBI capable HSS.

N71:	Reference point between an SBI capable IMS AS and an SBI capable HSS.

DC1:	Reference point between an SBI capable IMS AS and DCSF.

DC2:	Reference point between an SBI capable IMS AS and DCMF.

N72:	Reference point between DCSF and an SBI capable HSS.

## AA.1.3	Service based interface to support SBA in IMS

Npcf:	Service-based interface exhibited by PCF.

Nhss:	Service-based interface exhibited by an SBI capable HSS.

These SBI services provide equivalent functionality to the Diameter Rx and Cx/Sh reference points.

Nimsas:	Service-based interface exhibited by an SBI capable IMS AS.

Ndcmf:	Service-based interface exhibited by DCMF.

These SBI services provide functionality to support data channel management in IMS network.

To support co-existence of IMS nodes supporting SBA services and IMS nodes not supporting SBA services SBI enabled IMS nodes may support both SBI and non-SBI interfaces.

**** Second Change ****

AA.2.X		DCMF Services

AA.2.X.1	General

The following table illustrates the DCMF Services and Service Operations.

Table AA.2.X.1-1: NF services provided by the DCMF

| NF service | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Ndcmf_MediaResourceManagement (MRM) | Create | Request/Response | IMS AS |
|  | Update | Request/Response | IMS AS |
|  | Delete | Request/Response | IMS AS |

AA.2.X.2			Ndcmf_MediaResourceManagement (MRM) service

AA.2.X.2.1	General

Service description: This service enables the consumer to create, update and delete media resources related to IMS Data Channel, including the media resources for AR rendering if the DCMF supports media rendering for AR communication.

The media resource represents a media context including one or multiple media terminations. A media termination includes media resources for one or multiple media streams on the Mb interface.

Each media stream of a termination is identified by a media ID. Media streams of different terminations using the same media ID are connected with each other, i.e. the media stream is flowing between the terminations.

The service also enables the consumer to create, update and delete media resource requested by the upper layer, e.g. MDC1 or MDC2 requested by DCSF, or MDC4 requested by AR application server, which are conveyed in media data containers.

AA.2.X.2.2	Ndcmf_MRM_Create service operation

Service operation name: Ndcmf_MRM_Create

Description: The consumer NF requests the DCMF to create a media context including one or multiple media terminations and reserve media resources for one or multiple media streams of Mb interface in each termination. The consumer NF may also include media resource descriptions requested by the upper layer, e.g. DCSF, to be reserved on the DCMF.

Inputs, Required: List of Media Termination Descriptors.

Each Media Termination Descriptor of the list includes:

-	Termination Index: A unique identity of the termination within the media context resource. The identity is allocated by the consumer.

NOTE 1:	This termination index is only used for internal identification of a termination, not used for managing resources on the DCMF.

-	List of Media Stream Descriptors belonging to the Termination.

Editor’s note:	Whether and how media description for AR rendering is included in the Media Stream Descriptor is FFS.

Each Media Stream Descriptor of the list includes:

-	Media ID: A unique identity of the media stream within the media context instance. The MediaID value is set by the consumer.

NOTE 2:	When a media stream is added to different Terminations, the same MediaID is used to correlate the terminations for data forwarding.

-	 Remote Mb Specification: Optional data container specifying SDP parameters of a remote media endpoint e,g, IP address and ports of the UE.

-	Media operation: The operation for the media stream, e.g. media rendering

-	 Additional remote media specification: Optional data container specifing type and description of additional remote media resource needed for this media stream from upper layer, which includes:

-	Type of additional remote media resource: The value is set based on the type of the remote NF, e.g. “DCSF”.

-	Media Resource Description: Optional application specific data container specifying how the provider shall handle the media stream. Data content depends on Type of additional remote media resource.

If the Type of additional remote media resource is “DCSF”, the Media Resource Description includes:

-		Media proxy configuration (HTTP or UDP) applicable to the media flow.

-		Remote MDC1/MDC2 media endpoint address.

-		Replacement HTTP URL allocated by the upper layer representing the application list (e.g. graphical user   
   interface) offered to the IMS subscriber via the MDC1 interface.

If the Type of additional remote media resource is “AR Application Server”, the Media Resource Description includes:

-		Media proxy configuration (HTTP or UDP) applicable to the media flow.

-		Remote MDC4 media endpoint address.

-		Replacement HTTP URL allocated by the consumer offered to the IMS subscriber via the MDC4 interface.

Inputs, Optional: None

Outputs, Required: Result indication, Media Resource Context ID, List of allocated Media Termination Descriptor Resource information.

Media Resource Context ID uniquely identifies the media resources created by the DCMF. This identity shall be used as reference when updating or deleting the created media resource.

List of allocated Media Termination Descriptor Resource information includes one or multiple Media Termination Descriptor resources allocated. The list will include same number of terminations, in the same order as provided in the request.

Media Termination Descriptor Resource includes:

- Termination ID: Unique identity allocated by the consumer. The identity shall be used when updating/deleting the Media Termination.

- List of Media Stream Descriptor Resource information elements allocated within the media termination.

Media Stream Descriptor resource information includes:

- Media ID: Unique identity of the media stream within the media context instance allocated by the consumer.

- Local Mb Specification: Data container specifying local SDP parameters representing the media endpoint of the provider e,g, local IP address and ports etc.

- Additional local media resource description: Optional application specific data container specifying the information of additional local media resource depends on the type of the additional remote media resource requested.

If the Type of additional remote media resource  is “DCSF”, the Media Resource Description includes:

- DCMF local MDC1/MDC2 media point information (resource URL, IP address/port number etc) which shall be relayed to the DCSF by the consumer NF.

If the Type of additional remote media resource  is “AR Application Server”, the Media Resource Description includes:

- DCMF local MDC4 media point information (resource URL, IP address/port number etc) which shall be relayed to the DCSF by the consumer NF.

Outputs, Optional: None.

AA.2.X.2.3	Ndcmf_MRM_Update service operation

Service operation name: Ndcmf_MRM_Update

Description: The consumer NF requests the DCMF to update one or multiple media resources within a specific media resource context. Terminations and/or Media Streams can be added, modified or deleted within an Update request.

Inputs, Required: Media Resource Context ID, List of Media Termination Descriptors to be updated.

Media Resource Context ID specifies the media context resource to be updated.

List of Media Termination Descriptors includes one of multiple updated Media Stream Descriptors, which provides the media stream information that needs to be updated.

Inputs, Optional: None.

Outputs, Required: Result indication, List of Media Termination Descriptor Resource information.

List of Media Termination Descriptor Resource information includes one or multiple Media Stream Descriptor Resources impacted by the Update request.

Outputs, Optional: None.

AA.2.X.2.4	Ndcmf_MRM_Delete service operation

Service operation name: Ndcmf_MRM_Delete

Description: The consumer NF requests the DCMF to delete a specific media context resource including all existing terminations and media streams.

Inputs, Required: Media Context Resource ID.

Media Context Resource ID specifies the media context resource to be deleted.

Inputs, Optional: None.

Outputs, Required: Result indication.

Outputs, Optional: None.

**** End of Changes ****